Title:	Test point selection for Wide band Intermodulation for CA in FR1

Source:		CAICT

Introduction

At RAN#78, it was decided that for each NR test case a discussion of test point selection regarding frequency, channel BW, SCS, RB allocation and waveform is required and that the analysis is documented in TR 38.905.

In this contribution the selection of frequency, channel BW, SCS, RB allocation and waveform for Wide band Intermodulation for CA in FR1 is analyzed.

Discussion

The Wide band Intermodulation for CA in FR1 is defined in section 7.8A of TS 38.101-1.

- 7.8A	Intermodulation characteristics for CA

- 7.8A.1	General

- 7.8A.2	Wide band intermodulation for CA

- 7.8A.2.1	Wide band intermodulation for Intra-band contiguous CA

Table 7.8A.2.1-1: Wide band intermodulation parameters for intra-band contiguous CA with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz

| Rx parameter | Units | NR CA bandwidth class |  |  |
| --- | --- | --- | --- | --- |
|  |  | C | D | E |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | REFSENS + 6 | REFSENS + 13.8 | REFSENS + 15 |
| PInterferer 1 (CW) | dBm | -46 |  |  |
| PInterferer 2 (Modulated) | dBm | -46 |  |  |
| BWInterferer 2 | MHz | BWChannel_CA | 50 | 50 |
| FInterferer 1 (Offset) | MHz | -2BWChannel_CA / +2BWChannel_CA | -Foffset-75 / Foffset+75 | -Foffset-75 / Foffset+75 |
| FInterferer 2 (Offset) | MHz | 2*FInterferer 1 |  |  |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: Reference measurement channel is specified in Annexes A.2.2, A.2.3, A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1). NOTE 3: The modulated interferer consists of the Reference measurement channel specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1 and the same SCS as the closest carrier. NOTE 4: The Finterferer 1 (offset) is the frequency separation of the center frequency of the carrier closest to the interferer and the center frequency of the CW interferer and Finterferer 2 (offset) is the frequency separation of the center frequency of the carrier closest to the interferer and the center frequency of the modulated interferer. |  |  |  |  |

Table 7.8A.2.1-2: Wide band intermodulation parameters for intra-band contiguous CA with FDL_low  < 2700 MHz and FUL_low  < 2700 MHz

| Rx parameter | Units | NR CA bandwidth class |
| --- | --- | --- |
|  |  | C |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | REFSENS + 22 |
| PInterferer 1 (CW) | dBm | -46 |
| PInterferer 2 (Modulated) | dBm | -46 |
| BWInterferer 2 | MHz | 5 |
| FInterferer 1 (Offset) | MHz | -Foffset-7.5 / Foffset+7.5 |
| FInterferer 2 (Offset) | MHz | 2*FInterferer 1 |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: Reference measurement channel is specified in Annexes A.2.2, A.2.3, A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1). NOTE 3: The modulated interferer consists of the Reference measurement channel specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1 and 15 kHz SCS. NOTE 4: The Finterferer 1 (offset) is the frequency separation of the center frequency of the carrier closest to the interferer and the center frequency of the CW interferer and Finterferer 2 (offset) is the frequency separation of the center frequency of the carrier closest to the interferer and the center frequency of the modulated interferer. |  |  |

- 7.8A.2.2	Void

- 7.8A.2.3	Wide band intermodulation for Inter-band CA

For inter-band carrier aggregation with one component carrier per operating band and the uplink assigned to one NR band, the wide band intermodulation requirements are defined with the uplink active on the band(s) other than the band whose downlink is being tested. The UE shall meet the requirements specified in subclause 7.8 for each component carrier while all downlink carriers are active.

For the UE which supports inter-band CA configuration in Table 7.3A.3.2.1-1, Pinterferer power defined in Table 7.8.2-1 and 7.8.2-2 is increased by the amount given by ΔRIB,c in Table 7.3A.3.2.1-1.

The throughput of each carrier shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3, A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1).

Following subclauses introduce study for test environment, test frequencies, test bandwidth, test subcarrier spacing and uplink and downlink modulations and RB configurations.

Test Environment

In NR single carrier testing, the Wide band Intermodulation for CA is tested under Normal condition. The same condition can be selected for NR CA testing.

Proposal 1: Define test environment as Normal for Wide band Intermodulation for CA in FR1.

Test Subcarrier Spacing

In NR single carrier testing, the Wide band Intermodulation is tested using Highest subcarrier spacing. This can be followed in NR CA testing.

Proposal 2: For Wide band Intermodulation for CA, test Highest subcarrier spacing.

Test Frequencies

In LTE CA, the Test Frequency of Wide band Intermodulation for CA for intra-band contiguous CA is Mid range, and the Test Frequency of Wide band Intermodulation for CA for inter-band CA is Mid range for all PCC/SCCs. The same approach can be followed in NR testing.

Proposal 3: For Wide band Intermodulation for CA for intra-band contiguous CA, the Mid range is tested. For Wide band Intermodulation for CA for inter-band CA, the Mid range for all PCC/SCCs is tested.

Test Channel Bandwidth

In LTE CA, the test channel bandwidth of Wide band Intermodulation for CA for intra-band contiguous CA is Lowest NRB_agg and Highest NRB_agg and the test channel bandwidth of Wide band Intermodulation for CA for inter-band CA is Highest NRB_agg The same approach can be followed in NR testing.

Proposal 4: For Wide band Intermodulation for CA, select the Lowest NRB_agg and Highest NRB_agg for intra-band contiguous CA and the Highest NRB_agg for inter-band CA.

Modulation

In NR single carrier testing, the Wide band Intermodulation for CA is tested using CP-OFDM QPSK for downlink and DFT-s-OFDM QPSK for uplink. This can be followed in NR CA testing.

Proposal 5: For Wide band Intermodulation for CA, test CP-OFDM QPSK for downlink and DFT-s-OFDM QPSK for uplink.

RB allocation

In NR single carrier testing, the Wide band Intermodulation for CA is tested using Full RB allocations for downlink and uplink. This can be followed in NR CA testing.

In NR intra-band contiguous CA testing for Maximum input level and Adjacent channel selectivity, RBs are allocated for all downlink PCC/SCCs and uplink PCC. This can be followed in NR intra-band contiguous CA testing for Wide band Intermodulation for CA.

In LTE inter-band CA, RBs are allocated for all downlink PCC/SCCs and uplink PCC. This can be followed in NR CA testing.

Proposal 6: For Wide band Intermodulation for CA, test Full RB for all downlink PCC/SCCs and uplink PCC.

Conclusion

To sum up, the following selection of test points is proposed for Wide band Intermodulation for CA in FR1:

Proposal 1: Define test environment as Normal for Wide band Intermodulation for CA in FR1.

Proposal 2: For Wide band Intermodulation for CA, test Highest subcarrier spacing.

Proposal 3: For Wide band Intermodulation for CA for intra-band contiguous CA, the Mid range is tested. For Wide band Intermodulation for CA for inter-band CA, the Mid range for all PCC/SCCs is tested.

Proposal 4: For Wide band Intermodulation for CA, select the Lowest NRB_agg and Highest NRB_agg for intra-band contiguous CA and the Highest NRB_agg for inter-band CA.

Proposal 5: For Wide band Intermodulation for CA, test CP-OFDM QPSK for downlink and DFT-s-OFDM QPSK for uplink.

Proposal 6: For Wide band Intermodulation for CA, test Full RB for all downlink PCC/SCCs and uplink PCC.

Number of test points

In this document several proposals have been stated for the configuration parameters. In all of cases the analysis has been focused on reducing testing time without losing coverage in UE testing for NR FR1.

| Environmental conditions | Maximum Number of Frequencies | Maximum Number of ChBW | Number SCS | Number of steps (mod and RB) | Maximum Number of Test Steps |
| --- | --- | --- | --- | --- | --- |
| 1 | 1 | intra-band contiguous CA : 2; inter-band CA : 1 | 1 | 1 | intra-band contiguous CA : 2; inter-band CA : 1 |