3GPP TSG-RAN WG6 #1 	R6-160117

# Gothenburg, August 22-26, 2016	                Revision of R6-160072

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 45.004 | CR | 0025 | rev | 2 | Current version: | 13.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to EC-GSM-IoT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia Networks |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R6 |  |  |  |  |  |  |  |  |  |
| Work item code: | CIoT_EC_GSM-Core |  |  |  |  |  |  | Date: |  | 2016-08-16 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | Some corrections are needed to improve the consistency within the specification and to other specifications. The proposed changes are depicted below. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following changes are proposed: - In clause 1.2 the abbreviation for EC is added since not contained in TR 21.905. - In clause 2.2 a clarification is added that the contiguous bursts only refer to those bursts which carry blind physical layer transmissions, and not to other bursts in the same TDMA frame. - Note added section 2.7 - In clause 2.6 the term ‘phase coherency’ is changed to ‘phase and amplitude coherency’ to align with TS 45.005. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification is inconsistent with another specification and may cause misinterpretation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 1.2, 2.2, 2.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

1st modification

# 1.2	Abbreviations

Abbreviations used in this specification are listed in 3GPP TR 21.905. In addition to abbreviations in 3GPP TR 21.905 the following abbreviation apply:

AQPSK	Adaptive Quadrature Phase Shift Keying

EC	Extended Coverage

2nd modification

## 2.2	Start and stop of the burst

Before the first bit of the bursts as defined in 3GPP TS 45.002 enters the modulator, the modulator has an internal state as if a modulating bit stream consisting of consecutive ones (di = 1) had entered the differential encoder. Also after the last bit of the time slot, the modulator has an internal state as if a modulating bit stream consisting of consecutive ones (di = 1) had continued to enter the differential encoder. These bits are called dummy bits and define the start and the stop of the active and the useful part of the burst as illustrated in figure 1. Nothing is specified about the actual phase of the modulator output signal outside the useful part of the burst. In case of EC operation, the phase during the useful part of contiguous bursts, belonging to the same blind physical layer transmissions within a TDMA frame, has a fixed relation, see subclauses 2.6 and 2.7.

Figure 1: Relation between active part of burst, tail bits and dummy bits. For the normal burst the useful part lasts for 147 modulating bits

3rd modification

## 2.6	Modulation

The modulated RF carrier, except for start and stop of the TDMA burst may therefore be expressed as:

where Ec is the energy per modulating bit, f0 is the centre frequency and 0 is a random phase and is constant during one burst.

In case of EC operation when using blind physical layer transmissions (see 3GPP TS 43.064 [7]) the modulated RF carrier, except for start and stop of the burst may, for each blind physical layer transmission of a burst for which phase and amplitude coherency is required (see 3GPP TS 45.005 [4]), be expressed as:

where

-	t0 is a burst-specific time offset, constant during one burst, and is defined as the time difference between time instant t' = 0 for the current burst and t' = 0 for the first transmission of the same burst in the current TDMA frame.

-	157 is a phase shift of either 0 or πh, and is constant during one burst.

-	0 is a random phase and is constant during all blind physical layer transmissions of the same burst within the same TDMA frame.

For EC-GSM_IoT, only integer timeslot lengths are allowed (see 3GPP TS 45.010 [5]). If any blind physical layer transmission is transmitted in the uplink on timeslot 0 or timeslot 4, which are 157 symbols long, all following blind physical layer transmissions of the same burst in that TDMA frame shall be shifted in phase by πh, i.e. 157 = πh, otherwise 157 = 0. For the downlink this phase shift shall not be applied, i.e. 157 = 0.

## 2.7	Overlaid CDMA

In the case of Overlaid CDMA the modulated RF carrier may be expressed with an additional term compared to subclause 2.6 representing the Overlaid CDMA code:

where OC equals 0 or 1 in accordance with the applied Overlaid CDMA code (see 3GPP TS 45.002 [3]) and is constant during one burst. For each blind physical layer transmission within a TDMA frame, OC equals its respective Overlaid CDMA code element. For example for Overlaid CDMA code 0,0,1,1, OC equals 0 for the first two blind physical layer transmissions and 1 for the last two blind physical layer transmissions within a TDMA frame.

NOTE: If the blind physical layer transmission within a TDMA frame, are continuously modulated, a phase shift of   between two bursts can be well approximated by inverting all the bits of a burst, including at least three preceding guard bits.

End of modifications