3GPP TSG RAN WG6 #3                                                                       R6-170067

# Athens, Greece, February 13-17, 2017

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 45.001 | CR | 0097 | rev | - | Current version: | 14.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of new uplink coverage class CC5 for EC-PDTCH and EC-PACCH |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R6 |  |  |  |  |  |  |  |  |  |
| Work item code: | CIoT_EC_GSM_radio_enh-Core |  |  |  |  |  |  | Date: |  | 2017-02-07 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | Introduction of the new uplink coverage class for improved MCL performance in uplink for low power device has been agreed as part of WI in Rel-14 for radio interface enhancements for EC-GSM-IoT in RP-160806. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In clause 4 the channel block structure for EC-PDTCH and EC-PACCH for CC5 is included. - In sub-clause 7.1 the channel coding for CC5 EC-PACCH is included. - In sub-clause 7.2.4 the channel coding for the EC-PDTCH is defined through the use of a separate coding scheme MCS-1’. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The new coverage class CC5 for uplink MCL improvement is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4, 7.1, 7.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  | TS 43.064 CR 0111 TS 45.002 CR 0206 TS 45.003 CR 0145 TS 45.005 CR 0600 TS 45.008 CR 0649 TS 44.018 CR 1061 TS 44.060 CR 1639 TS 24.008 (draft CR) |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS 51.010 CR XXXX, TS 51.021 CR XXXX |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

1st Modification

# 4	The block structures

The different block structures are described in more detail in 3GPP TS 45.003. A summarised description appears in table 1, in terms of net bit rate, length and recurrence of blocks.

# Table 1: Channel block structures

| Type of channel | net bit rate (kbit/s) | block length (bits) | block recurrence (ms) |
| --- | --- | --- | --- |
| full rate speech TCH1 | 13,0 | 182 + 78 | 20 |
| enhanced full rate speech TCH1 | 12,2 | 170 + 74 | 20 |
| half rate speech TCH2 | 5,6 | 95 + 17 | 20 |
| Adaptive full rate speech TCH (12,2 kbit/s) | 12,2 | 244 | 20 |
| Adaptive full rate speech TCH (10,2 kbit/s) | 10,2 | 204 | 20 |
| Adaptive full rate speech TCH (7,95 kbit/s) | 7,95 | 159 | 20 |
| Adaptive full rate speech TCH (7,4 kbit/s) | 7,4 | 148 | 20 |
| Adaptive full rate speech TCH (6,7 kbit/s) | 6,7 | 134 | 20 |
| Adaptive full rate speech TCH (5,9 kbit/s) | 5,9 | 118 | 20 |
| Adaptive full rate speech TCH (5,15 kbit/s) | 5,15 | 103 | 20 |
| Adaptive full rate speech TCH (4,75 kbit/s) | 4,75 | 95 | 20 |
| Adaptive half rate speech TCH (7,95 kbit/s)8 | 7,95 | 123 + 36 | 20 |
| Adaptive half rate speech TCH (7,4 kbit/s)8 | 7,4 | 120 + 28 | 20 |
| Adaptive half rate speech TCH (6,7 kbit/s)8 | 6,7 | 110 + 24 | 20 |
| Adaptive half rate speech TCH (5,9 kbit/s)8 | 5,9 | 102 + 16 | 20 |
| Adaptive half rate speech TCH (5,15 kbit/s)8 | 5,15 | 91 + 12 | 20 |
| Adaptive half rate speech TCH (4,75 kbit/s)8 | 4,75 | 83 + 12 | 20 |
| Adaptive half rate 8-PSK speech TCH (12,2 kbit/s) | 12,2 | 244 | 20 |
| Adaptive half rate 8-PSK speech TCH (10,2 kbit/s) | 10,2 | 204 | 20 |
| Adaptive half rate 8-PSK speech TCH (7,95 kbit/s) | 7,95 | 159 | 20 |
| Adaptive half rate 8-PSK speech TCH (7,4 kbit/s) | 7,4 | 148 | 20 |
| Adaptive half rate 8-PSK speech TCH (6,7 kbit/s) | 6,7 | 134 | 20 |
| Adaptive half rate 8-PSK speech TCH (5,9 kbit/s) | 5,9 | 118 | 20 |
| Adaptive half rate 8-PSK speech TCH (5,15 kbit/s) | 5,15 | 103 | 20 |
| Adaptive half rate 8-PSK speech TCH (4,75 kbit/s) | 4,75 | 95 | 20 |
| Wideband Adaptive full rate speech TCH (12,65 kbit/s) | 12,65 | 253 | 20 |
| Wideband Adaptive full rate speech TCH (8,85 kbit/s) | 8,85 | 177 | 20 |
| Wideband Adaptive full rate speech TCH (6,60 kbit/s) | 6,60 | 132 | 20 |
| Wideband Adaptive full rate 8-PSK speech TCH (23,85 kbit/s) | 23,85 | 477 | 20 |
| Wideband Adaptive full rate 8-PSK speech TCH (15,85 kbit/s) | 15,85 | 317 | 20 |
| Wideband Adaptive full rate 8-PSK speech TCH (12,65 kbit/s) | 12,65 | 253 | 20 |
| Wideband Adaptive full rate 8-PSK speech TCH (8,85 kbit/s) | 8,85 | 177 | 20 |
| Wideband Adaptive full rate 8-PSK speech TCH (6,6 kbit/s) | 6,60 | 132 | 20 |
| (continued) |  |  |  |
| Wideband Adaptive half rate 8-PSK speech TCH (12,65 kbit/s) | 12,65 | 253 | 20 |
| Wideband Adaptive half rate 8-PSK speech TCH (8,85 kbit/s) | 8,85 | 177 | 20 |
| Wideband Adaptive half rate 8-PSK speech TCH (6,6 kbit/s) | 6,60 | 132 | 20 |
| data E-TCH (43,2 kbit/s) 3 | 43,5 | 870 | 20 |
| data E-TCH (32,0 kbit/s) 3 | 32,0 | 640 | 20 |
| data E-TCH (28,8 kbit/s)3 | 29,0 | 580 | 20 |
| data TCH (14,4 kbit/s) 3 | 14,5 | 290 | 20 |
| data TCH (9,6 kbit/s)3 | 12,0 | 60 | 5 |
| data TCH (4,8 kbit/s)3 | 6,0 | 60 | 10 |
| data TCH (£ 2,4 kbit/s)3 | 3,6 | 36 | 10 |
| PDTCH/F (CS-1) | 9,05 | 181 | - |
| PDTCH/F (CS-2) | 13,4 | 268 | - |
| PDTCH/F (CS-3) | 15,6 | 312 | - |
| PDTCH/F (CS-4) | 21,4 | 428 | - |
| PDTCH/H (CS-1) | 4,525 | 181 | - |
| PDTCH/H (CS-2) | 6,7 | 268 | - |
| PDTCH/H (CS-3) | 7,8 | 312 | - |
| PDTCH/H (CS-4) | 10,7 | 428 | - |
| EC-PDTCH/F (MCS-1/16)10,18 | 2,65 | 212 | - |
| EC-PDTCH/F (MCS-1/8)10,17 | 5,3 | 212 | - |
| EC-PDTCH/F (MCS-1/4)10,16 | 10,6 | 212 | - |
| EC-PDTCH/F (MCS-1’/48)23 | 0,88 | 212 | - |
| EC-PDTCH/2TS/F (MCS-1/16)10,18 | 1,325 | 212 | - |
| EC-PDTCH/2TS/F (MCS-1/8)10,17 | 2,65 | 212 | - |
| EC-PDTCH/2TS/F (MCS-1/4)10,16 | 5,3 | 212 | - |
| (EC-)PDTCH/F (MCS-1)10,15 | 10,6 (21,2)11 | 212 | - |
| (EC-)PDTCH/F (MCS-2)10,15 | 13,0 (26,0)11 | 260 | - |
| (EC-)PDTCH/F (MCS-3)10,15 | 16,6 (33,2)11 | 332 | - |
| (EC-)PDTCH/F (MCS-4)10,15 | 19,4 (38,8)11 | 388 | - |
| (EC-)PDTCH/F (MCS-5)10,15 | 24,05 (48,1)11 | 481 | - |
| (EC-)PDTCH/F (MCS-6)10,15 | 31,25 (62,5)11 | 625 | - |
| (EC-)PDTCH/F (MCS-7)10,15 | 47,45 (94,9)11 | 949 | - |
| (EC-)PDTCH/F (MCS-8)10,15 | 57,05 (114,1)11 | 1141 | - |
| (EC-)PDTCH/F (MCS-9)10,15 | 61,85 (123,7)11 | 1237 | - |
| PDTCH/F (DAS-5)13 | 23,75 (47,50)11 | 475 | - |
| PDTCH/F (DAS-6)13 | 28,55 (57,10)11 | 571 | - |
| PDTCH/F (DAS-7)13 | 34,15 (68,30)11 | 683 | - |
| PDTCH/F (DAS-8)13 | 46,90 (93,80)11 | 938 | - |
| PDTCH/F (DAS-9)13 | 56,50 (113,00)11 | 1130 | - |
| PDTCH/F (DAS-10)13 | 67,60 (135,20)11 | 1352 | - |
| PDTCH/F (DAS-11)13 | 84,40 (168,80)11 | 1688 | - |
| PDTCH/F (DAS-12)13 | 101,20 (202,40)11 | 2024 | - |
| PDTCH/F (DBS-5)13 | 23,80 (47,60)11 | 476 | - |
| PDTCH/F (DBS-6)13 | 31,00 (62,00)11 | 620 | - |
| PDTCH/F (DBS-7)13 | 46,85 (93,70)11 | 937 | - |
| PDTCH/F (DBS-8)13 | 61,25 (122,50)11 | 1225 | - |
| PDTCH/F (DBS-9)13 | 69,95 (139,90)11 | 1399 | - |
| PDTCH/F (DBS-10)13 | 91,559 (183,10)11 | 1831 | - |
| PDTCH/F (DBS-11)13 | 112,30 (224,60)11 | 2246 | - |
| PDTCH/F (DBS-12)13 | 121,90 (243,80)11 | 2438 | - |
| PDTCH/F (UAS-7)14 | 47,65 (95,30)11 | 953 | - |
| PDTCH/F (UAS-8)14 | 57,25 (114,50)11 | 1145 | - |
| PDTCH/F (UAS-9)14 | 62,05 (124,10)11 | 1241 | - |
| PDTCH/F (UAS-10)14 | 71,00 (142,00)11 | 1420 | - |
| PDTCH/F (UAS-11)14 | 80,60 (161,20)11 | 1612 | - |
| PDTCH/F (UBS-5)14 | 24,20 (48,40)11 | 484 | - |
| PDTCH/F (UBS-6)14 | 31,40 (62,80)11 | 628 | - |
| PDTCH/F (UBS-7)14 | 47,55 (95,10)11 | 951 | - |
| PDTCH/F (UBS-8)14 | 61,95 (123,90)11 | 1239 | - |
| PDTCH/F (UBS-9)14 | 70,95 (141,90)11 | 1419 | - |
| PDTCH/F (UBS-10)14 | 92,55 (185,10)11 | 1851 | - |
| PDTCH/F (UBS-11)14 | 113,60 (227,20)11 | 2272 | - |
| PDTCH/F (UBS-12)14 | 123,20 (246,40)11 | 2464 | - |
| PDTCH/H (MCS-1)10 | 5,3 | 212 | - |
| PDTCH/H (MCS-2)10 | 6,5 | 260 | - |
| PDTCH/H (MCS-3)10 | 8,3 | 332 | - |
| PDTCH/H (MCS-4)10 | 9,7 | 388 | - |
| PDTCH/H (MCS-5)10 | 12,025 | 481 | - |
| PDTCH/H (MCS-6)10 | 15,625 | 625 | - |
| PDTCH/H (MCS-7)10 | 23,725 | 949 | - |
| PDTCH/H (MCS-8)10 | 28,525 | 1141 | - |
| PDTCH/H (MCS-9)10 | 30,925 | 1237 | - |
| full rate FACCH (FACCH/F) | 9,2 | 184 | 20 |
| half rate FACCH (FACCH/H) enhanced circuit switched full rate FACCH (E-FACCH/F) | 4,6 9,2 | 184 184 | 40 20 |
| full rate octal FACCH (O-FACCH/F) | 9,2 | 184 | 20 |
| half rate octal FACCH (O-FACCH/H) | 4,6 | 184 | 40 |
| SDCCH | 598/765 ( 0,782) | 184 | 3 060/13 (235) |
| SACCH (with TCH)4 | 115/300 ( 0,383) | 168 + 16 | 480 |
| SACCH (with SDCCH)4 | 299/765 ( 0,391) | 168 + 16 | 6 120/13 ( 471) |
| PACCH/F7 | - | 181 (204)12 | - |
| EC-PACCH/D15,16,17,18,19 | - | 80 | - |
| EC-PACCH/U15,16,17,18,20 | - | 64 | - |
| EC-PACCH/U/48 | - | 56 |  |
| PACCH/H7 | - | 181 |  |
| BCCH | 598/765 ( 0,782) | 184 | 3 060/13 ( 235) |
| EC-BCCH | 598/ (8*765) ( 0,098) | 184 | 8*3 060/13 (1 883) |
| PBCCH6 | s*181/120 (1,508) | 181 | 120 |
| AGCH5 | n*598/765 ( 0,782) | 184 | 3 060/13 ( 235) |
| EC-AGCH, CC1 | 18*286/765 (6,729)21 25*286/765 (9,346)22 | 88 | 3 060/13 ( 235) |
| EC-AGCH, CC2 | 4*286/(2*765) (0,748)21 6*286/(2*765) (1,122)22 | 88 | 2*3 060/13 ( 471) |
| EC-AGCH, CC3 | 2*286/(2*765) ( 0,374)21 3*286/(2*765) ( 0,561)22 | 88 | 2*3 060/13 ( 471) |
| EC-AGCH, CC4 | 2*286/(4*765) (0,187)21 3*286/(4*765) ( 0,280)22 | 88 | 4*3 060/13 ( 942) |
| PAGCH7 |  | 181 |  |
| NCH5 | m*598/765 ( 0,782) | 184 | 3 060/13 ( 235) |
| PCH5 | p*598/765 ( 0,782) | 184 | 3 060/13 ( 235) |
| EC-PCH, CC1 | 16*88*650/153 ( 5,982) | 88 | 3 060/13 ( 235) |
| EC-PCH, CC2 | 4*88*650/(2*153) ( 0,748) | 88 | 2*3 060/13 ( 471) |
| EC-PCH, CC3 | 2*88*650/(2*153) (0,374) | 88 | 2*3 060/13 ( 471) |
| EC-PCH, CC4 | 2*88*650/(4*153) (0,187) | 88 | 4*3 060/13 ( 942) |
| PPCH7 |  | 181 |  |
| RACH5 | r*26/765 ( 0,034) | 8 | 3 060/13 ( 235) |
| PRACH (8 bit Access Burst)7 | r*26/765 ( 0,034) | 8 |  |
| EC-RACH, 1 TS, CC1 (11 bit Access Burst) | 51*11*650/153 ( 2,383) | 11 | 3 060/13 ( 235) |
| EC-RACH, 1 TS, CC2 (11 bit Access Burst) | 12*11*650/153 ( 0,561) | 11 | 3 060/13 ( 235) |
| EC-RACH, 1 TS, CC3 (11 bit Access Burst) | 3*11*650/153 ( 0,140) | 11 | 3 060/13 ( 235) |
| EC-RACH, 1 TS, CC4 (11 bit Access Burst) | 2*11*650/(2*153) ( 0,047) | 11 | 2*3 060/13 ( 471) |
| EC-RACH, 2 TS, CC2 (11 bit Access Burst) | 25*11*650/153 ( 1,168) | 11 | 3 060/13 ( 235) |
| EC-RACH, 2 TS, CC3 (11 bit Access Burst) | 6*11*650/153 ( 0,280) | 11 | 3 060/13 ( 235) |
| EC-RACH, 2 TS, CC4 (11 bit Access Burst) | 4*11*650/(2*153) ( 0,093) | 11 | 2*3 060/13 ( 471) |
| PRACH (11 bit Access Burst)7 | r*143/3060 ( 0,047) | 11 |  |
| MPRACH (8 bit Access Burst)7 | r*26/765 ( 0,034) | 8 |  |
| MPRACH (11 bit Access Burst)7 | r*143/3060 ( 0,047) | 11 |  |
| CBCH | 598/765 ( 0,782) | 184 | 3 060/13 ( 235) |
| CTSBCH-SB | 25/240 ( 0,104) | 25 | 240 |
| CTSPCH | 184/240 ( 0,767) | 184 | 240 |
| CTSARCH | 14*25/240 ( 0,104) | 25 | 240 |
| CTSAGCH | 2*184/240 ( 0,767) | 184 | 240 |
| NOTE 1: For full rate speech, the block is divided into two classes according to the importance of the bits (182 bits for class I and 78 bits for class II). For enhanced full rate speech, the block is divided into two classes according to the importance of the bits (170 bits for class I and 74 bits for class II). |  |  |  |
| NOTE 2: For half rate speech, the block is divided into two classes according to the importance of the bits (95 bits for class I and 17 bits for class II). |  |  |  |
| NOTE 3: For data services, the net bit rate is the adaptation rate as defined in 3GPP TS 44.021. |  |  |  |
| NOTE 4: On SACCH, 16 bits are reserved for control information on layer 1, and 168 bits are used for higher layers. |  |  |  |
| NOTE 5: CCCH channels are common to all users of a cell; the total number of blocks (m, n, p, r) per recurrence period is adjustable on a cell by cell basis and depends upon the parameters (BS_CC_CHANS, BS_BCCH_SDCCH_COMB, BS_AG_BLKS_RES and NCP) broadcast on the BCCH and specified in 3GPP TS 45.002 and 3GPP TS 44.018. |  |  |  |
| NOTE 6: The total number of PBCCH blocks (s) is adjustable on a cell by cell basis and depends upon the parameter BS_PBCCH_BLKS broadcast on the first PBCCH block and specified in 3GPP TS 45.002 and 3GPP TS 44.018. |  |  |  |
| NOTE 7: The net bit rate for these channels in a cell can change dynamically and depends on how PDCH are configured in a cell, and upon the parameters BS_PBCCH_BLKS, BS_PAG_BLKS_RES and BS_PRACH_BLKS broadcast on the PBCCH and specified in 3GPP TS 45.002 and 3GPP TS 44.018, as well as upon how certain blocks on the PDCH are used (indicated by the message type). |  |  |  |
| NOTE 8: For adaptive half rate speech, the blocks are divided into two classes according to the importance of the bits (the first number in the block length corresponds to the class I bits, the second number corresponds to the class II bits). |  |  |  |
| NOTE 9: CTSBCH, CTSARCH, CTSPCH and CTSAGCH are only used in CTS. |  |  |  |
| NOTE 10: For EGPRS PDTCH, the block length in bits excludes the USF bits (downlink traffic) and all the error-check bits, but includes the tail bits. For transmission using FANR, they also exclude the PAN bits. For Turbo coded coding schemes (DAS-5 to DAS-12 and DBS-5 to DBS-12) the tail bits are not included in this calculation. |  |  |  |
| NOTE 11: The value outside the bracket applies for transmission using BTTI configuration, whereas the value in bracket refers to the case of transmission using RTTI configuration. |  |  |  |
| NOTE 12: The value in bracket applies to TBFs using FANR (see 3GPP TS 44.060), when transmission of the PACCH uses MCS-0. |  |  |  |
| NOTE 13: These channels are available only for the EGPRS2 downlink. |  |  |  |
| NOTE 14: These channels are available only for the EGPRS2 uplink. NOTE 15: Applies to EC operation for CC1. NOTE 16: Applies to EC operation for CC2 using 4 or 2 consecutive PDCHs. NOTE 17: Applies to EC operation for CC3 using 4 or 2 consecutive PDCHs. NOTE 18: Applies to EC operation for CC4 using 4 or 2 consecutive PDCHs. NOTE 19: Applies to all EC-PACCH/D configurations, i.e. EC-PACCH/D/1, EC-PACCH/D/4, EC-PACCH/D/8, EC-PACCH/D/16, EC-PACCH/D/2TS/4, EC-PACCH/D/2TS/8, EC-PACCH/D/2TS/16 NOTE 20: Applies to all EC-PACCH/U configurations, i.e. EC-PACCH/U/1, EC-PACCH/U/4, EC-PACCH/U/8, EC-PACCH/U/16, EC-PACCH/U/2TS/4, EC-PACCH/U/2TS/8, EC-PACCH/U/2TS/16 NOTE 21: Applies for an EC-CCCH mapped on timeslot 1. NOTE 22: Applies for an EC-CCCH mapped on timeslot 3, 5 or 7. NOTE 23: Uplink EC-PDTCH with 48 blind physical layer transmissions uses modified MCS-1 coding scheme MCS-1’ where the number of header bits are reduced compared to MCS-1. |  |  |  |

2nd Modification

# 7	Coding and interleaving

## 7.1	General

A brief description of the coding schemes that are used for the logical channels mentioned in clause 2, except packet traffic channels and packet control channels that can be found in sub clause 7.2, plus the synchronization channels (see subclause 5.2), is made in the following tables. For all the types of channels the following operations are made in this order:

-	external coding (block coding);

-	internal coding (convolutional or turbo coding);

-	interleaving.

After coding, the different channels (except RACH, EC-RACH, SCH, EC-SCH, CTSBCH-SB and CTSARCH) are constituted by blocks of coded information bits plus coded header (the purpose of the header is to distinguish between TCH and FACCH blocks). These blocks are interleaved over a number of bursts. The block size and interleaving depth are channel dependent. All these operations are specified in 3GPP TS 45.003.

For the adaptive speech traffic channels, a signaling codeword is attached to the block of coded information bits before interleaving. The signaling codeword is a block code representation of a 2-bits inband information word (rate ¼ for the adaptive full rate speech traffic channels, ½ for the adaptive half rate speech traffic channels, rate 1/6 for the adaptive half rate 8-PSK speech and 8-PSK wideband speech traffic channels, and rate 1/12 for the adaptive full rate 8-PSK wideband speech traffic channels).

| Type of channel | bits/block data+parity+tail1 | convolutional code rate | coded bits per block | interleaving depth |
| --- | --- | --- | --- | --- |
| TCH/FS |  |  | 456 | 8 |
| class I2 | 182 + 3 + 4 | ½ | 378 |  |
| class II | 78 + 0 + 0 | - | 78 |  |
| TCH/EFS |  |  | 456 | 8 |
| class I2 | 170 + 15 + 4 | ½ | 378 |  |
| class II | 74 + 4 + 0 | - | 78 |  |
| TCH/HS |  |  | 228 | 4 |
| class I3 | 95+3+6 | 104/211 | 211 |  |
| class II | 17+0+0 |  | 17 |  |
| TCH/AFS12.24 |  |  | 456 | 8 |
| Class I5 | 244 + 6 + 4 | 127/224 | 448 |  |
| TCH/AFS10.24 |  |  | 456 | 8 |
| Class I6 | 204 + 6 + 4 | 107/224 | 448 |  |
| TCH/AFS7.954 |  |  | 456 | 8 |
| Class I7 | 159 + 6 + 6 | 171/448 | 448 |  |
| TCH/AFS7.44 |  |  | 456 | 8 |
| Class I8 | 148 + 6 + 4 | 79/224 | 448 |  |
| TCH/AFS6.74 |  |  | 456 | 8 |
| Class I9 | 134 + 6 + 4 | 9/28 | 448 |  |
| TCH/AFS5.94 |  |  | 456 | 8 |
| Class I10 | 118 + 6 + 6 | 65/224 | 448 |  |
| TCH/AFS5.154 |  |  | 456 | 8 |
| Class I11 | 103 + 6 + 4 | 113/448 | 448 |  |
| TCH/AFS4.754 |  |  | 456 | 8 |
| Class I12 | 95 + 6 + 6 | 107/448 | 448 |  |
| TCH/AHS7.9513 |  |  | 228 | 4 |
| Class I14 | 123 + 6 + 4 | 133/188 | 188 |  |
| Class II | 36+0+0 |  | 36 |  |
| TCH/AHS7.413 |  |  | 228 | 4 |
| Class I15 | 120 + 6 + 4 | 65/98 | 196 |  |
| Class II | 28+0+0 |  | 28 |  |
| TCH/AHS6.713 |  |  | 228 | 4 |
| Class I16 | 110 + 6 + 4 | 3/5 | 200 |  |
| Class II | 24+0+0 |  | 24 |  |
| TCH/AHS5.913 |  |  | 228 | 4 |
| Class I17 | 102 + 6 + 4 | 7/13 | 208 |  |
| Class II | 16+0+0 |  | 16 |  |
| TCH/AHS5.1513 |  |  | 228 | 4 |
| Class I18 | 91 + 6 + 4 | 101/212 | 212 |  |
| Class II | 12+0+0 |  | 12 |  |
| TCH/AHS4.7513 |  |  | 228 | 4 |
| Class I19 | 83 + 6 + 6 | 95/212 | 212 |  |
| Class II | 12+0+0 |  | 12 |  |
| (continued) |  |  |  |  |

| (continued) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Type of channel | bits/block data+parity+tail1 | convolutional code rate | coded bits per block | interleaving depth |
| TCH/WFS12.654 |  |  | 456 | 8 |
| Class I23 | 253 + 6 + 4 | 263/448 | 448 |  |
| TCH/WFS8.854 |  |  | 456 | 8 |
| Class I24 | 177 + 6 + 4 | 187/448 | 448 |  |
| TCH/WFS6.64 |  |  | 456 | 8 |
| Class I25 | 132 + 8 + 4 | 9/28 | 448 |  |
| O-TCH/WFS23.8527 |  |  | 1368 | 8 |
| Class I23 | 477 + 6 + 6 | 163/448 | 1344 |  |
| O-TCH/WFS15.8527 |  |  | 1368 | 8 |
| Class I23 | 317 + 6 + 6 | 47/192 | 1344 |  |
| O-TCH/WFS12.6527 |  |  | 1368 | 8 |
| Class I23 | 253 + 6 + 6 | 14/71 | 1344 |  |
| O-TCH/WFS8.8527 |  |  | 1368 | 8 |
| Class I24 | 177 + 6 + 6 | 9/64 | 1344 |  |
| O-TCH/WFS6.627 |  |  | 1368 | 8 |
| Class I28 | 132 + 6 + 6 | 3/28 | 1344 |  |
| O-TCH/WHS12.6526 |  |  | 684 | 4 |
| Class I23 | 253 + 6 + 6 | 265/672 | 672 |  |
| O-TCH/WHS8.8526 |  |  | 684 | 4 |
| Class I24 | 177 + 6 + 6 | 9/32 | 672 |  |
| O-TCH/WHS6.626 |  |  | 684 | 4 |
| Class I28 | 132 + 6 + 6 | 3/14 | 672 |  |
| O-TCH/AHS12.226 |  |  | 684 | 4 |
| Class I5 | 244 + 6 + 6 | 8/21 | 672 |  |
| O-TCH/AHS10.226 |  |  | 684 | 4 |
| Class I6 | 204 + 6 + 6 | 9/28 | 672 |  |
| O-TCH/AHS7.9526 |  |  | 684 | 4 |
| Class I7 | 159 + 6 + 6 | 57/224 | 672 |  |
| O-TCH/AHS7.426 |  |  | 684 | 4 |
| Class I8 | 148 + 6 + 6 | 5/21 | 672 |  |
| O-TCH/AHS6.726 |  |  | 684 | 4 |
| Class I9 | 134 + 6 + 6 | 73/336 | 672 |  |
| O-TCH/AHS5.926 |  |  | 684 | 4 |
| Class I10 | 118 + 6 + 6 | 65/336 | 672 |  |
| O-TCH/AHS5.1526 |  |  | 684 | 4 |
| Class I11 | 103 + 6 + 6 | 115/672 | 672 |  |
| O-TCH/AHS4.7526 |  |  | 684 | 4 |
| Class I12 | 95 + 6 + 6 | 107/672 | 672 |  |
| TCH/F14.4 TCH/F9.6 | 290 + 0 + 4 4*60 + 0 + 4 | 294/456 244/456 | 294/456 456 | 19 19 |
| TCH/F4.8 | 60 + 0 + 16 | 1/3 | 228 | 19 |
| TCH/H4.8 | 4*60 + 0 + 4 | 244/456 | 456 | 19 |
| TCH/F2.4 | 72 + 0 + 4 | 1/6 | 456 | 8 |
| TCH/H2.4 | 72 + 0 + 4 | 1/3 | 228 | 19 |
| (continued) |  |  |  |  |

| (continued) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Type of channel | bits/block data+parity+tail1 | convolutional code rate | coded bits per block | interleaving depth |
| FACCH/F | 184 + 40 + 4 | ½ | 456 | 8 |
| E-FACCH/F | 184 + 40 + 4 | ½ | 456 | 4 |
| FACCH/H | 184 + 40 + 4 | ½ | 456 | 6 |
| O-FACCH/F | 184 + 40 + 6 | 1/6 | 1368 | 8 |
| O-FACCH/H | 184 + 40 + 6 | 1/6 | 1368 | 6 |
| SDCCHs, SACCHs20, BCCH, EC-BCCH, NCH, AGCH, PCH, CBCH | 184 + 40 + 4 | ½ | 456 | 4 |
| EC-AGCH, EC-PCH | 88+18+0 | 106/116 | 116 | 1 |
| EC-PACCH, downlink | 80+18+0 | 98/114 | 114 | 1 |
| EC-PACCH, uplink | 64+18+0 | 82/116 | 116 | 1 |
| EC-PACCH/48, uplink | 56+18+0 | 74/106 | 106 | 1 |
| SACCH/TP SACCH/MP | 184 + 18 + 6 + 4021 | ½ | 456 | 4 |
| E-IACCH | 3 | 1/822 | 24 | 4 |
| EPCCH | 3 | ¼22 | 12 | 1 |
| RACH EC-RACH | 8 + 6 + 4 11 + 6 + 4 | ½ ½ | 36 36 | 1 1 |
| EC-SCH29 | 30 + 10 + 4 | ½ | 78 | 1 |
| SCH | 25 + 10 + 4 | ½ | 78 | 1 |
| CTSBCH-SB | 25 + 10 + 4 | ½ | 78 | 1 |
| CTSPCH | 184 + 40 + 4 | ½ | 456 | 4 |
| CTSARCH | 25 + 10 + 4 | ½ | 78 | 1 |
| CTSAGCH | 184 + 40 + 4 | ½ | 456 | 4 |
| NOTE 1: The tail bits mentioned here are the tail bits of the convolutional code. |  |  |  |  |
| NOTE 2: The 3 parity bits for TCH/FS detect an error on 50 bits of class I. |  |  |  |  |
| NOTE 3: The 3 parity bits for TCH/HS detect an error on 22 bits of class I. |  |  |  |  |
| NOTE 4: For TCH/AFS and TCH/WFS an 8 bits in band signalling codeword is attached to the block of coded information before interleaving. A dedicated block structure to carry the comfort noise information associated with the adaptive full rate speech traffic channels is also specified in 3GPP TS 45.003. |  |  |  |  |
| NOTE 5: The 6 parity bits for TCH/AFS12.2 and O-TCH/AHS12.2 detect an error on 81 bits of class I. |  |  |  |  |
| NOTE 6: The 6 parity bits for TCH/AFS10.2 and O-TCH/AHS10.2 detect an error on 65 bits of class I. |  |  |  |  |
| NOTE 7: The 6 parity bits for TCH/AFS7.95 and O-TCH/AHS7.95 detect an error on 75 bits of class I. |  |  |  |  |
| NOTE 8: The 6 parity bits for TCH/AFS7.4 and O-TCH/AHS7.4 detect an error on 61 bits of class I. |  |  |  |  |
| NOTE 9: The 6 parity bits for TCH/AFS6.7 and O-TCH/AHS6.7 detect an error on 55 bits of class I. |  |  |  |  |
| NOTE 10: The 6 parity bits for TCH/AFS5.9 and O-TCH/AHS5.9 detect an error on 55 bits of class I. |  |  |  |  |
| NOTE 11: The 6 parity bits for TCH/AFS5.15 and O-TCH/AHS5.15 detect an error on 49 bits of class I. |  |  |  |  |
| NOTE 12: The 6 parity bits for TCH/AFS4.75 and O-TCH/AHS4.75 detect an error on 39 bits of class I. |  |  |  |  |
| NOTE 13: For TCH/AHS a 4 bits in band signalling codeword is attached to the block of coded information before interleaving A dedicated block structure to carry the comfort noise information associated with the adaptive half rate speech traffic channels is also specified in 3GPP TS 45.003. |  |  |  |  |
| NOTE 14: The 6 parity bits for TCH/AHS7.95 detect an error on 67 bits of class I. |  |  |  |  |
| NOTE 15: The 6 parity bits for TCH/AHS7.4 detect an error on 61 bits of class I. |  |  |  |  |
| NOTE 16: The 6 parity bits for TCH/AHS6.7 detect an error on 55 bits of class I. |  |  |  |  |
| NOTE 17: The 6 parity bits for TCH/AHS5.9 detect an error on 55 bits of class I. |  |  |  |  |
| NOTE 18: The 6 parity bits for TCH/AHS5.15 detect an error on 49 bits of class I. |  |  |  |  |
| NOTE 19: The 6 parity bits for TCH/AHS4.75 detect an error on 39 bits of class I. |  |  |  |  |
| NOTE 20: with the exception of SACCH/TP and SACCH/MP |  |  |  |  |
| NOTE 21: 40 uncoded dummy bits are inserted for the mapping of the enhanced power control signalling |  |  |  |  |
| NOTE 22: block code is applied |  |  |  |  |
| NOTE 23: The 6 parity bits for TCH/WFS12.65, O-TCH/WFS23.85, O-TCH/WFS15.85, O-TCH/WFS12.65 and O-TCH/WHS12.65 detect an error on 72 bits of class I. |  |  |  |  |
| NOTE 24: The 6 parity bits for TCH/WFS8.85, O-TCH/WFS8.85 and O-TCH/WHS8.85 detect an error on 64 bits of class I. |  |  |  |  |
| NOTE 25: The 8 parity bits for TCH/WFS6.60 detect an error on 54 bits of class I. |  |  |  |  |
| NOTE 27: For O-TCH/WFS a 24 bits in band signalling codeword is attached to the block of coded information before interleaving. A dedicated block structure to carry the comfort noise information associated with the adaptive full rate 8PSK wideband speech traffic channels is also specified in 3GPP TS 45.003. |  |  |  |  |
| NOTE 28: The 6 parity bits for O-TCH/WFS6.6 and O-TCH/WHS6.6 detect an error on 54 bits of class I. NOTE 29: Two additional information bits available to the physical layer are encoded as a cyclic shift of the mapping of the burst on the physical channel as specified in in 3GPP TS 45.003. |  |  |  |  |

| Type of channel | bits/block data+parity+tail1 | Reed-Solomon code rate | convolutional code rate | coded bits per block | interleaving depth |
| --- | --- | --- | --- | --- | --- |
| E-TCH/F43.2 E-TCH/F32.0 E-TCH/F28.8 | 870 + 0 + 6 640 + 0 + 6 580 + 0 + 6 | N/A N/A 73/85 | 876/1368 646/1392 686/1368 | 1368 1392 1368 | 19 12 19 |

## 7.2	Packet Traffic and Control Channels

All packet traffic and control channels, except PRACH and EC-PACCH, use rectangular interleaving of one Radio Block over four bursts in consecutive TDMA frames of one PDCH. As an exception, in RTTI configuration, the PDTCH uses rectangular interleaving of one Radio Block over four bursts in two consecutive TDMA frames of a PDCH-pair.

### 7.2.1	Channel coding for PDTCH

#### 7.2.1.1	Channel coding for GPRS PDTCH

Four different coding schemes, CS-1 to CS-4, are defined for the GPRS Radio Blocks carrying RLC data blocks. For the Radio Blocks carrying RLC/MAC Control blocks code CS-1 is always used. The exceptions are messages that use the existing Access Burst (see 3GPP TS 45.003, e.g. Packet Channel Request). An additional coding scheme is defined for the Access Burst that includes 11 information bits.

The first step of the coding procedure is to add a Block Check Sequence (BCS) for error detection. For CS-1 - CS-3, the second step consists of pre-coding USF (except for CS-1), adding four tail bits and a convolutional coding for error correction that is punctured to give the desired coding rate. For CS-4 there is no coding for error correction.

The details of the codes are shown in the table below, including:

-	the length of each field;

-	the number of coded bits (after adding tail bits and convolutional coding);

-	the number of punctured bits;

-	the data rate, including the RLC header and RLC information.

| Scheme | Code rate | USF | Pre-coded USF | Radio Block excl. USF and BCS | BCS | Tail | Coded bits | Punctured bits |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CS-1 | ½ | 3 | 3 | 181 | 40 | 4 | 456 | 0 |
| CS-2 | »2/3 | 3 | 6 | 268 | 16 | 4 | 588 | 132 |
| CS-3 | »3/4 | 3 | 6 | 312 | 16 | 4 | 676 | 220 |
| CS-4 | 1 | 3 | 12 | 428 | 16 | - | 456 | - |

CS-1 is the same coding scheme as specified for SDCCH. It consists of a half rate convolutional code for FEC and a 40 bit FIRE code for BCS (and optionally FEC). CS-2 and CS-3 are punctured versions of the same half rate convolutional code as CS-1 for FEC. CS-4 has no FEC.

The USF has 8 states, which are represented by a binary 3 bit field in the MAC Header.

All coding schemes are mandatory for MSs supporting GPRS. Only CS-1 is mandatory for the network.

#### 7.2.1.2	Channel coding for EGPRS and EGPRS2 PDTCH

Nine different modulation and coding schemes, MCS-1 to MCS-9, are defined for the EGPRS Radio Blocks (4 bursts, 20ms in case of BTTI configuration or 10ms in case of RTTI configuration) carrying RLC data blocks. Further, 16 more modulation and coding schemes, DAS-5 to DAS-12 and DBS-5 to DBS-12, are defined for EGPRS2 downlink radio blocks (4 bursts, 20ms in case of BTTI configuration or 10ms in case of RTTI configuration) carrying RLC data blocks and 13 more modulation and coding schemes, UAS-7 to UAS-11 and UBS-5 to UBS-12, are defined for EGPRS2 uplink radio blocks (4 bursts, 20ms in case of BTTI configuration or 10ms in case of RTTI configuration) carrying RLC data blocks. For the Radio Blocks carrying RLC/MAC Control blocks code CS-1 is always used, except in the case of RTTI configuration, when code MCS-0 may be used in the downlink (see subclause 7.2.2). The exceptions are messages that use the existing Access Burst (see 3GPP TS 45.003, e.g. Packet Channel Request). An additional coding scheme is defined for the Access Burst that includes 11 information bits.

To ensure strong header protection, the header part of the Radio Block is independently coded from the data part of the Radio Block (8 bit CRC calculated over the header -excl. USF- for error detection, followed by rate 1/3 convolutional coding – and possibly puncturing – for error correction). Additionally, for transmission using FANR, a PAN field is included which is independently coded from the header and data parts of the Radio Block (the coding is the same as for the header part).

For EGPRS (see 3GPP TS 44.060), the MCSs are divided into different families A, B and C. Each family has a different basic unit of payload (see 3GPP TS  43.064). Different code rates within a family are achieved by transmitting a different number of payload units within one Radio Block. For families A and B, 1, 2 or 4 payload units are transmitted, for family C, only 1 or 2 payload units are transmitted.

For EGPRS2 (see 3GPP TS 44.060), the coding schemes are divided into different families A, B and C. Each family has a different basic unit of payload (see 3GPP TS 43.064). Different code rates within a family are achieved by transmitting a different number of payload units within one Radio Block. For family A, 1, 2, 4, 6 or 8 payload units are transmitted, for family B, 1, 2, 3, 4, 6 or 9 payload units are transmitted, for family C, only 1 or 2 payload units are transmitted.

For EGPRS, when 4 payload units are transmitted (MCS-7, MCS-8 and MCS-9), these are splitted into two separate RLC blocks (i.e. with separate sequence numbers and block check sequences).

For EGPRS2, when 4 payload units are transmitted (UAS-7 to UAS-9, UBS-7, UBS-8, DAS-8, DAS-9, DBS-7 and DBS-8), these are split into two separate RLC blocks (i.e. with separate sequence numbers and block check sequences). When 6 payload units are transmitted (UAS-10, UAS-11, UBS-9, UBS-10, DAS-10, DAS-11, DBS-9 and DBS-10), these are split into three separate RLC blocks (i.e. with separate sequence numbers and block check sequences), except for DAS-10 for which they are split into two separate RLC blocks. When 8 payload units are transmitted (DBS-11, DBS-12, UBS-11 and UBS-12), these are split into four separate RLC blocks (i.e. with separate sequence numbers and block check sequences). When 9 payload units are transmitted (DAS-12), these are split into three separate RLC blocks (i.e. with separate sequence numbers and block check sequences).

# The first step of the coding procedure is to add a Block Check Sequence (BCS) for error detection.

In case of convolutional coding, the second step consists of adding six tail bits (TB) and a 1/3 rate convolutional coding for error correction that is punctured to give the desired coding rate.

In case of turbo coding, the second step consists of 1/3 rate turbo coding for error correction (see 3GPP TS 45.003) that is punctured to give the desired coding rate.

The USF has 8 states, which are represented by a binary 3 bit field in the MAC Header. The USF is encoded to 12 symbols for bursts using legacy symbol rate and 16 symbols for bursts using higher symbol rate, (this results in12 bits for GMSK modes, 32 bits for QPSK modes, 36 bits for 8PSK modes, 48 bits for 16-QAM modes at legacy symbol rate (See 3GPP TS 45.004), 64 bits for 16-QAM at higher symbol rate (see 3GPP TS 45.004), 60 bits for 32-QAM at legacy symbol rate and 80 bits for 32-QAM at higher symbol rate).

MSs supporting EGPRS shall support MCS-1 to MCS-9 in downlink and MCS-1 to MCS-4 in uplink. In case an MS supporting EGPRS is 8-PSK capable in uplink, it shall also support MCS-5 to MCS-9 in uplink. A network supporting EGPRS may support only some of the MCSs.

MSs supporting EGPRS2 (see 3GPP TS 44.060) shall support EGPRS coding schemes MCS-1 to MCS-9 in downlink and uplink.

MSs supporting EGPRS2-A (see 3GPP TS 44.060) in the downlink shall support DAS-5 to DAS-12.

MSs supporting EGPRS2-B (see 3GPP TS 44.060) in the downlink shall support DAS-5 to DAS-12 and DBS-5 to DBS-12.

MSs supporting EGPRS2-A in the uplink shall support UAS-7 to UAS-11.

MSs supporting EGPRS2-B in the uplink shall support UAS-7 to UAS-11 and UBS-5 to UBS-12.

A network supporting EGPRS2-A in downlink (respectively uplink) may support only some of DAS-5 to DAS-12 (respectively UAS-7 to UAS-11).

A network supporting EGPRS2-B in downlink (respectively uplink) may support only some of DBS-5 to DBS-12 (respectively UBS-5 to UBS-12).

The details of the EGPRS coding schemes are shown in Table 7.2.1.2.1. An exhaustive description of the EGPRS coding schemes can be found in 3GPP TS 45.003.

# Table 7.2.1.2.1 - Coding parameters for the EGPRS coding schemes

| Scheme | Code rate (Note 2) | Header Code rate | PAN Code rate (if present) | Modulation | RLC blocks per Radio Block (20ms) | Raw Data within one Radio Block (Note 1) | Family | BCS | Tail payload | HCS | PCS (if present) | Data rate kb/s (Note 3) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MCS-9 | 1,0 | 0,36 | n/a | 8PSK | 2 | 2x592 | A | 2x12 | 2x6 | 8 | 10 | 59,2 |
| MCS-8 | 0,92 (0,98) | 0,36 | 0,42 |  | 2 | 2x544 | A |  |  |  |  | 54,4 |
| MCS-7 | 0,76 (0,81) | 0,36 | 0,42 |  | 2 | 2x448 | B |  |  |  |  | 44,8 |
| MCS-6 | 0,49 (0,52) | 1/3 | 0,39 |  | 1 | 592 48+544 | A | 12 | 6 |  |  | 29,6 27,2 |
| MCS-5 | 0,37 (0,40) | 1/3 | 0,39 |  | 1 | 448 | B |  |  |  |  | 22,4 |
| MCS-4 | 1,0 | 0,53 | n/a | GMSK | 1 | 352 | C |  |  |  |  | 17,6 |
| MCS-3 | 0,85 (0,96) | 0,53 | 0,63 |  | 1 | 296 48+248 and 296 | A |  |  |  |  | 14,8 13,6 |
| MCS-2 | 0,66 (0,75) | 0,53 | 0,63 |  | 1 | 224 | B |  |  |  |  | 11,2 |
| MCS-1 | 0,53 (0,60) | 0,53 | 0,63 |  | 1 | 176 | C |  |  |  |  | 8,8 |
| Note 1: The italic caption indicates the 6 octets of padding when retransmitting MCS-8 block with MCS-3 or MCS-6. For MCS-3, the 6 octets of padding are sent every second block (see 3GPP TS 44.060). Note 2: The number in bracket indicates the coding rate for transmission using FANR, when the PAN is present. Note 3: These data rates are applicable for BTTI configuration. The data rates are doubled in case of RTTI configuration. |  |  |  |  |  |  |  |  |  |  |  |  |

The details of the EGPRS2 coding schemes are shown in the table below. An exhaustive description of the EGPRS2 coding schemes can be found in 3GPP TS 45.003.

# Table 7.2.1.2.2: Coding parameters for the EGPRS2 coding schemes

| Scheme | Code rate (Note 5) | Header Code rate | PAN code rate (if present) | Modulation | RLC blocks per Radio Block (20ms) | Raw Data within one Radio Block | Family | BCS | Tail payload | HCS | PCS | Data rate kb/s (Note 6) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DAS-12 | 0,96 (0,99) | 0,38 | 0.38 | 32QAM (Note 4) | 3 | 658 | B | 3x12 | (Note 7) | 8 | 10 | 98,4 |
| DAS-11 | 0,80 (0,83) | 038 | 0.38 |  | 3 | 546 | A | 3x12 | (Note 7) |  |  | 81,6 |
| DAS-10 | 0,64 (0,66) | 0,33 | 0.38 |  | 2 | 658 | B | 2x12 | (Note 7) |  |  | 65,6 |
| DAS-9 | 0,68 (0,70) | 0,34 | 0.38 | 16QAM (Note 4) | 2 | 546 | A | 2x12 | (Note 7) |  |  | 54,4 |
| DAS-8 | 0,56 (0,58) | 0,34 | 0.38 |  | 2 | 450 | B | 2x12 | (Note 7) |  |  | 44,8 |
| DAS-7 | 0,54 (0,57) | 033 | 0.38 | 8PSK (Note 4) | 1 | 658 | B | 12 | (Note 7) |  |  | 32,8 |
| DAS-6 | 0,45 (0,48) | 0,33 | 0.38 |  | 1 | 546 | A | 12 | (Note 7) |  |  | 27,2 |
| DAS-5 | 0,37 (0,39) | 0,33 | 0.38 |  | 1 | 450 | B | 12 | (Note 7) |  |  | 22,4 |
| DBS-12 | 0,98 (1,00) | 0,37 | 0.54 | 32QAM (Note 3) | 4 | 594 | A | 4x12 | (Note 7) |  |  | 118,4 |
| DBS-11 | 0,91 (0,93) | 0,37 | 0.38 |  | 4 | 546 | A | 4x12 | (Note 7) |  |  | 108,8 |
| DBS-10 | 0,72 (0,75) | 0,34 | 0.38 |  | 3 | 594 | A | 3x12 | (Note 7) |  |  | 88,8 |
| DBS-9 | 0,71 (0,.73) | 0,34 | 0.38 | 16QAM (Note 3) | 3 | 450 | B | 3x12 | (Note 7) |  |  | 67,2 |
| DBS-8 | 0,60 (0.63) | 0,31 | 0.38 |  | 2 | 594 | A | 2x12 | (Note 7) |  |  | 59,2 |
| DBS-7 | 0,47 (0,48) | 0,31 | 0.38 |  | 2 | 450 | B | 2x12 | (Note 7) |  |  | 44,8 |
| DBS-6 | 0,63 (069) | 0,31 | 0.38 | QPSK (Note 3) | 1 | 594 | A | 1x12 | (Note 7) |  |  | 29,6 |
| DBS-5 | 0,49 (0,53) | 0,31 | 0.38 |  | 1 | 450 | B | 1x12 | (Note 7) |  |  | 22,4 |
| UAS-11 | 0,95 (0,99) | 0,36 | 0.38 | 16QAM (Note 2) | 3 | 514 | A | 3x12 | 3x6 |  |  | 76,8 |
| UAS-10 | 0,84 (0,87 | 0,36 | 0.38 |  | 3 | 450 | B | 3x12 | 3x6 |  |  | 67,2 |
| UAS-9 | 0,71 (0,74) | 0,36 | 0.38 |  | 2 | 594 | A | 2x12 | 2x6 |  |  | 59,2 |
| UAS-8 | 0,62 (0,64) | 0,36 | 0.38 |  | 2 | 514 | A | 2x12 | 2x6 |  |  | 51,2 |
| UAS-7 | 0,55 (0,57) | 0,36 | 0.38 |  | 2 | 450 | B | 2x12 | 2x6 |  |  | 44,8 |
| UBS-12 | 0,96 (0,99) | 0,35 | 0.38 | 32QAM (Note 1) | 4 | 594 | A | 4x12 | 4x6 |  |  | 118,4 |
| UBS-11 | 0,89 (0,91) | 0,35 | 0.38 |  | 4 | 546 | A | 4x12 | 4x6 |  |  | 108,8 |
| UBS-10 | 0,71 (0,73) | 0,35 | 0.36 |  | 3 | 594 | A | 3x12 | 3x6 |  |  | 88,8 |
| UBS-9 | 0,70 (0,72) | 0,32 | 0.36 | 16QAM (Note 1) | 3 | 450 | B | 3x12 | 3x6 |  |  | 67,2 |
| UBS-8 | 0,60 (0,61) | 0,33 | 0.38 |  | 2 | 594 | A | 2x12 | 2x6 |  |  | 59,2 |
| UBS-7 | 0,46 (0,47) | 0,33 | 0.38 |  | 2 | 450 | B | 2x12 | 2x6 |  |  | 44,8 |
| UBS-6 | 0,62 (0,67) | 0,35 | 0.38 | QPSK (Note 1) | 1 | 594 | A | 12 | 6 |  |  | 29,6 |
| UBS-5 | 0,47 (0,51) | 0,35 | 0.38 |  | 1 | 450 | B | 12 | 6 |  |  | 22,4 |
| Note 1: These coding schemes use the higher symbol rate (See 3GPP TS 45.004) and are only available in the uplink Note 2: These coding schemes use the normal symbol rate (See 3GPP TS 45.004) and are only available in the uplink Note 3: These coding schemes use the higher symbol rate (See 3GPP TS 45.004) and are only available in the downlink Note 4: These coding schemes use the normal symbol rate (See 3GPP TS 45.004) and are only available in the downlink Note 5: The number in bracket indicates the code rate for transmission using FANR, when the PAN is present Note 6: These data rates apply in case of BTTI configuration, the data rates are doubled in case of RTTI configuration. Note 7: The turbo code is terminated using 2x3 tail bits resulting in 12 coded bits per RLC block. |  |  |  |  |  |  |  |  |  |  |  |  |

### 7.2.2	Channel coding for PACCH, PBCCH, PAGCH, PPCH, CPBCCH, CPAGCH, CPPCH, and CSCH

The channel coding for the PBCCH, PAGCH, PPCH, CPBCCH, CPAGCH, and CPPCH corresponds to the coding scheme CS-1. The channel coding for the PACCH corresponds to the coding scheme CS-1; in case of RTTI configuration, the channel coding for the downlink PACCH corresponds to the coding scheme MCS-0 if BTTI USF mode is used and may correspond to either CS-1 or MCS-0 if RTTI USF mode is used. The channel coding for the CSCH is identical to SCH.

### 7.2.3	Channel Coding for the PRACH and MPRACH

Two types of packet random access burst may be transmitted on the PRACH and MPRACH: an 8 information bits random access burst or an 11 information bits random access burst called the extended packet random access burst. The MS shall support both random access bursts. The channel coding used for the burst carrying the 8 data bit packet random access uplink message is identical to the coding of the random access burst on the RACH. The channel coding used for the burst carrying the 11 data bit packet random access uplink message is a punctured version of the coding of the random access burst on the RACH.

### 7.2.4	Channel coding for EC-PDTCH, EC-PACCH, EC-AGCH, EC-PCH, EC-BCCH and EC-SCH

The channel coding for the EC-BCCH corresponds to the coding scheme CS-1. EC-SCH, EC-PACCH, EC-AGCH, and EC-PCH are all defined by separate coding schemes, see 3GPP TS 45.003. The channel coding for the EC-PDTCH for coverage classes CC1 to CC4 is identical to PDTCH for EGPRS, see subclause 7.2.1.2, except for a limitation in puncturing schemes used for MCS-1 to MCS-4. The channel coding for the EC-PDTCH for coverage class CC5 is defined by a separate coding scheme MCS-1’ which uses MCS-1 for encoding of the data bits while adopting a reduced number of header bits, see 3GPP TS 45.003.