3GPP TSG RAN WG6 #3 	R6-170135

# Athens, Greece, February 13-17, 2017	                  Revision of R6-170101

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 45.002 | CR | 0205 | rev | 2 | Current version: | 14.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of Extended Access Burst for connectionless Multilateration Positioning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R6 |  |  |  |  |  |  |  |  |  |
| Work item code: | ePOS_GERAN-Core |  |  |  |  |  |  | Date: |  | 2017-02-03 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | The Extended Access Burst is used for Multilateration Timing Advance (MTA) for accessing neighbour cells that are not in extended coverage if indicated by the network in the Multilateration TA Request message and if the cell indicates support for MTA as depicted in R6-170047. The usage of Extended Access Burst within the Connectionless MTA procedure provides energy saving benefits compared to the Connection-oriented MTA procedure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In sub-clause 5.2.6 the abbreviation ‘DB’ for Dummy Burst is defined that is used in other specifications (e.g. in 45.005, Annex B). - In sub-clause 5.2.7 referencing errors to tables 5.2.7-3 and 5.2.7-4 are removed. - In sub-clause 5.2.9 the burst structure for the Extended Access Burst is specified and the abbreviation Extended AB is introduced. - In clause 7, tables 3 and 6a the burst structure is added for RACH and EC-RACH in case of CC1 selected for random channel access. - Changes in Revision 1 are marked in yellow. In particular the procedure is referred to as Multilateration Timing Advance using the Extended Access Burst method. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No burst format for the Extended Access Burst is defined, hence missed energy saving for MS requested to perform MTA. Inconsistency to stage 2 content. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.6, 5.2.7, 5.2.9 (new), 7 (tables 3 and 6a) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  | TS 43.059 CR 0083 TS 45.001 CR 0096 TS 45.003 CR 0144 TS 45.005 CR 0598 TS 44.018 CR 1060 TS 44.031 CR 0229 TS 48.008 CR 0411 TS 49.031 CR 0068 |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS 51.010 CR xxxx TS 51.021 CR xxxx |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

1st Modification

### 5.2.6	Dummy burst (DB)

| Bit Number |  | Length | Contents | Definition |
| --- | --- | --- | --- | --- |
| (BN) |  | of field | of field |  |
| 0 | - 2 | 3 | tail bits | (below) |
| 3 | - 144 | 142 | mixed bits | (below) |
| 145 | - 147 | 3 | tail bits | (below) |
| (148 | - 156 | 8,25 | guard period (bits) | subclause 5.2.8) |

-	where the "tail bits" are defined as modulating bits with states as follows:

(BN0, BN1, BN2) 	= (0, 0, 0)	and

(BN145, BN146, BN147)	= (0, 0, 0)

-	where the "mixed bits" are defined as modulating bits with states as follows:

(BN3, BN4 .. BN144)	=	(1, 1, 1, 1, 1, 0, 1, 1, 0, 1, 1, 1, 0, 1, 1, 0, 0, 0, 0, 0, 1, 0, 1,
		0, 0, 1, 0, 0, 1, 1, 1, 0, 0, 0, 0, 0, 1, 0, 0, 1, 0, 0, 0, 1, 0,
		0, 0, 0, 0, 0, 0, 1, 1, 1, 1, 1, 0, 0, 0, 1, 1, 1, 0, 0, 0, 1, 0, 1, 1, 1, 0, 
		0, 0, 1, 0, 1, 1, 1, 0, 0, 0, 1, 0, 1

0, 1, 1, 1, 0, 1, 0, 0, 1, 0, 1, 0, 0, 0, 1, 1, 0, 0, 1, 1, 0, 0, 1, 1, 1, 0, 0, 
1, 1, 1, 1, 0, 1, 0, 0, 1, 1, 1, 1, 1, 0, 0, 0, 1, 0, 0, 1, 0, 1, 1, 1, 1, 1, 0, 
1, 0, 1, 0)

### 5.2.7	Access burst (AB)

Table 5.2.7-1: Access Burst

| Bit Number (BN) | Length of field | Content of field | Definition |
| --- | --- | --- | --- |
| 0-7 | 8 | Extended tail bits | see table 5.2.7-2 |
| 8-48 | 41 | Synch. sequence bits | see table 5.2.7-3 and table 5.2.7-4 |
| 49-84 | 36 | Encrypted bits (e0..e35) | see 3GPP TS 45.003 |
| 85-87 | 3 | Tail bits | see table 5.2.7-2 |
| 88-155 88-156 | 68, 68.25, or 691 | Extended guard period (bits) | see subclause 5.2.8 |
| NOTE1: 68 or 69 always applies in case of EC-GSM-IoT access burst, see 3GPP TS 45.010 |  |  |  |

The “extended tail bits” and the “tail bits” are defined as modulating bits with states according to table 5.2.7-2.

Table 5.2.7-2: Tail bits and extended tail bits

| Tail bit type | Bit number (BN) | Modulating bits |
| --- | --- | --- |
| Extended tail bits | (BN0, BN1, BN2 .. BN7) | (0, 0, 1, 1, 1, 0, 1, 0) |
| Tail bits | (BN85, BN86, BN87) | (0, 0, 0) |

The “Synch. sequence bits” are defined as modulating bits with states according to table 5.2.7-3.

Table 5.2.7-3: Synch. Sequence bits

| Bit number (BN) | Modulating bits | Applicability |
| --- | --- | --- |
| (BN8, BN9 .. BN48) | (0, 1, 0, 0, 1, 0, 1, 1, 0, 1, 1, 1, 1, 1, 1, 1, 1, 0, 0, 1, 1, 0, 0, 1, 1, 0, 1, 0, 1, 0, 1, 0, 0, 0, 1, 1, 1, 1, 0, 0, 0) | unless explicitly stated otherwise this training (synchronization) sequence "TS0" shall be used (includes PEO and EC operation), see 3GPP TS 44.060 |
|  | (0, 1, 0, 1, 0, 1, 0, 0, 1, 1, 1, 1, 1, 0, 0, 0, 1, 0, 0, 0, 0, 1, 1, 0, 0, 0, 1, 0, 1, 1, 1, 1, 0, 0, 1, 0, 0, 1, 1, 0, 1) | in case alternative training (synchronization) sequence "TS1" is used except in EC operation. |
|  | (1, 1, 1, 0, 1, 1, 1, 1, 0, 0, 1, 0, 0, 1, 1, 1, 0, 1, 0, 1, 0, 1, 1, 0, 0, 0, 0, 0, 1, 1, 0, 1, 1, 0, 1, 1, 1, 0, 1, 1, 1) | in case alternative training (synchronization) sequence "TS2" is used except in EC operation. |
|  | (0,1, 0, 0, 0, 0, 1, 0, 1, 1, 0, 0, 0, 0, 0, 1, 1, 1, 0, 1, 0, 0, 1, 0, 1, 0, 1, 1, 1, 0, 1, 1, 1, 0, 0, 0, 1, 0, 0, 0, 0) | in case alternative training (synchronization) sequence "TS4" is used for PEO operation. |

Table 5.2.7-4: Synch. Sequence bits, in EC operation

| Bit number (BN) | Modulating bits | Uplink CC | Applicability |
| --- | --- | --- | --- |
| (BN8, BN9 .. BN48) | (1, 0, 0, 0, 1, 0, 0, 0, 1, 1, 1, 0, 1, 0, 1, 1, 1, 0, 1, 1, 0, 1, 0, 0, 0, 0, 0, 1, 0, 0, 0, 0, 1, 0, 1, 1, 0, 0, 0, 1, 0) | 1 | For operation on RACH or EC-RACH in case "TS3" is used1 |
|  | (0, 1, 0, 1, 0, 0, 0, 0, 1, 1, 1, 1, 1, 1, 1, 1, 0, 1, 0, 1, 1, 1, 0, 1, 0, 1, 1, 0, 1, 1, 0, 0, 1, 1, 0, 0, 1, 0, 1, 0, 0) | 2 | For operation on EC-RACH in case "TS5" is used1 |
|  | (0, 1, 0, 1, 1, 1, 1, 0, 0, 1, 1, 1, 0, 1, 0, 1, 1, 1, 1, 0, 1, 1, 0, 1, 0, 0, 0, 1, 0, 0, 1, 1, 0, 0, 0, 0, 1, 0, 1, 1, 1) | 3 | For operation on EC-RACH in case "TS6" is used1 |
|  | (0,1, 0, 0, 0, 0, 1, 0, 1, 1, 0, 0, 0, 0, 0, 1, 1, 1, 0, 1, 0, 0, 1, 0, 1, 0, 1, 1, 1, 0, 1, 1, 1, 0, 0, 0, 1, 0, 0, 0, 0) | 4 | For operation on EC-RACH in case "TS7" is used1 |
| NOTE1: see 3GPP TS 44.018 |  |  |  |

For information only

### 5.2.8	Guard period

The guard period is provided because it is required for the MSs that transmission be attenuated for the period between bursts with the necessary ramp up and down occurring during the guard periods as defined in 3GPP TS 45.005. A base transceiver station is not required to have a capability to ramp down and up between adjacent bursts, but is required to have a capability to ramp down and up for non-used time-slots, as defined in 3GPP TS 45.005. In any case where the amplitude of transmission is ramped up and down, then by applying an appropriate modulation bit stream interference to other RF channels can be minimized.

In some cases, there is more than one option for the duration of the guard period.  The actual duration of the guard period between bursts with different symbol rates is defined in 3GPP TS 45.010.

2nd Modification

### 5.2.9	Extended Access burst (Extended AB)

Table 5.2.9-1: Extended Access Burst

| Bit Number (BN) | Length of field | Content of field | Definition |
| --- | --- | --- | --- |
| 0-7 | 8 | Extended tail bits | see table 5.2.9-2 |
| 8-48 | 41 | Synch. sequence bits | see table 5.2.7-3 and table 5.2.7-4 |
| 49-84 | 36 | Encrypted bits part 1 (e0..e35) | see 3GPP TS 45.003 |
| 85-87 | 3 | Tail bits 1 | see table 5.2.9-2 |
| 88-144 | 57 | Encrypted bits part 2 (e36..e92) | see 3GPP TS 45.003 |
| 145-147 | 3 | Tail bits 2 | see table 5.2.9-2 |
| 148-155 148-156 | 8, 8.25, or, 91 | Guard period (bits) | see subclause 5.2.8 |
| NOTE1: 8 or 9 always applies in case of EC-GSM-IoT, see 3GPP TS 45.010 |  |  |  |

The “extended tail bits”, “tail bits 1” and “tail bits 2” are defined as modulating bits with states according to table   5.2.9-2.

Table 5.2.9-2: Tail bits and extended tail bits

| Tail bit type | Bit number (BN) | Modulating bits |
| --- | --- | --- |
| Extended tail bits | (BN0, BN1, BN2 .. BN7) | (0, 0, 1, 1, 1, 0, 1, 0) |
| Tail bits 1 | (BN85, BN86, BN87) | (0, 0, 0) |
| Tail bits 2 | (BN145, BN146, BN147) | (0, 0, 0) |

The “Synch. sequence bits” are defined as modulating bits with states according to table 5.2.7-3 and 5.2.7-4. They are identical to those used for the access burst.

3rd Modification

# 7	Mapping tables

Table 3 : Mapping of logical channels onto physical channels (see subclauses 6.3, 6.4, 6.5)

Channel	Sub-	Direction	Allowable	Allowable	Burst	Repeat	Interleaved block
designation	channel		timeslot	RF channel	type	length in	TDMA frame
	number		assignments	assignments		TDMA frames	mapping

FCCH		D	0	C0	FB	51	B0(0),B1(10),B2(20),B3(30),B4(40)

SCH		D	0	C0	SB	51	B0(1),B1(11),B2(21),B3(31),B4(41)

BCCH Norm		D	0,2,4,6	C0	NB	51	B(2..5)

BCCH Ext		D	0,2,4,6	C0	NB	51	B(6...9)

PCH		D	0,2,4,6	C0	NB	51	B0(6..9),B1(12..15),B2(16..19)
AGCH							B3(22..25),B4(26..29),B5(32..35),
							B6(36..39),B7(42..45),B8(46..49)

NCH		D	0	C0	NB	51	B0(6..9),B1(12..15),B2(16..19)
							B3(22..25),B4(26..29),B5(32..35),
							B6(36..39)

RACH		U	0,2,4,6	C0	AB, Extended AB2	51	B0(0),B1(1)..B50(50)

CBCH(SDCCH/4)		D	0	C0	NB	51	B(32..35)

CBCH(SDCCH/8)		D	0 ... 3	C0 ... Cn	NB	51	B(8..11)

SDCCH/4	0	D	0	C0	NB1	51	B(22..25)
		U					B(37..40)
	1	D					B(26..29)
		U					B(41..44)
	2	D					B(32..35)
		U					B(47..50)
	3	D					B(36..39)
		U					B(0..3)

SACCH/C4	0	D	0	C0	NB3	102	B(42..45)
		U					B(57..60)
	1	D					B(46..49)
		U					B(61..64)
	2	D					B(93..96)
		U					B(6..9)
	3	D					B(97..100)
		U					B(10..13)

NOTE 1:	An Access Burst (AB) is used on the uplink during handover.

NOTE 2: For Multilateration Timing Advance using the Extended Access Burst method, the Extended AB format is used.

NOTE 3:	An Access Burst (AB) may be used on the uplink during handover.

Table 6a : Mapping of Extended Coverage logical channels onto physical channels (see subclauses 6.3, 6.4, 6.5)

| Channel designation | Dir. | Allowable timeslot assignment | Allowable RF channel assignment | Burst type | Repeat length TDMA frames | Coverage class (CC) | Interleaved block TDMA frame mapping |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EC-SCH | D | 1 | C0 | SB | 204 | - | B([0,1,2,3,4,5,6] + 51 N), N=0,1,2,34 |
| EC-BCCH | D | 1 | C0 | NB | 408 | - | B([7,8,9,10,11,12,13,14]+51N), N=0,…,75 |
| EC-PCH | D | 1,3,5,7 | C0 | NB | 51 | 1 | B0(19,20),B1(21,22),…,B15(49,50) |
|  |  |  |  |  | 102 | 2 | B0([19,…,26] + 51N),B1([27,…,34] + 51N),…,B3([43,…,50] + 51N), N=0,13 |
|  |  |  |  |  | 102 | 3 | B0([19,…,34] + 51N), B1([35,…,50] + 51N), N=0,13 |
|  |  |  |  |  | 204 | 4 | B0([19,…,34] + 51N), B1([35,…,50] + 51N), N=0,1,2,34 |
| EC-AGCH | D | 1 | C0 | NB | 51 | 1 | B0(15,16),B1(17,18),…,B17(49,50) |
|  |  |  |  |  | 102 | 2 | B0([19,…,26] + 51N),B1([27,…,34] + 51N),…,B3([43,…,50] + 51N), N=0,13 |
|  |  |  |  |  | 102 | 3 | B0([19,…,34] + 51N), B1([35,…,50] + 51N), N=0,13 |
|  |  |  |  |  | 204 | 4 | B0([19,…,34] + 51N), B1([35,…,50] + 51N), N=0,1,2,34 |
|  |  | 3,5,7 |  |  | 51 | 1 | B0(1,2),B1(3,4),…,B24(49,50) |
|  |  |  |  |  | 102 | 2 | B0([3,…,10] + 51N),B1([11,…,18] + 51N),…,B5([43,…,50] + 51N), N=0,13 |
|  |  |  |  |  | 102 | 3 | B0([3,…,18] + 51N), B1([19,…,34] + 51N), B2([35,…,50] + 51N), N=0,13 |
|  |  |  |  |  | 204 | 4 | B0([3,…,18] + 51N), B1([19,…,34] + 51N), B2([35,…,50] + 51N), N=0,1,2,34 |
| EC-RACH, 1 TS | U | 1,3,5,7 | C0 | AB, Exten-ded AB7 | 51 | 1 | B0(0),B1(1),..B50(50) |
|  |  |  |  |  |  | 2 | B0(1,…,4), B1(5,…,8),...B11(45,…,48) |
|  |  |  |  |  |  | 3 | B0(2,…, 17),B1(18,…,33), B2(34,…,49) |
|  |  |  |  |  | 102 | 4 | B0([0,…,23] + 51N),B1([24,…,47] + 51N), N=0,13 |
| EC-RACH, 2 TS | U | 0,1, or, 2,3, or, 4,5, or, 6,7 | C0 | AB | 51 | 2 | B0(1,2,1’,2’), B1(3,4,3’,4’),...,B24(49,50,49’,50’) |
|  |  |  |  |  |  | 3 | B0(2,…,9,2’,…,9’), B1(10,…,17,10’,…,17’),...,B5(42,...,49,42’,...,49’) |
|  |  |  |  |  | 102 | 4 | B0([0,…,11,0’,…,11’] + 51N),…,B3([36,…,47,36’,…,47’] + 51N), N=0,13 |
| EC-PACCH, EC-PDTCH | D&U | 0…7 | C0…Cn | NB2 | 52 | 1 | B0(0...3), B1(4...7), B2(8...11), B3(13...16), B4(17...20), B5(21…24), B6(26...29), B7(30...33), B8(34...37), B9(39...42), B10(43...46), B11(47...50) |
|  |  |  |  |  |  | 2 | B0(0...3,0'...3',0''...3'',0'''...3'''),B1(4...7,4'...7',4''...7'',4'''...7'''), B2(8...11, 8'...11',8''...11'', 8'''...11'''), B3(13...16,13'...16',13''...16'',13'''...16'''), B4(17...20,17'...20',17''...20'',17'''...20'''), B5(21...24,21'...24',21''...24'',21'''...24'''), B6(26...29,26'...29',26''...29'',26'''...29'''), B7(30...33,30'...33',30''...33'',30'''...33'''), B8(34...37,34'...37',34''...37'',34'''...37'''), B9(39...42,39'...42',39''...42'',39'''...42'''), B10(43...46,43'...46',43''...46'',43'''...46'''), B11(47...50,47'...50',47''...50'',47'''...50''')1 |
|  |  |  |  |  |  | 3 | B0(0...7,0'...7',0''...7'',0'''...7'''), B1(8...11,13…16,8'...11',13'…16',8''...11'',13''…16'',8'''...11''',13''',…,16'''), B2(17...24, 17'...24',17''...24'', 17'''...24'''),B3(26...33,26'...33',26''...33'',26'''...33'''), B4(34...37,39…42,34'...37',39'…42',34''...37'',39''…42'',34'''...37''',39'''…42'''), B5(43...50,43'...50',43''...50'',43'''...50''')1 |
|  |  |  |  |  |  | 4 | B0(0...11,13…16,0'...11',13'…16',0''...11'',13''…16'',0'''...11''',13'''…16'''), B1(17...24,26…33,17'...24',26'…33',17''...24'',26''…33'',17'''...24''',26'''…33'''), B2(34...37,39...50,34'...37',39'...50',34’’...37’’,39'’...50'’,34'’'...37'’',39'''...50''')1 |
|  |  |  |  |  | 52 | 26 | B0(0..7,0'..7'), B1(8..11,13..16,8'..11',13'..16'), B2(17..24,17'..24'), B3(26..33,26'..33'), B4(34..37,39..42,34'..37',39'..42'), B5(43..50,43'..50') |
|  |  |  |  |  | 52 | 36 | B0(0..11,13..16,0’..11,13..16), B1(17..24,26..33, 17'..24',26'..33'), B2(34..37,39..50,34'..37',39'..50') |
|  |  |  |  |  | 104 | 46 | B0(0..11,13..24,26..33,0’..11’,13’..24’,26’..33’), B1(34..37,39..50,52..63,34’..37’,39’..50’,52’..63’), B2(65..76,78..90,92..103,65’..76’,78’..90’,92’..103’) |
| NOTE 1: EC-PDTCH and EC-PACCH are mapped onto four PDCHs. Number n indicates mapping on the PDCH with the lowest timeslot number in TDMA frame n, whereas number n', n'', and n''' indicates mapping on the PDCH with the second lowest, second highest, and highest timeslot number in TDMA frame n respectively. The four timeslots shall be consecutive. NOTE 2: Access Bursts (AB) instead of Normal Bursts (NB) may be used on the uplink as polling response on EC-PACCH/U. The same resources are used by the Access Burst. NOTE 3 N = (FN div 51) mod 2 NOTE 4 N = (FN div 51) mod 4 NOTE 5 N = (FN div 51) mod 8 NOTE 6: EC-PDTCH/2TS and EC-PACCH/2TS are mapped onto two PDCHs in case of blind physical layer transmissions. NOTE 7: For Multilateration Timing Advance using the Extended Access Burst method in EC operation, the Extended AB format is used in case of CC1 selected in downlink and uplink. |  |  |  |  |  |  |  |

End of Modifications