3GPP TSG-RAN6 Meeting #4	R6-170184

# Hangzhou, China, 15th May 2017 - 19th May 2017

| CR-Form-v11.2.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 44.060 | CR | 1641 | rev | - | Current version: | 14.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of enhanced DECOR for GERAN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eDecor |  |  |  |  |  | Date: |  |  | 2017-05-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Enhanced DECOR has been finalized in SA2#116 and for UTRAN and LTE in RAN#75. It needs to be introduced in GERAN specifications to support the functionality. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The possibility to transfer information about DCN-ID from the MS to the BSC in the PS domain is introduced. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Enhanced DECOR is not supported in GERAN system. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 3, 3.2, 5.5.0.1, 5.5.0.2 (new), 10.2.2, 10.3a.2.1, 10.3a.2.2, 10.4.14, 10.4.14a, 10.4.30 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS 44.018 CR 1065 TS 48.018 CR 0438 |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

First Change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

- References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

- For a specific reference, subsequent revisions do not apply.

- For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.060: "General Packet Radio Service (GPRS); Service Description; Stage 1".

[3]	3GPP TS 23.003: "Numbering, addressing and identification".

[4]	3GPP TS 43.013: "Discontinuous Reception (DRX) in the GSM system".

[5]	3GPP TS 43.064: " Overall description of GPRS radio interface; Stage 2".

[6]	3GPP TS 24.002: "GSM - UMTS Public Land Mobile Network (PLMN) access reference configuration".

[7]	3GPP TS 44.003: "Mobile Station - Base Station System (MS-BSS) interface; Channel structures and access capabilities".

[8]	3GPP TS 44.004: "Layer 1; General requirements".

[9]	3GPP TS 44.005: "Data Link (DL) layer; General aspects".

[10]	3GPP TS 24.007: "Mobile radio interface signalling layer 3; General aspects".

[11]	3GPP TS 44.018: "Mobile radio interface layer 3 specification; Radio Resource Control Protocol".

[12]	3GPP TS 44.064: " Mobile Station - Serving GPRS Support Node (MS-SGSN) Logical Link Control (LLC) Layer Specification".

[13]	3GPP TS 45.002: "Multiplexing and multiple access on the radio path".

[14]	3GPP TS 45.003: "Channel coding".

[15]	3GPP TS 45.008: "Radio subsystem link control".

[16]	3GPP TS 45.010: "Radio subsystem synchronization".

[17]	(void)

[18]	(void).

[19]	3GPP TS 23.060: "General Packet Radio Service (GPRS); Service Description; Stage 2".

[20]	3GPP TS 25.331: "Radio Resource Control (RRC) protocol specification".

[21]	3GPP TS 25.133: "Requirements for support of radio resource management (FDD)".

[22]	3GPP TS 25.123: "Requirements for support of radio resource management (TDD)".

[23]	ITU-T Recommendation T.4: "Standardization of Group 3 facsimile terminals for document transmission".

[24]	3GPP TS 22.011: "Service accessibility".

[25]	3GPP TS 25.101: "UE Radio Transmission and Reception (FDD)".

[26]	3GPP TS 25.102: "UTRA (UE) TDD; Radio transmission and reception".

[27]	3GPP TS 25.213: "Spreading and modulation (FDD)".

[28]	3GPP TS 25.223: "Spreading and modulation (TDD)".

[29]	3GPP TS 25.304: "UE Procedures in Idle Mode and Procedures for Cell Reselection in Connected Mode".

[30]	3GPP TS 45.005: "Radio transmission and reception".

[31]	3GPP TS 45.009: "Link adaptation".

[32]	TIA/EIA-IS-2000-5-A: "CDMA 2000 Series".

[33]	3GPP TS 51.010-1 (Rel-6): "Mobile Station (MS) conformance specification; Part 1: Conformance specification ".

[34]	3GPP TS 51.010-2 (Rel-6): "Mobile Station (MS) conformance specification; Part 2: Protocol Implementation Conformance Statement (ICS); proforma specification".

[35]	3GPP TS 51.010-3 (Rel-6): "Mobile Station (MS) conformance specification; Part 3: Layer 3 Abstract Test Suite (ATS)".

[36]	3GPP TS 51.010-4 (Rel-6): "Mobile Station (MS) conformance specification; Part 4: SIM application toolkit conformance specification".

[37]	3GPP TS 51.021: "Base Station System (BSS) equipment specification; Radio aspects".

[38]	3GPP TS 43.051: "GSM/EDGE Radio Access Network (GERAN); Overall Description - Stage 2".

[39]	3GPP TS 44.160: "Mobile Station (MS) - Base Station System (BSS) interface; Radio Link Control/ Medium Access Control (RLC/MAC) protocol in Iu mode".

[40]		Michel MOULY, "CSN.1 Specification, Version 2.0", Cell & Sys, ISBN: 2-9510062-0-9. 
Web: http://perso.wanadoo.fr/cell.sys/.

[41]	3GPP TS 25.224: "Physical Layer Procedures (TDD)".

[42]	3GPP TS 44.318: "Generic access to the A/Gb interface; Mobile GA interface layer 3 specification".

[43]		3GPP TS 43.129: "Packet-switched handover for GERAN A/Gb mode; Stage 2".

[44]	3GPP TS 43.318: "Generic access to the A/Gb interface; Stage 2".

[45]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification".

[46]	3GPP TS 36.104: "Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) radio transmission and reception".

[47]	3GPP TS 36.211: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical Channels and Modulation".

[48]	3GPP TS 23.041: "Technical realization of Cell Broadcast Service (CBS)".

[49]	3GPP TS 23.122: "Non-Access-Stratum (NAS) functions related to Mobile Station (MS) in idle mode".

[50]	3GPP TS 23.251: "Network sharing; Architecture and functional description".

[51]	3GPP TS 24.008: "Mobile Radio Interface Layer 3 specification; Core Network Protocols - Stage 3".

[52]	3GPP TS 24.368: "Non-Access Stratum (NAS) configuration Management Object (MO)".

[53]	3GPP TS 31.102: "Characteristics of the USIM Application".

[54]	3GPP TS 24.301: “General Packet Radio Service (GPRS) enhancements for  Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access”.

Second Change

# 3	Definitions, abbreviations and symbols

## 3.1	Definitions

This document uses the following definitions:

Additional PLMN: when network sharing is in use within a given cell, an Additional PLMN refers to a PLMN of which the PLMN ID is broadcast in the SYSTEM INFORMATION TYPE 22 message (see 3GPP TS 44.018)

A/Gb mode: mode of operation of the MS when connected to the Core Network via GERAN and the A and/or Gb interfaces.

Basic radio block period: the time needed to transmit a radio block on one PDCH using a basic TTI configuration i.e. four TDMA frames.

Blind Physical Layer Transmissions: See 3GPP TS 43.064

Block period: block period is the sequence of four timeslots on a PDCH used to convey one radio block

Broadcast/multicast receive mode: In broadcast/multicast receive mode, the mobile station is receiving upper layer PDUs on packet data physical channels used for point-to-multipoint transmission (see sub-clause 5.3.1); it is not allocated any additional radio resource on packet data physical channels. Broadcast/multicast receive mode is a sub-state of packet idle mode. The mobile station listens to the PBCCH and PCCCH or, if those are not provided by the network, to the BCCH and CCCH.

BTTI configuration: a configuration in which a radio block is sent using one PDCH in each of four consecutive TDMA frames. In a BTTI configuration, one radio block can be transmitted in a basic radio block period.

BTTI USF mode: a mode in which the USF is received on one PDCH of a downlink PDCH-pair during four consecutive TDMA frames.

Cell Change Notification (CCN): See sub-clause 5.5.1.1a.

Common PLMN: when network sharing is in use within a given cell, the Common PLMN refers to the PLMN of which the PLMN ID is contained in the Location Area Identification IE broadcast in the SYSTEM INFORMATION TYPE 3 and SYSTEM INFORMATION TYPE 4 messages (see 3GPP TS 44.018, 3GPP TS 23.251 and 3GPP TS 24.008).

Corresponding PDCH pair: The Corresponding PDCH pair associated with an uplink PDCH pair which forms part of an uplink TBF assignment is the downlink PDCH pair which is monitored by the mobile station for the USF for the TBF and on which PACCH/D is transmitted. The Corresponding PDCH pair associated with a downlink PDCH pair which forms part of an downlink TBF assignment is the uplink PDCH pair on which PACCH/U is transmitted.

Coverage Class: See 3GPP TS 43.064.

CSG Cells Reporting: refers to the ability of the mobile station to report measurements, CSG-ID and routing parameters of CSG cells in packet transfer mode. Routing parameters include Cell Identity and optionally the PLMN-ID and (for E-UTRAN cells only) the Tracking Area Code.

Downlink Dual Carrier: Downlink Dual Carrier is a feature allowing resources (for uplink and/or downlink TBFs and/or dedicated resources) to be assigned to a mobile station on up to two radio frequency channels within the same frequency band.

Downlink Dual Carrier configuration: A Downlink Dual Carrier configuration is one in which the mobile station has radio resources assigned over two radio frequency channels. In packet transfer mode, RLC/MAC blocks for uplink TBFs can only be transmitted on one radio frequency channel in any given radio block period, and RLC/MAC blocks for downlink TBFs can be transmitted on two radio frequency channels in any given radio block period. In dual transfer mode, uplink RLC/MAC blocks can be transmitted only on the radio frequency channel on which the dedicated resource is assigned.

Downlink Multi Carrier (DLMC): Downlink Multi Carrier is a feature allowing resources for a downlink TBF (and optionally an uplink TBF) to be assigned to a mobile station on two or more radio frequency channels in the same or different frequency bands using EGPRS or EGPRS2-A.

DLMC configuration: A DLMC configuration is one in which a mobile station in packet transfer mode has a downlink TBF where radio resources are configured on two or more radio frequency channels in the same or in different frequency bands. A mobile station in a DLMC configuration with an uplink TBF can only transmit RLC/MAC blocks for the uplink TBF on one radio frequency channel in any given radio block period whereas RLC/MAC blocks for the downlink TBF can be transmitted on one or more frequency channels in any given radio block period.

DTM handover: DTM handover is a feature used by the network to command a mobile station to move from its old (source) cell to a new (target) cell while operating in dual transfer mode and continue the operation of its ongoing circuit switched service and one or more of its ongoing packet switched services in the new cell. The mobile station is allocated one circuit switched radio resource and packet switched radio resources applicable to the new cell within a DTM HANDOVER COMMAND message.

Dual transfer mode: In dual transfer mode, the mobile station is allocated radio resources providing an RR connection (3GPP TS 44.018) and one or more Temporary Block Flows on one or more packet data physical channels. The allocation of radio resource for the RR connection and the Temporary Block Flow(s) is co-ordinated by the network in agreement with the capabilities of the mobile station in dual transfer mode.

Dynamic Timeslot Reduction: Dynamic Timeslot Reduction (DTR) is a feature used by the network to reduce the number of downlink timeslots monitored by the mobile station during inactivity periods of an EGPRS TBF. It is applicable when extended uplink TBF mode and/or delayed downlink TBF release are used.

Early TBF establishment: Procedure applicable when both the network and the mobile station support the extended uplink TBF mode, where the network keeps the uplink TBF open (by means of the extended uplink TBF mode operation) upon explicit request from the mobile station that pre-allocation is required. This allows the possibility to pre-allocate a TBF before actual data is ready for transmission.

EC-GSM-IoT: See 3GPP TS 43.064.

EC operation: See 3GPP TS 43.064.

EC TBF: a TBF operating in EC TBF mode.

EC TBF mode: refers to a TBF supporting extended coverage by operation on the EC-PDTCH and EC-PACCH channels. A TBF in EC TBF mode may consist of multiple EC-PDTCH and EC-PACCH channels if in coverage class 1 according to defined multislot capablility of the mobile station. The EC TBF uses the EGPRS Modulation and Coding Schemes with GMSK modulation (MCS-1 to MCS-4) and optionally, with 8-PSK modulation (MCS-5 to MCS-9). An uplink TBF in EC TBF mode is allocated using fixed uplink allocation.

EGPRS: Enhanced GPRS enables higher data rates through usage of 8PSK modulation in addition to GMSK. EGPRS also enables Incremental Redundancy operation.

EGPRS TBF mode: refers to a TBF utilising the EGPRS enhancements (e.g. Incremental Redundancy and possibly 8-PSK) or the EGPRS2 enhancements.

EGPRS TBF: refers to a TBF utilising the EGPRS enhancements, e.g. Incremental Redundancy and possibly 8-PSK.

EGPRS2: In the downlink direction, Enhanced GPRS Phase 2 enables higher data rates through usage of higher symbol rate, turbo codes, and QPSK, 16-QAM and 32-QAM modulations in addition to (or instead of) GMSK and 8PSK. In the uplink direction, Enhanced GPRS Phase 2 enables higher data rates through usage of higher symbol rate, and QPSK, 16-QAM and 32-QAM modulations in addition to (or instead of) GMSK and 8PSK. EGPRS2 consists of EGPRS2-A and EGPRS2-B in both directions.

EGPRS2-A: In the downlink direction, Enhanced GPRS Phase 2 Level A enables higher data rates through usage of turbo codes, and 16-QAM and 32-QAM modulations in addition to GMSK and 8-PSK. In the uplink direction, Enhanced GPRS Phase 2 Level A enables higher data rates through usage of 16-QAM modulation in addition to GMSK and 8-PSK.

EGPRS2-B: In the downlink direction, Enhanced GPRS Phase 2 Level B enables higher data rates through usage of higher symbol rate, turbo codes, and QPSK, 16-QAM and 32-QAM modulations in addition to GMSK or instead of 8PSK. In the uplink direction, Enhanced GPRS Phase 2 Level B enables higher data rates through usage of higher symbol rate, and QPSK, 16-QAM and 32-QAM modulations in addition to GMSK or instead of 8PSK.

EGPRS2 TBF: refers to a TBF utilising the EGPRS2 enhancements in the direction of data transfer, downlink or uplink. More specifically, in each direction, downlink or uplink, an EGPRS2-A TBF refers to a TBF utilising the EGPRS2-A enhancements and an EGPRS2-B TBF refers to a TBF utilising the EGPRS2-B enhancements. An EGPRS2 TBF operates in EGPRS TBF mode.

EGPRS-GMSK only TBF: refers to a TBF in EGPRS TBF mode but making only use of MCS-1 to MCS-4 modulation and coding schemes. The number of PDCHs assigned to an EGPRS-GMSK only TBF can be extended to the maximum number of timeslots compatible with the GPRS multislot class of the MS. This mode is determined by the mobile station based on the aggregate timeslot allocation assigned by the network. In the case the aggregate timeslot allocation is not within the indicated EGPRS multislot class, but is within the indicated GPRS multislot class, a mobile station supporting this mode shall consider the TBF to be in EGPRS-GMSK only mode (see sub-clause 9.1.9.2). This mode is only applicable in packet transfer mode.

Enhanced Flexible Timeslot Assignment (EFTA): Enhanced Flexible Timeslot Assignment gives the network the possibility to allocate one and the same mobile station uplink and downlink PDCH resources that overlap in time. This mobile station shall then for these overlapping instances prioritize uplink radio block transmission over attempting to read downlink radio blocks (unless performing pre-emptive retransmissions), but shall always attempt to read downlink radio blocks if it has nothing to transmit If it does not need to use all its allocated uplink PDCH resources, the mobile station shall transmit its uplink radio blocks in such a way that the number of downlink radio blocks it can read is maximized. Both network and mobile station must support extended uplink TBF mode in order for Enhanced Flexible Timeslot Assignment, EFTA, to be supported.

Enhanced Multiplexing for Single RLC Entity (EMSR): Refers to the multiplexing of upper layer data flows associated with multiple PFCs with at least two of them having unique TFIs within the context of a single RLC entity for single TBF operation. EMSR may only be used on a TBF operating in EGPRS TBF mode. EMSR is not applicable to Multiple TBF operation or when Exclusive Allocation is used.

Enhanced Multiplexing for Single TBF (EMST): refers to the multiplexing of upper layer data flows using one or more RLC entities on a single TBF where each RLC entity operates in a different RLC mode. EMST is applicable only to TBFs in EGPRS TBF mode. EMST is not applicable to Multiple TBF operation.

Extended EARFCNs: refers to extended EARFCN value range (0 …262143) with EARFCNs coded over 18 bits. MS and network support of extended EARFCNs is mandatory when network sharing is supported; support of extended EARFCNs is optional otherwise.
When network sharing is in use within a given cell, the network shall provide extended EARFCNs within the SYSTEM INFORMATION TYPE 23 message, if broadcasted in the cell; additionally when extended EARFCNs is in use within a given cell, the network may provide at least one EARFCN value > 65535 within the SYSTEM INFORMATION TYPE 2quater message.

Extended uplink TBF mode: In the extended uplink TBF mode, the uplink TBF may be maintained during temporary inactive periods, where the mobile station has no RLC information to send. The network determines the release of the uplink TBF (see sub-clause 9.3.1b).

Fast Ack/Nack Reporting (FANR): Fast Ack/Nack Reporting enables the use of a PAN field within an RLC/MAC block for EGPRS data transfer or for EGPRS2 data transfer. FANR enables the mobile station to transmit in the uplink direction a PAN field corresponding to a downlink TBF. Similarly FANR enables the network to transmit in the downlink direction a PAN field corresponding to an uplink TBF.

Fixed Uplink Allocation: Static allocation (and assignment) of limited resources in the uplink over one or more TTIs, using one or more PDCHs, that does not make use of USF based allocation.

Flexible Timeslot Assignment (FTA): Flexible Timeslot Assignment allows increased flexibility in the assignment of PDCH resources by letting some of the limitations imposed by the mobile stations multislot class (see 3GPP 45.002) to be respected on an allocation basis (i.e. per radio block period) rather than on an assignment basis (i.e. at TBF setup time).

GAN Cell: A cell under control of a GANC.

GAN Mode: MS mode of operation where the MS is connected to the Core Network via a GANC and the A and/or Gb interfaces.

GPRS multislot class / EGPRS multislot class: refers to the different mobile station capabilities to transmit and receive on different combinations of multiple PDCHs. The multislot classes are defined in 3GPP TS 45.002. Note that the mobile station may indicate different multislot classes for circuit mode services for GPRS and for EGPRS (see 3GPP TS 24.008). Different multislot class mobile stations are capable of supporting different medium access modes (see sub-clause 5.2.4).

GPRS TBF mode: refers to a TBF not utilising EGPRS or EGPRS2.

Inter-band reception: see 3GPP TS 45.005.

IR: Incremental redundancy, enables higher data rates through combining information from different transmissions of RLC data blocks when decoding. Also known as Hybrid Type II/III ARQ.

# Iu mode: mode of operation of the MS when connected to the Core Network via GERAN or UTRAN and the Iu interface.

# MS assisted Dedicated Core Networks selection: MS assisted Dedicated Core Network selection is an optional feature wherein the selection of the Dedicated Core Network to which a BSS routes LLC PDUs is based on, in addition to the selected PLMN, the “DCN-ID” value provided by the MS (see 3GPP TS 23.401 [49]).

# MAC-dedicated state: a MAC-control-entity state where a DBPSCH is assigned and no SBPSCH is assigned. This state only applies in Iu mode.

# MAC-DTM state: a MAC-control-entity state where at least one DBPSCH and one SBPSCH are assigned. This state only applies in Iu mode.

# MAC-idle state: a MAC-control-entity state where no basic physical subchannels are assigned. This state only applies in Iu mode.

# MAC-shared state: a MAC-control-entity state where at least one SBPSCH is assigned. This state only applies in Iu mode.

MCS: Modulation and Coding Scheme.

MS multislot class: refers to GPRS multislot class in case of a GPRS TBF mode or EGPRS-GMSK only TBF mode. In case of EGPRS TBF mode, MS multislot class refers to EGPRS multislot class.

Multiple TBF procedures: A mobile station that supports multiple TBF procedures can support one or more concurrent TBFs in either direction while in packet transfer mode (A/Gb mode). A network that supports multiple TBF procedures can support one or more concurrent TBFs in either direction for a mobile station that supports multiple TBF procedures in packet transfer mode (A/Gb mode).

Multiple TTI (MTTI) configuration: A mobile station that supports MTTI Configurations may be assigned a TBF using both RTTI resources (PDCH-pairs) and BTTI resources (PDCHs).

Network sharing: network sharing is an optional feature that allows different core network operators to connect to the same shared radio access network (see 3GPP TS 23.251). When network sharing is in use within a given cell, the network broadcasts within system information the PLMN identities of the PLMNs sharing the cell. A mobile station supporting network sharing uses this information for its PLMN (re)selection processes and indicates the selected PLMN to the BSS.

Non-contiguous intra-band reception: see 3GPP TS 45.005.

Non-extended uplink TBF mode: Where a distinction is needed, an uplink TBF, not operating in the extended uplink TBF mode, is referred as operating in the non-extended uplink TBF mode.

Non-synchronized PS handover: The basic type of PS handover which is used when the time bases of the involved cells bear no particular relationship between themselves and when the MS cannot predict the timing advance to be used in the target cell before access in this cell. The requirements that apply for the non-synchronized PS handover are given in 3GPP TS 45.010.

Packet access failure: Packet access failure refers to the access cases where the mobile station is explicitly denied access to the network, i.e. is not allowed to transmit (EGPRS) PACKET CHANNEL REQUEST or MPRACH PACKET CHANNEL REQUEST messages or receives a PACKET QUEUING NOTIFICATION message or a PACKET ACCESS REJECT message.

Packet flow context: Packet Flow Context (PFC) procedures are described in 3GPP TS 23.060. A Packet Flow Identifier (PFI) is used to identify a PFC.

Packet idle mode: In packet idle mode, the mobile station is prepared to transfer LLC PDUs on packet data physical channels (see sub-clause 5.3). The mobile station is not allocated any radio resource on a packet data physical channel; it listens to the PBCCH and PCCCH or, if those are not provided by the network, to the BCCH and the CCCH.

Packet transfer mode: In packet transfer mode, the mobile station is prepared to transfer LLC PDUs on packet data physical channels (see sub-clause 5.4). The mobile station is allocated radio resource on one or more packet data physical channels for the transfer of LLC PDUs.

Piggy-backed Ack/Nack (PAN) field: A Piggy-backed Ack/Nack field provides acknowledgement status of downlink (respectively uplink) RLC data blocks within an uplink (respectively downlink) RLC/MAC block for data transfer.

Power Efficient Operation (PEO): Used by a MS to reduce its power consumption through the use of relaxed mobility related requirements and the use of eDRX or PSM - see 3GPP TS 23.060 [3]. A PEO capable MS in a cell supporting PEO, that has negotiated the use of eDRX or PSM is said to have enabled PEO.

Pre-synchronized PS handover: A type of PS handover where the MS uses the timing advance included in the PS HANDOVER COMMAND message for immediate use in the target cell. The requirements that apply for the pre-synchronized PS handover are given in 3GPP TS 45.010.

PS handover: PS handover is a feature used by the network to command a mobile station to move from its old (source) cell to a new (target) cell while operating in packet transfer mode and continue the operation of one or more of its ongoing packet switched services in the new cell using TBF resource allocations provided within a PS HANDOVER COMMAND message. For PS handover to a GAN cell the mobile station receives the assignment of the target cell radio resources prior to receiving the PS HANDOVER COMMAND message.

Radio block: A radio block is the sequence of four normal bursts carrying one RLC/MAC protocol data units (see 3GPP TS 44.004). (The one exception is a radio block occasionally used on PACCH consisting of a sequence of four access bursts, each carrying a repetition of one short RLC/MAC block.). A radio block is sent either on a PDCH (BTTI configuration) or a PDCH-pair (RTTI configuration).

Random access failure: Random access failure refers to the access case when the mobile station does not get any response from the network to its (EGPRS) PACKET CHANNEL REQUEST or MPRACH PACKET CHANNEL REQUEST messages.

Random values: In a number of places in this Technical Specification, it is mentioned that some value must take a "random" value, in a given range, or more generally with some statistical distribution. For such random values refer to 3GPP TS 44.018.

Reduced Latency: refers to the use of FANR either in BTTI configuration or in RTTI configuration. Reduced Latency capability is required for the use of MTTI configurations.

Reduced radio block period: the time needed to transmit a radio block on a PDCH-pair using a reduced TTI configuration i.e. two TDMA frames.

RLC/MAC block: A RLC/MAC block is the protocol data unit exchanged between RLC/MAC entities (see clause 10 and 3GPP TS 44.004).

RLC/MAC control block: A RLC/MAC control block is the part of an RLC/MAC block carrying a control message between RLC/MAC entities (see sub-clause 10.3).

RLC data block: A RLC data block is the part of a RLC/MAC block carrying user data or signalling data received from an upper layer (see sub-clause 10.2).

RLC Non-Persistent Mode: A mode of RLC operation where retransmissions are possible but it is not required that all RLC data blocks are correctly received at the receiving RLC endpoint.

RR connection: An RR connection is a physical connection established between a mobile station and the network to support the upper layers’ exchange of information flows. An RR connection is maintained and released by the two peer entities.

RRC connection: An RRC connection is a point-to-point, bi-directional, logical connection between RRC peer entities in the mobile station and the GERAN characterised by the allocation of a G-RNTI. A mobile station has either zero or one RRC connections. RRC connections only apply in Iu mode.

RTTI configuration: a configuration in which a radio block is sent using two PDCHs (PDCH-pair) in either the first two or the last two TDMA frames of any given basic radio block period.

RTTI USF mode: a mode in which the USF is received on a downlink PDCH-pair during two consecutive TDMA frames.

Source BSS: Within the context of a PS handover the source BSS is the BSS controlling the cell in which the mobile station is camping prior to performing PS handover (i.e. it controls the old cell).

Synchronized PS handover: A type of PS handover which is possible when the time bases of the involved cells are synchronized, and for which no timing advance needs to be provided to the MS. The requirements that apply for the synchronized PS handover are given in 3GPP TS 45.010.

Target BSS: Within the context of a PS handover the target BSS is the BSS controlling the cell in which the mobile station is camping after successful completion of PS handover (i.e. it controls the new cell). If the same BSS controls both the old cell and the new cell associated with the PS handover of any given mobile station then the source BSS and the target BSS are the same.

TBF abort: The term "abort" as applied to TBF is used when the TBF is abruptly stopped without using the Release of TBF procedures defined in clause 9.

TBF release: The term "release" as applied to TBF is used when the TBF is stopped using one of the Release of TBF procedures defined in clause 9.

Temporary Block Flow (TBF): A Temporary Block Flow is, in A/Gb mode, a physical connection used by the two RR peer entities to support the unidirectional transfer of LLC PDUs on packet data physical channels (see sub-clause 5.2.1). In Iu mode, a TBF is a logical connection offered by two MAC entities to support the unidirectional transfer of RLC PDUs on basic physical subchannels.

Timer Expiry: A started timer has run the time specified.

Timer Restart: A timer that may already be running is stopped and then started again to run the time specified.

Timer Start: A timer is started to run the time specified.

Timer Stop: A started timer is stopped and its value is then undefined.

Unique Frequency Parameter Set (UFPS):  see 3GPP TS 45.008

Uplink control timeslot: refers to the timeslot number of the timeslot where the PACCH/U for the MS is located in case of a BTTI configuration, or to the uplink PDCH pair where the PACCH/U for the MS is located in case of an RTTI configuration. This parameter is set to the value of the Uplink Control timeslot IE in an assignment message. Otherwise, this parameter is unassigned. In A/Gb mode, this parameter is applicable only to multiple TBF procedures.

Uplink State Flag (USF): The Uplink State Flag (USF) is used on PDCH channel(s) to allow multiplexing of uplink Radio blocks from different TBFs belonging to the same or different mobile stations (see sub-clause 5.2.3, clause 10 and 3GPP TS 45.002).

Upper-layer PDU: An upper-layer PDU is, in A/Gb mode, an LLC PDU and, in Iu mode, an RRC message, a PDCP PDU or a PDU from the NAS user plane.

Third Change

## 3.2	Abbreviations

This document uses abbreviations from 3GPP TR 21.905 and 3GPP TS 43.064. It also uses the following abbreviations:

ARI	Access Request Identifier

ARQ	Automatic Repeat reQuest

AS	Access Stratum

BCCH	Broadcast Control CHannel

BSS	Base Station Subsystem

BTTI	Basic Transmission Time Interval

CBCH	Cell Broadcast CHannel

CC	Call Control

CC	Coverage Class

CCCCH	Compact CCCH

CCCH	Common Control CHannel

CCN	Cell Change Notification

CN	Core Network

CPBCCH	Compact PBCCH

CS-i	GPRS Coding Scheme i

DAS-i	EGPRS2 Downlink level A modulation and coding Scheme i

DBS-i	EGPRS2 Downlink level B modulation and coding Scheme i

DC	Dedicated Control

DCN-ID	Dedicated Core Network ID, see TS 23.401

DLC	Data Link Control

DLMC	Downlink Multi Carrier

DBPSCH	Dedicated Basic Physical Sub CHannel

DTR	Dynamic Timeslot Reduction

EC-AGCH	Extended Coverage Access Grant CHannel

EC-BCCH	Extended Coverage BroadCast CHannel

EC-CCCH	Extended Coverage Common Control CHannel

EC-PCH	Extended Coverage Paging CHannel

EC-RACH	Extended Coverage Random Access CHannel

EC-SCH	Extended Coverage Synchronization CHannel

ECSD	Enhanced Circuit Switched Data

EC SI	Extended Coverage System Information

EDGE	Enhanced Data rates for Global Evolution

E-FACCH	Enhanced FACCH

EGPRS	Enhanced General Packet Radio Service

EGPRS2	EGPRS Phase 2

eNB	E-UTRAN NodeB

E-TCH	Enhanced TCH

eTFI	extended Temporary Flow Identifier

EUTRA	Evolved UTRA

E-UTRAN	Evolved UTRAN

FACCH	Fast Associated Control CHannel

FANR	Fast Ack/Nack Reporting

GAN	Generic Access Network

GANC	Generic Access Network Controller

GC	General Control

GERAN	Gsm/Edge Radio Access Network

GPRS	General Packet Radio Service

GRA	Geran Registration Area

G-RNTI	Geran Radio Network Temporary Identity

GSM	Global System for Mobile communications

IETF	Internet Engineering Task Force

IMSI	International Mobile Subscriber Identity

IP	Internet Protocol

LCS	LoCation Services

LLC	Logical Link Control

MAC	Medium Access Control

MCS-i	EGPRS Modulation and Coding Scheme i

MM	Mobility Management

MME	Mobility Management Entity

MPRACH	MBMS Packet Random Access Channel

MS	Mobile Station

NAS	Non Access Stratum

NSAPI	Network-layer SAPI

Nt	Notification

O-FACCH	Octal FACCH

O-TCH	Octal TCH

PAN	Piggy-backed Ack/Nack

PBCCH	Packet BCCH

PDCH	Packet Data CHannel

PDCP	Packet Data Convergence Protocol

PDP	Packet Data Protocol

PDTCH	Packet Data TCH

PDU	Protocol Data Unit

PEO	Power Efficient Operation

PFC	Packet Flow Context

PFI	Packet Flow Identifier

PLMN	Public Land Mobile Network

PTCCH	Packet Timing-advance Control CHannel

p-t-m	point-to-multipoint

P-TMSI	Packet TMSI

QoS	Quality of Service

RAB	Radio Access Bearer

RANAP	Radio Access Network Application Part

RB	Radio Bearer

RLC	Radio Link Control

RNC	Radio Network Controller

RNS	Radio Network Subsystem

RNSAP	Radio Network Subsystem Application Part

ROHC	Robust Header Compression

RR	Radio Resource

RRC	Radio Resource Control

RTP	Real Time Protocol

RTTI	Reduced Transmission Time Interval

SACCH	Slow Associated Control CHannel

SAP	Service Access Point

SAPI	Service Access Point Identifier

SDCCH	Stand-alone Dedicated Control CHannel

SDU	Service Data Unit

SBPSCH	Shared Basic Physical Sub CHannel

TBF	Temporary Block Flow

TCH	Traffic Channel

TCP	Transmission Control Protocol

TFI	Temporary Flow Identifier

TLLI	Temporary Logical Link Identifier

TMSI	Temporary Mobile Subscriber Identity

TSC	Training Sequence Code

TTI	Transmission Time Interval

UAS-i	EGPRS2 Uplink level A modulation and coding Scheme i

UBS-i	EGPRS2 Uplink level B modulation and coding Scheme i

UDP	User Datagram Protocol

UMTS	Universal Mobile Telecommunication System

USF	Uplink State Flag

UTRAN	UMTS Terrestrial Radio Access Network

Fourth Change

#### 5.5.0.1 Indication of a selected PLMN using PLMN INdex

A mobile station supporting network sharing shall indicate the selected PLMN to the network when sending an upper layer PDU using a foreign TLLI or a random TLLI (see 3GPP TS 23.003) address for this transmission. The mobile station shall set the value of the Selected PLMN Index field in the first RLC data block used to transfer the PDU (see sub-clauses 10.4.14 and 10.4.14a.) to whichever of the following was the most recently received:

-	the corresponding PLMN ID index of the selected PLMN, if the selected PLMN is acquired from the system information when the SI22IND field contained in the Control Channel Description IE in the SYSTEM INFORMATION TYPE 3 message (see 3GPP TS 44.018) indicates that network sharing is used; or

-	the Selected PLMN Index received in the PS HANDOVER COMMAND message; or

-	the Selected PLMN index as indicated by the PLMN Index IE received in the CS HANDOVER COMMAND or DTM HANDOVER COMMAND (see 3GPP TS  44.018); or

-	for a mobile station in EC operation, the corresponding PLMN ID index of the selected PLMN, if the selected PLMN is acquired from the EC SYSTEM INFORMATION TYPE 4 message (see 3GPP TS 44.018).

Fifth Change

#### 5.5.0.2	Indication of DCN-ID

A mobile station supporting MS assisted Dedicated Core Network selection shall indicate the DCN-ID (within its selected PLMN) to a network supporting MS assisted Dedicated Core Network selection when sending an upper layer PDU using a foreign TLLI or a random TLLI (see 3GPP TS 23.003) address for this transmission. The mobile station shall set the value of the DCN-ID in the first RLC data block used to transfer the PDU (see sub-clauses 10.4.14 and 10.4.14a.) according to the DCN-ID value indicated by upper layers (see 3GPP TS 23.401 [54] and 3GPP TS 24.008 [51]).

Sixth Change

### 10.2.2	Uplink RLC data block

The Uplink RLC data block together with its MAC header is formatted as shown in figure 10.2.2.1.

| Bit |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 | 6 | 5 | 4 | 3 |  | 2 | 1 |  |
| Payload Type |  | Countdown Value |  |  |  |  | SI | R | MAC header |
| spare | PI | TFI |  |  |  |  |  | TI | Octet 1 |
|  |  |  | BSN |  |  |  |  | E | Octet 2 |
| Length indicator |  |  |  |  |  |  | M | E | Octet 3 (optional) |
| spare |  |  |  |  | Selected PLMN Index |  |  |  | Octet 4 (note 2) (optional) |
| . . . |  |  |  |  |  |  |  |  | . . . |
| Length indicator |  |  |  |  |  |  | M | E | Octet M (optional) |
|  |  |  |  |  |  |  |  |  | Octet M+1 \ |
|  |  |  | TLLI |  |  |  |  |  | Octet M+2 } (optional) |
|  |  |  |  |  |  |  |  |  | Octet M+3 / |
|  |  |  |  |  |  |  |  |  | Octet M+4 / |
| Length indicator |  |  |  |  |  | M |  | E | Octet M + 5 (Note 3) (Optional) |
| DCN-ID (octet 1) |  |  |  |  |  |  |  |  | Octet M+6 |
| DCN-ID (octet 2) |  |  |  |  |  |  |  |  | Octet M+7 |
| PFI |  |  |  |  |  |  |  | E | Octet M + 8 / |
|  |  |  |  |  |  |  |  |  | Octet M+9 |
| RLC data |  |  |  |  |  |  |  |  | . . . |
|  |  |  |  |  |  |  |  |  | Octet N-1 |
|  |  |  |  |  |  |  |  |  | Octet N |
|  |  | spare |  |  |  |  | spare |  | (if present) |

NOTE 1:	The field mapping convention for GPRS (sub-clause 10.0b.3.1) applies. According to that, in particular regarding the TLLI field, the most significant byte of the TLLI value shall be mapped on octet M+1 and the least significant byte of the TLLI value shall be mapped on octet M+4 of the uplink RLC data block.

NOTE 2:	This octet is only included if the value of the first instance of the Length indicator in the RLC data block is 62.

NOTE 3: 	This octet and the following two octets are only included if both the network and MS support MS assisted Dedicate Core Network selection (see 3GPP TS 44.018).

Figure 10.2.2.1: Uplink RLC data block with MAC header

Seventh Change

#### 10.3a.2.1	EGPRS Uplink RLC data block

The EGPRS uplink RLC data block is formatted according to figure 10.3a.2.1.1.

|  |  |  | Bit |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 2 | 1 |  |  |  |  |  |
|  |  |  | TI | E |  |  |  |  |  |
| Bit |  |  |  |  |  |  |  |  |  |
| 8 | 7 | 6 | 5 | 4 | 3 |  | 2 | 1 |  |
| Length indicator |  |  |  |  |  |  |  | E | Octet 1 (note 1) (optional) |
| spare |  |  |  |  |  | Selected PLMN Index |  |  | Octet 2 (note 3) (optional) |
| . . . |  |  |  |  |  |  |  |  | . . . |
| Length indicator |  |  |  |  |  |  |  | E | Octet M (optional) |
|  |  |  |  |  |  |  |  |  | Octet M+1 \ |
|  |  |  | TLLI |  |  |  |  |  | Octet M+2 } (optional) |
|  |  |  |  |  |  |  |  |  | Octet M+3 / |
|  |  |  |  |  |  |  |  |  | Octet M+4 / |
| Length indicator |  |  |  |  |  |  |  | E | Octet M + 5 (Note 4) (Optional) |  |
| DCN-ID (octet 1) |  |  |  |  |  |  |  |  | Octet M+6 |  |
| DCN-ID (octet 2) |  |  |  |  |  |  |  |  | Octet M+7 |  |
| PFI |  |  |  |  |  |  |  | E | Octet M+8 / |  |
|  |  |  |  |  |  |  |  |  | Octet M+9 |
| RLC data |  |  |  |  |  |  |  |  | . . . |
|  |  |  |  |  |  |  |  |  | Octet N2-1 |
|  |  |  |  |  |  |  |  |  | Octet N2 |

NOTE 1:	If padding is used, then "Octet 1" shall be replaced by "Octet 7", see example in annex J.

NOTE 2:	The field mapping convention for EGPRS (sub-clause 10.0b.3.2) applies. According to that, in particular regarding the TLLI field, the least significant byte of the TLLI value shall be mapped on octet M+1 and the most significant byte of the TLLI value shall be mapped on octet M+4 of the uplink EGPRS RLC data block.

NOTE 3:	This octet is only included if the value of the first instance of the Length indicator in the RLC data block is 123.

NOTE 4:	This octet and the following two octets are only included if both the network and MS support MS assisted Dedicate Core Network selection (see 3GPP TS 44.018)..

Figure 10.3a.2.1.1: Uplink EGPRS RLC data block

Eighth Change

#### 10.3a.2.2	EC-GSM-IoT Uplink RLC data block

For an EC TBF, the uplink RLC data block is formatted according to figure 10.3a.2.2.1.

|  |  |  | Bit |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 2 | 1 |  |  |  |  |  |  |
|  |  |  | TI | E |  |  |  |  |  |  |
| Bit |  |  |  |  |  |  |  |  |  |  |
| 8 | 7 | 6 | 5 | 4 | 3 |  | 2 | 1 |  |  |
| Length indicator |  |  |  |  |  |  |  | E |  | Octet 1 (note 1) (optional) |
| spare |  |  |  |  |  | Selected PLMN Index |  |  |  | Octet 2 (note 3) (optional) |
| . . . |  |  |  |  |  |  |  |  |  | . . . |
| Length indicator |  |  |  |  |  |  |  | E |  | Octet M (optional) |
|  |  |  |  |  |  |  |  |  |  | Octet M+1 \ |
|  |  |  | TLLI |  |  |  |  |  |  | Octet M+2 } (optional) |
|  |  |  |  |  |  |  |  |  |  | Octet M+3 / |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Octet M+4 / |  |
| Length indicator |  |  |  |  |  |  |  |  | E | Octet M + 5 (Note 4) (Optional) |
| DCN-ID (octet 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Octet M+6 |  |
| DCN-ID (octet 2) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Octet M+7 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Octet M+8 |  |
| RLC data |  |  |  |  |  |  |  |  |  | . . . |
|  |  |  |  |  |  |  |  |  |  | Octet N2-1 |
|  |  |  |  |  |  |  |  |  |  | Octet N2 |

NOTE 1:	If padding is used, then "Octet 1" shall be replaced by "Octet 7", see example in annex J.

NOTE 2:	The field mapping convention for EC-GSM-IoT (sub-clause 10.0b.3.2) applies. According to that, in particular regarding the TLLI field, the least significant byte of the TLLI value shall be mapped on octet M+1 and the most significant byte of the TLLI value shall be mapped on octet M+4 of the uplink EC-GSM-IoT RLC data block.

NOTE 3:	This octet is only included if the value of the first instance of the Length indicator in the RLC data block is 123.

NOTE 4:	This octet and the following two octets are only included if both the network and MS support MS assisted Dedicate Core Network selection (see 3GPP TS 44.018).

Figure 10.3a.2.2.1: Uplink EC-GSM-IoT RLC data block

Ninth Change

### 10.4.14	Length Indicator (LI) field in GPRS TBF mode and DCCH TBF mode (Iu mode)

The Length Indicator is used to delimit Upper Layer PDUs within the RLC data block. Additionally for Iu mode, a Length Indicator of value 63 may be used to indicate the presence of filler information within the RLC data block. If the first length indicator in the RLC data block is set to 63, then the entire RLC data block contains filler information.

A Length Indicator of value 62 is used by a mobile station supporting network sharing to indicate the presence of a Selected PLMN Index field (see sub-clause 10.4.27) in the octet immediately following this Length Indicator. In this case, it shall always be the first Length Indicator in the RLC data block.

A Length Indicator of value 61 is used by a mobile station supporting MS assisted Dedicated Core Network selection and indicates the presence of a DCN-ID value (see sub-clause 10.4.30) in the two octets immediately following this Length Indicator.

The first Length Indicator (respectively, second length Indicator, if the first is of value 62, as described above) shall indicate the number of octets of the RLC data field belonging to the first Upper Layer PDU, the second Length Indicator (respectively third Length Indicator, if the first is of value 62, as described above) shall indicate the number of octets of the RLC data field belonging to the second Upper Layer PDU, etc. Only the last segment of any Upper Layer PDU of a TBF (either this segment carries the entire Upper Layer PDU or not) shall be identified with a Length Indicator within the corresponding RLC data block.

A singular case occurs when the end of the Upper Layer PDU would fit within the RLC data block but the addition of the corresponding Length Indicator octet (to indicate the Upper Layer PDU boundary) causes the Upper Layer PDU to extend into the next RLC data block. In this case, this additional LI field shall take the value 0 whatever is the length of the last but one Upper Layer PDU segment. The inclusion of a Length Indicator that indicates the presence of a Selected PLMN Index field will never correspond to an Upper Layer PDU and therefore will never result in this singular case.

The final RLC data block of a TBF shall have a Length Indicator field corresponding to the final Upper Layer PDU unless this PDU fills the RLC data block precisely without the LI field being added (i.e. the singular case mentioned above never applies in this situation).

The LI field is 6 bits in length and shall be encoded as a binary number with range 1 to 19, 29, 35 or 49, according to the coding scheme in use, i.e. CS-1, CS-2, CS-3 or CS-4 respectively. The value 0 shall indicate that no Upper Layer PDU boundary exists. In this case the M bit shall be set to' 0' and the E bit shall be set to '1' on the transmitting side, while on the receiving side the M bit shall be ignored and the E bit shall be interpreted as having the value '1'. The value 61 or 62 may also be used as described above. In Iu mode, a value of 63 shall indicate the presence of filler information within the RLC data block. All other values are reserved, and in this version of the protocol, the mobile station shall ignore all fields of the RLC data block except for the USF field.

Tenth Change

### 10.4.14a	Length Indicator (LI) field in EGPRS TBF mode, EC TBF mode and TCH TBF mode (Iu mode)

The Length indicator is used to delimit Upper Layer PDUs within the RLC data block. Additionally for Iu mode, a Length Indicator of value 127 may be used to indicate the presence of filler information within the RLC data block. If the first length indicator in the RLC data block is set to 127, then the entire RLC data block contains filler information.

The first Length Indicator (respectively, second length Indicator, if the first is of value 123, as described below) shall indicate the number of octets of the RLC data field belonging to the first Upper Layer PDU, the second Length Indicator (respectively third Length Indicator, if the first is of value 123, as described below) shall indicate the number of octets of the RLC data field belonging to the second Upper Layer PDU, etc. Only the last segment of any Upper Layer PDU, including those with only one segment, shall be identified with a Length Indicator. The length indicator shall be placed in the RLC data block corresponding to the last segment of the Upper Layer PDU, unless the Upper Layer PDU without the corresponding LI field fills the RLC data block precisely. In this singular case, the Length Indicator shall be placed as the first Length Indicator in the next in sequence RLC data block and take the value 0. The inclusion of a Length Indicator that indicates the presence of a Selected PLMN Index field (see sub-clause 10.4.27) will never correspond to an Upper Layer PDU and therefore will never result in this singular case.

If the Upper Layer PDU does not fill the current RLC data block, a Length Indicator with value 127 (111 1111) shall be included as the last Length Indicator of the current RLC data block, indicating that there is no following Upper Layer PDU in this RLC data block. If the Upper Layer PDU does not fill the RLC data block and there is only one octet left, then the Length Indicator corresponding to the Upper Layer PDU is the last Length Indicator field that shall be included in the RLC data block.

During RLC non-persistent mode of operation, or, during RLC unacknowledged mode of operation if the network has commanded the usage of "Indication of Upper Layer PDU Start for RLC UM" for the TBF, and if the current RLC data block starts with the first segment of a new Upper Layer PDU, then a Length Indicator taking the value 126 shall be placed as the first Length Indicator in the RLC data block unless a Length Indicator with the value 0, signalling that the last segment of the previous Upper Layer PDU precisely fills the previous in sequence RLC data block, shall be included.

If the network and the mobile station support dynamic timeslot reduction (see sub-clause 8.1.8), a Length Indicator with value 125 indicates the presence of dynamic timeslot reduction control information which shall be included after the last Upper Layer PDU.

If EMSR is enabled, the transmission of an Upper Layer PDU can be suspended to transmit a higher priority Upper Layer PDU by including an LI with a value of 124 as the first LI field within the next transmitted RLC data block where an octet containing the TFI value associated with the higher priority Upper Layer PDU immediately follows the LI field. The transmission of the suspended Upper Layer PDU is resumed by including an LI with a value of 124 followed by an octet containing the TFI (and eTFI if assigned, see sub-clause 5.13) associated with the Upper Layer PDU whose transmission is resumed after the LI associated with the last segment of the higher priority Upper Layer PDU whose transmission is completed. The transition from the last segment of an Upper Layer PDU to the first segment of a new Upper Layer PDU associated with a different PFC can also occur within an RLC data block (i.e. no suspend occurs) in which case the transition is indicated by including an LI with a value of 124 followed by an octet containing the TFI (and eTFI if assigned) associated with the new Upper Layer PDU after the LI associated with the last segment of the Upper Layer PDU whose transmission is completed. In addition:

-	If the last segment of the Upper Layer PDU without the corresponding LI field fills the RLC data block precisely and the next in sequence RLC data block is in the same radio block, then a Length Indicator with the value 0 shall be placed as the first Length Indicator in the next in sequence RLC data block.

-	If the last segment of the Upper Layer PDU without the corresponding LI field fills the RLC data block precisely and the next in sequence RLC data block is in a different radio block, then a Length Indicator with the value 0 shall be placed as the first Length Indicator in the next in sequence RLC data block and the TFI in the RLC/MAC header associated with the next in sequence RLC data block and, if eTFI is assigned, the eTFI indicated by the CRC of the  next in sequence RLC data block, shall correspond to the last segment of the Upper Layer PDU.

-	When a Length Indicator having the value 124 is included in the same RLC data block as the last segment of the Upper Layer PDU and fills the RLC data block precisely then the first octet of the RLC data field of the next in sequence RLC data block shall include the TFI (and eTFI if assigned) corresponding to the Upper Layer PDU whose transmission is resumed (if a suspend occurred) or to a new Upper Layer PDU (if no suspend occurred).

A Length Indicator of value 123 is used by a mobile station supporting network sharing to indicate the presence of the Selected PLMN Index field (see sub-clause 10.4.27) in the octet immediately following this Length Indicator. In this case  it shall always be the first Length Indicator in the RLC data block. The first Length Indicator used for the case of EMSR requires that at least one previous RLC data block has been sent for the uplink TBF whereas the first Length Indicator used to indicate the presence of the Selected PLMN Index field shall always occur within the very first RLC data block sent for the uplink TBF and as such they can never occur within the same RLC data block.

A Length Indicator of value 120 is used by a mobile station supporting MS assisted Dedicated Core Network selection and indicates the presence of a DCN-ID value (see sub-clause 10.4.30) in the two octets immediately following this Length Indicator.

The final RLC data block of a TBF shall have a Length Indicator field corresponding to the final Upper Layer PDU unless the final Upper Layer PDU fills the RLC data block precisely. If the final Upper Layer PDU fills the final RLC data block precisely, the final Upper Layer PDU shall be sent without a corresponding Length Indicator field.

The Length Indicator field is 7 bits in length and shall be encoded as a binary number. The valid values are the values ranging from 0 to 74 for an EGPRS TBF, an EC TBF, an EGPRS2-A uplink TBF or an EGPRS2-B TBF, from 0 to 82 for an EGPRS2-A downlink TBF, from 0 to 103 in TCH TBF mode (Iu mode), and the values 120, and 123 to 127. All other values are reserved. A mobile station detecting a reserved Length Indicator value or an inconsistent encoding of the Length Indicator and E fields shall ignore the RLC data block.

The interpretation of the value contained in the length indicator with corresponding E bit is summarized in table 10.4.14a.1 and some examples are shown in annex B.

Table 10.4.14a.1: Interpretation of values of LI field and E bit

| Value of LI in a RLC data block | Value of E bit in the same octet | Interpretation |
| --- | --- | --- |
| k-th LI (k>0 integer): 0< value <75 (EGPRS/EC-GSM-IoT except EGPRS2-A downlink) 0< value <83 (EGPRS2-A downlink) 0< value <104 (TCH) |  | The value of the k-th LI is the number of octets of the k-th Upper Layer PDU, or the last segment of it, in the current RLC data block. |
|  | 0 | There is at least one Upper Layer PDU following the k-th Upper Layer PDU in the current RLC data block. |
|  | 1 | There is no more than one Upper Layer PDU following the k-th Upper Layer PDU in the current RLC data block. |
| 1st LI: value =0 | 0 | The last Upper Layer PDU of the previous in sequence RLC data block ends at the boundary of that RLC data block and it has no LI in the header of that RLC data block. Thus the current RLC data block contains the first segment of all included Upper Layer PDUs. |
| 1st LI: value = 126 | 0 | The current RLC data block contains the first segment of all included Upper Layer PDUs. |
| k-th LI (k>1 integer): 0< value <75 (EGPRS/EC-GSM-IoT except EGPRS2-A downlink) 0< value <83 (EGPRS2-A downlink) 0< value <104 (TCH) |  | The k-th LI contains the number of octets of the (k-1)-th Upper Layer PDU in the current RLC data block. |
|  | 0 | There is at least one Upper Layer PDU following the (k-1)-th Upper Layer PDU in the current RLC data block. |
|  | 1 | There is no more than one Upper Layer PDU following the (k-1)-th Upper Layer PDU in the current RLC data block. |
| k-th LI: value=127 | 1 | The octets between the end of the Upper Layer PDU indicated by the (k-1)-th LI and the end of the current RLC data block are filling octets. |
| 1st LI: value=0 | 1 | The previous RLC data block contains a Upper Layer PDU, or a part of it, that fills precisely the previous data block and for which there is no length indicator in that RLC data block. The current RLC data block contains a Upper Layer PDU that either fills the current RLC data block precisely or continues in the next RLC data block. |
| 1st LI: value = 126 | 1 | The current RLC data block contains the first segment of an Upper Layer PDU that either fills the current RLC data block precisely or continues in the next RLC data block. |
| 1st LI: value=127 (Iu mode only) | 1 | All octets of the RLC Data block contain filling information. |
| k-th LI: value=125 (this value is not supported in EC operation) | 0/1 | The current RLC data block contains the dynamic timeslot reduction control information. |
| k-th LI:value=124 (this value is only allowed if EMSR is enabled) | 0 | An LI value of 124 serves as a TFI transition indicator and is immediately followed by an octet containing the TFI associated with the next Upper Layer PDU segment in bit positions 1 to 5 (where bit 5 is the most significant bit) and reserved bits in bit positions 6 to 8. If eTFI is assigned then it shall be indicated by bit positions 6 to 8 (where bit 8 is the most significant bit). |
| 1 st LI:value=123 (this value is only allowed if network sharing is supported both by the network and the mobile station) | 0/1 | An LI value of 123 serves as a Selected PLMN Index field indicator and is immediately followed by an octet that contains the Selected PLMN Index field (see sub-clause 10.4.27). |
| LI:value=120 (this value is only allowed if MS assisted Dedicated Core Network selection is supported both by the network (see 3GPP TS 44.018) and the mobile station) | 0/1 | An LI value of 120 serves as a DCN-ID field indicator and is immediately followed by two octets that contains the DCN-ID field (see sub-clause 10.4.30). |
| No LI field present | n.a. | The Upper Layer PDU that starts with the current RLC data block either fills the current RLC data block precisely or continues in the following in-sequence RLC data block |

Eleventh Change

### 10.4.30	DCN-ID field

This field contains the DCN-ID value (as indicated to the RLC/MAC layer by upper layers) sent from a mobile station to a BSS when both the mobile station and BSS support MS assisted Dedicated Core Network selection, see 3GPP TS 24.008 [51] and TS 3GPP TS 23.401 [54].

Table 10.4.30.1: DCN-ID field

|  | Bits 87654321 |
| --- | --- |
| Octet 1 | 8 most significant bits (16 15 14 13 12 11 10 9) |
| Octet 2 | 8 least most significant bits (8 7 6 5 4 3 2 1) |

End of Changes