3GPP TSG-RAN WG6 Meeting #8 	R6-180098

# Busan, Korea, 21 – 25 May 2018                   			  Revision of R6-180056

| Issue | Brief description | Proposed solution/Comment | Priority | Status/Comment |
| --- | --- | --- | --- | --- |
| Number | Provide a brief description of the issue here; include a sub-clause reference to the relevant message/IE in the tabular. | If possible, briefly propose a solution (a description of what might need to be done; not a detailed CR) and provide other comments here. | Suggest a priority here. | Leave empty |
| 101 | Placing Rel-15 IE RadioBearerRelease in correct order. Currently it exists on top before RadioBearerRelease-r3. It should however appear at the end after the last added rel IE. RadioBearerRelease-r15-IEs ::= SEQUENCE { -- User equipment IEs integrityProtectionModeInfo IntegrityProtectionModeInfo-r7 OPTIONAL, cipheringModeInfo CipheringModeInfo-r7 OPTIONAL, activationTime ActivationTime OPTIONAL, dynamicActivationTime DynamicActivationTime OPTIONAL, new-U-RNTI U-RNTI OPTIONAL, new-C-RNTI C-RNTI OPTIONAL, -- The IE "new-DSCH-RNTI" should not be included in FDD mode, -- and if received the UE behaviour is unspecified new-DSCH-RNTI DSCH-RNTI OPTIONAL, new-H-RNTI H-RNTI OPTIONAL, newPrimary-E-RNTI E-RNTI OPTIONAL, newSecondary-E-RNTI E-RNTI OPTIONAL, rrc-StateIndicator RRC-StateIndicator, ueMobilityStateIndicator High-MobilityDetected OPTIONAL, utran-DRX-CycleLengthCoeff UTRAN-DRX-CycleLengthCoefficient-r7 OPTIONAL, accessGroupIdentity AccessGroupIdentity OPTIONAL, -- Core network IEs cn-InformationInfo CN-InformationInfo-r6 OPTIONAL, signallingConnectionRelIndication CN-DomainIdentity OPTIONAL, -- UTRAN mobility IEs ura-Identity URA-Identity OPTIONAL, supportForChangeOfUE-Capability BOOLEAN OPTIONAL, -- Radio bearer IEs rab-InformationReconfigList RAB-InformationReconfigList-r8 OPTIONAL, rb-InformationReleaseList RB-InformationReleaseList, rb-InformationReconfigList RB-InformationReconfigList-r11 OPTIONAL, rb-InformationAffectedList RB-InformationAffectedList-r8 OPTIONAL, dl-CounterSynchronisationInfo DL-CounterSynchronisationInfo-r5 OPTIONAL, retrievableConfigInfo RetrievableConfigInfo OPTIONAL, -- Transport channel IEs ul-CommonTransChInfo UL-CommonTransChInfo-r12 OPTIONAL, ul-deletedTransChInfoList UL-DeletedTransChInfoList-r6 OPTIONAL, ul-AddReconfTransChInfoList UL-AddReconfTransChInfoList-r8 OPTIONAL, dl-CommonTransChInfo DL-CommonTransChInfo-r4 OPTIONAL, dl-DeletedTransChInfoList DL-DeletedTransChInfoList-r7 OPTIONAL, dl-AddReconfTransChInfoList DL-AddReconfTransChInfoList-r13 OPTIONAL, -- Physical channel IEs frequencyInfo FrequencyInfo OPTIONAL, multi-frequencyInfo Multi-frequencyInfo-LCR-r7 OPTIONAL, dtx-drx-TimingInfo DTX-DRX-TimingInfo-r7 OPTIONAL, dtx-drx-Info DTX-DRX-Info-r12 OPTIONAL, hs-scch-LessInfo HS-SCCH-LessInfo-r7 OPTIONAL, mimoParameters MIMO-Parameters-r9 OPTIONAL, mimo4x4Parameters MIMO4x4-Parameters OPTIONAL, dch-Enhancements-Info-FDD DCH-Enhancements-Info-FDD OPTIONAL, maxAllowedUL-TX-Power MaxAllowedUL-TX-Power OPTIONAL, ul-DPCH-Info UL-DPCH-Info-r11 OPTIONAL, ul-EDCH-Information UL-EDCH-Information-r11 OPTIONAL, ul-SecondaryCellInfoFDD UL-SecondaryCellInfoFDD-r13 OPTIONAL, ul-MulticarrierEDCHInfo-TDD128 UL-MulticarrierEDCHInfo-TDD128 OPTIONAL, ul-CLTD-InfoFDD UL-CLTD-InfoFDD OPTIONAL, ul-OLTD-InfoFDD UL-OLTD-InfoFDD OPTIONAL, ul-OtherTTIConfiguration-Info UL-OtherTTIConfiguration-Information OPTIONAL, filteredUPHReportInfo FilteredUEPowerHeadroomReportInfo OPTIONAL, dl-HSPDSCH-Information DL-HSPDSCH-Information-r12 OPTIONAL, dl-CommonInformation DL-CommonInformation-r12 OPTIONAL, dl-InformationPerRL-List DL-InformationPerRL-List-r13 OPTIONAL, dl-SecondaryCellInfoFDD DL-SecondaryCellInfoFDD-r11 OPTIONAL, additionalDLSecCellInfoListFDD AdditionalDLSecCellInfoListFDD-r11 OPTIONAL, additionalDLSecCellInfoListFDD2 AdditionalDLSecCellInfoListFDD2 OPTIONAL, commonERGCHInfoFDD CommonERGCHInfoFDD OPTIONAL, controlChannelDRXInfo-TDD128 ControlChannelDRXInfo-TDD128-r8 OPTIONAL, sps-Information-TDD128 SPS-Information-TDD128-r8 OPTIONAL, mu-MIMO-Info-TDD128 MU-MIMO-Info-TDD128 OPTIONAL, non-rectResAllocInd-TDD128 Non-rectResAllocInd-TDD128 OPTIONAL, non-rectResSpecTSset-TDD128 Non-rectResSpecTSset-TDD128 OPTIONAL, simplified-HS-SCCH-type1-operation ENUMERATED { true } OPTIONAL, -- MBMS IEs mbms-PL-ServiceRestrictInfo MBMS-PL-ServiceRestrictInfo-r6 OPTIONAL, mbms-RB-ListReleasedToChangeTransferMode RB-InformationReleaseList OPTIONAL, -- Measurement IEs for LCR cellDCHMeasOccasionInfo-TDD128 CellDCHMeasOccasionInfo-TDD128-r9 OPTIONAL } RadioBearerRelease-r3-IEs ::= SEQUENCE { -- User equipment IEs rrc-TransactionIdentifier RRC-TransactionIdentifier, integrityProtectionModeInfo IntegrityProtectionModeInfo OPTIONAL, cipheringModeInfo CipheringModeInfo OPTIONAL, activationTime ActivationTime OPTIONAL, new-U-RNTI U-RNTI OPTIONAL, new-C-RNTI C-RNTI OPTIONAL, rrc-StateIndicator RRC-StateIndicator, utran-DRX-CycleLengthCoeff UTRAN-DRX-CycleLengthCoefficient OPTIONAL, -- Core network IEs | RadioBearerRelease-r13-IEs ::= SEQUENCE { -- User equipment IEs integrityProtectionModeInfo IntegrityProtectionModeInfo-r7 OPTIONAL, cipheringModeInfo CipheringModeInfo-r7 OPTIONAL, activationTime ActivationTime OPTIONAL, dynamicActivationTime DynamicActivationTime OPTIONAL, new-U-RNTI U-RNTI OPTIONAL, new-C-RNTI C-RNTI OPTIONAL, -- The IE "new-DSCH-RNTI" should not be included in FDD mode, -- and if received the UE behaviour is unspecified new-DSCH-RNTI DSCH-RNTI OPTIONAL, new-H-RNTI H-RNTI OPTIONAL, newPrimary-E-RNTI E-RNTI OPTIONAL, newSecondary-E-RNTI E-RNTI OPTIONAL, rrc-StateIndicator RRC-StateIndicator, ueMobilityStateIndicator High-MobilityDetected OPTIONAL, utran-DRX-CycleLengthCoeff UTRAN-DRX-CycleLengthCoefficient-r7 OPTIONAL, accessGroupIdentity AccessGroupIdentity OPTIONAL, -- Core network IEs cn-InformationInfo CN-InformationInfo-r6 OPTIONAL, signallingConnectionRelIndication CN-DomainIdentity OPTIONAL, -- UTRAN mobility IEs ura-Identity URA-Identity OPTIONAL, supportForChangeOfUE-Capability BOOLEAN OPTIONAL, -- Radio bearer IEs rab-InformationReconfigList RAB-InformationReconfigList-r8 OPTIONAL, rb-InformationReleaseList RB-InformationReleaseList, rb-InformationReconfigList RB-InformationReconfigList-r11 OPTIONAL, rb-InformationAffectedList RB-InformationAffectedList-r8 OPTIONAL, dl-CounterSynchronisationInfo DL-CounterSynchronisationInfo-r5 OPTIONAL, retrievableConfigInfo RetrievableConfigInfo OPTIONAL, -- Transport channel IEs ul-CommonTransChInfo UL-CommonTransChInfo-r12 OPTIONAL, ul-deletedTransChInfoList UL-DeletedTransChInfoList-r6 OPTIONAL, ul-AddReconfTransChInfoList UL-AddReconfTransChInfoList-r8 OPTIONAL, dl-CommonTransChInfo DL-CommonTransChInfo-r4 OPTIONAL, dl-DeletedTransChInfoList DL-DeletedTransChInfoList-r7 OPTIONAL, dl-AddReconfTransChInfoList DL-AddReconfTransChInfoList-r13 OPTIONAL, -- Physical channel IEs frequencyInfo FrequencyInfo OPTIONAL, multi-frequencyInfo Multi-frequencyInfo-LCR-r7 OPTIONAL, dtx-drx-TimingInfo DTX-DRX-TimingInfo-r7 OPTIONAL, dtx-drx-Info DTX-DRX-Info-r12 OPTIONAL, hs-scch-LessInfo HS-SCCH-LessInfo-r7 OPTIONAL, mimoParameters MIMO-Parameters-r9 OPTIONAL, mimo4x4Parameters MIMO4x4-Parameters OPTIONAL, dch-Enhancements-Info-FDD DCH-Enhancements-Info-FDD OPTIONAL, maxAllowedUL-TX-Power MaxAllowedUL-TX-Power OPTIONAL, ul-DPCH-Info UL-DPCH-Info-r11 OPTIONAL, ul-EDCH-Information UL-EDCH-Information-r11 OPTIONAL, ul-SecondaryCellInfoFDD UL-SecondaryCellInfoFDD-r13 OPTIONAL, ul-MulticarrierEDCHInfo-TDD128 UL-MulticarrierEDCHInfo-TDD128 OPTIONAL, ul-CLTD-InfoFDD UL-CLTD-InfoFDD OPTIONAL, ul-OLTD-InfoFDD UL-OLTD-InfoFDD OPTIONAL, ul-OtherTTIConfiguration-Info UL-OtherTTIConfiguration-Information OPTIONAL, filteredUPHReportInfo FilteredUEPowerHeadroomReportInfo OPTIONAL, dl-HSPDSCH-Information DL-HSPDSCH-Information-r12 OPTIONAL, dl-CommonInformation DL-CommonInformation-r12 OPTIONAL, dl-InformationPerRL-List DL-InformationPerRL-List-r13 OPTIONAL, dl-SecondaryCellInfoFDD DL-SecondaryCellInfoFDD-r11 OPTIONAL, additionalDLSecCellInfoListFDD AdditionalDLSecCellInfoListFDD-r11 OPTIONAL, additionalDLSecCellInfoListFDD2 AdditionalDLSecCellInfoListFDD2 OPTIONAL, commonERGCHInfoFDD CommonERGCHInfoFDD OPTIONAL, controlChannelDRXInfo-TDD128 ControlChannelDRXInfo-TDD128-r8 OPTIONAL, sps-Information-TDD128 SPS-Information-TDD128-r8 OPTIONAL, mu-MIMO-Info-TDD128 MU-MIMO-Info-TDD128 OPTIONAL, non-rectResAllocInd-TDD128 Non-rectResAllocInd-TDD128 OPTIONAL, non-rectResSpecTSset-TDD128 Non-rectResSpecTSset-TDD128 OPTIONAL, -- MBMS IEs mbms-PL-ServiceRestrictInfo MBMS-PL-ServiceRestrictInfo-r6 OPTIONAL, mbms-RB-ListReleasedToChangeTransferMode RB-InformationReleaseList OPTIONAL, -- Measurement IEs for LCR cellDCHMeasOccasionInfo-TDD128 CellDCHMeasOccasionInfo-TDD128-r9 OPTIONAL } RadioBearerRelease-LaterCriticalExtensions-r15-IEs ::= SEQUENCE { r15 SEQUENCE { radioBearerRelease-r15 RadioBearerRelease-r15-IEs, -- Container for adding non critical extensions -- after freezing REL-16 radioBearerRelease-r15-add-ext BIT STRING OPTIONAL, nonCriticalExtensions SEQUENCE {} OPTIONAL }, criticalExtensions SEQUENCE {} } RadioBearerRelease-r15-IEs ::= SEQUENCE { -- User equipment IEs integrityProtectionModeInfo IntegrityProtectionModeInfo-r7 OPTIONAL, cipheringModeInfo CipheringModeInfo-r7 OPTIONAL, activationTime ActivationTime OPTIONAL, dynamicActivationTime DynamicActivationTime OPTIONAL, new-U-RNTI U-RNTI OPTIONAL, new-C-RNTI C-RNTI OPTIONAL, -- The IE "new-DSCH-RNTI" should not be included in FDD mode, -- and if received the UE behaviour is unspecified new-DSCH-RNTI DSCH-RNTI OPTIONAL, new-H-RNTI H-RNTI OPTIONAL, newPrimary-E-RNTI E-RNTI OPTIONAL, newSecondary-E-RNTI E-RNTI OPTIONAL, rrc-StateIndicator RRC-StateIndicator, ueMobilityStateIndicator High-MobilityDetected OPTIONAL, utran-DRX-CycleLengthCoeff UTRAN-DRX-CycleLengthCoefficient-r7 OPTIONAL, accessGroupIdentity AccessGroupIdentity OPTIONAL, -- Core network IEs cn-InformationInfo CN-InformationInfo-r6 | 3 (Low) |  |
| 102 | Placing Rel-15 IE RadioBearerSetup Currently it exists on top before RadioBearerSetup-r3. It should however appear at the end after the last added rel IE. RadioBearerSetup-r15-IEs ::= SEQUENCE { -- User equipment IEs integrityProtectionModeInfo IntegrityProtectionModeInfo-r7 OPTIONAL, cipheringModeInfo CipheringModeInfo-r7 OPTIONAL, sr-vcc-Info SR-VCC-Info OPTIONAL, activationTime ActivationTime OPTIONAL, dynamicActivationTime DynamicActivationTime OPTIONAL, new-U-RNTI U-RNTI OPTIONAL, new-C-RNTI C-RNTI OPTIONAL, -- The IE "new-DSCH-RNTI" should not be included in FDD mode, -- and if received the UE behaviour is unspecified new-DSCH-RNTI DSCH-RNTI OPTIONAL, new-H-RNTI H-RNTI OPTIONAL, newPrimary-E-RNTI E-RNTI OPTIONAL, newSecondary-E-RNTI E-RNTI OPTIONAL, rrc-StateIndicator RRC-StateIndicator, utran-DRX-CycleLengthCoeff UTRAN-DRX-CycleLengthCoefficient-r7 OPTIONAL, accessGroupIdentity AccessGroupIdentity OPTIONAL, -- UTRAN mobility IEs ura-Identity URA-Identity OPTIONAL, supportForChangeOfUE-Capability BOOLEAN OPTIONAL, -- Core network IEs cn-InformationInfo CN-InformationInfo-r6 OPTIONAL, -- Radio bearer IEs srb-InformationSetupList SRB-InformationSetupList-r11 OPTIONAL, rab-InformationSetupList RAB-InformationSetupList-r11 OPTIONAL, rab-InformationReconfigList RAB-InformationReconfigList-r8 OPTIONAL, rb-InformationReconfigList RB-InformationReconfigList-r11 OPTIONAL, rb-InformationAffectedList RB-InformationAffectedList-r8 OPTIONAL, dl-CounterSynchronisationInfo DL-CounterSynchronisationInfo-r5 OPTIONAL, pdcp-ROHC-TargetMode PDCP-ROHC-TargetMode OPTIONAL, -- Transport channel IEs ul-CommonTransChInfo UL-CommonTransChInfo-r12 OPTIONAL, ul-deletedTransChInfoList UL-DeletedTransChInfoList-r6 OPTIONAL, ul-AddReconfTransChInfoList UL-AddReconfTransChInfoList-r8 OPTIONAL, dl-CommonTransChInfo DL-CommonTransChInfo-r4 OPTIONAL, dl-DeletedTransChInfoList DL-DeletedTransChInfoList-r7 OPTIONAL, dl-AddReconfTransChInfoList DL-AddReconfTransChInfoList-r13 OPTIONAL, -- Retrievable configuration retrievableConfigInfo RetrievableConfigInfo OPTIONAL, -- Physical channel IEs frequencyInfo FrequencyInfo OPTIONAL, multi-frequencyInfo Multi-frequencyInfo-LCR-r7 OPTIONAL, dtx-drx-TimingInfo DTX-DRX-TimingInfo-r7 OPTIONAL, dtx-drx-Info DTX-DRX-Info-r12 OPTIONAL, hs-scch-LessInfo HS-SCCH-LessInfo-r7 OPTIONAL, mimoParameters MIMO-Parameters-r9 OPTIONAL, mimo4x4Parameters MIMO4x4-Parameters OPTIONAL, dch-Enhancements-Info-FDD DCH-Enhancements-Info-FDD OPTIONAL, maxAllowedUL-TX-Power MaxAllowedUL-TX-Power OPTIONAL, ul-DPCH-Info UL-DPCH-Info-r11 OPTIONAL, ul-EDCH-Information UL-EDCH-Information-r11 OPTIONAL, ul-SecondaryCellInfoFDD UL-SecondaryCellInfoFDD-r13 OPTIONAL, ul-MulticarrierEDCHInfo-TDD128 UL-MulticarrierEDCHInfo-TDD128 OPTIONAL, ul-CLTD-InfoFDD UL-CLTD-InfoFDD OPTIONAL, ul-OLTD-InfoFDD UL-OLTD-InfoFDD OPTIONAL, ul-OtherTTIConfiguration-Info UL-OtherTTIConfiguration-Information OPTIONAL, filteredUPHReportInfo FilteredUEPowerHeadroomReportInfo OPTIONAL, dl-HSPDSCH-Information DL-HSPDSCH-Information-r12 OPTIONAL, dl-CommonInformation DL-CommonInformation-r12 OPTIONAL, dl-InformationPerRL-List DL-InformationPerRL-List-r13 OPTIONAL, dl-SecondaryCellInfoFDD DL-SecondaryCellInfoFDD-r11 OPTIONAL, additionalDLSecCellInfoListFDD AdditionalDLSecCellInfoListFDD-r11 OPTIONAL, additionalDLSecCellInfoListFDD2 AdditionalDLSecCellInfoListFDD2 OPTIONAL, commonERGCHInfoFDD CommonERGCHInfoFDD OPTIONAL, controlChannelDRXInfo-TDD128 ControlChannelDRXInfo-TDD128-r8 OPTIONAL, sps-Information-TDD128 SPS-Information-TDD128-r8 OPTIONAL, mu-MIMO-Info-TDD128 MU-MIMO-Info-TDD128 OPTIONAL, non-rectResAllocInd-TDD128 Non-rectResAllocInd-TDD128 OPTIONAL, non-rectResSpecTSset-TDD128 Non-rectResSpecTSset-TDD128 OPTIONAL, simplified-HS-SCCH-type1-operation ENUMERATED { true } OPTIONAL, -- MBMS IEs mbms-PL-ServiceRestrictInfo MBMS-PL-ServiceRestrictInfo-r6 OPTIONAL, -- Measurement IEs for LCR cellDCHMeasOccasionInfo-TDD128 CellDCHMeasOccasionInfo-TDD128-r9 OPTIONAL } RadioBearerSetup-r3-IEs ::= SEQUENCE { -- User equipment IEs rrc-TransactionIdentifier RRC-TransactionIdentifier, integrityProtectionModeInfo IntegrityProtectionModeInfo OPTIONAL, cipheringModeInfo CipheringModeInfo OPTIONAL, activationTime ActivationTime OPTIONAL, new-U-RNTI U-RNTI OPTIONAL, new-C-RNTI C-RNTI OPTIONAL, rrc-StateIndicator RRC-StateIndicator, utran-DRX-CycleLengthCoeff UTRAN-DRX-CycleLengthCoefficient OPTIONAL, -- UTRAN mobility IEs ura-Identity URA-Identity OPTIONAL, -- Core network IEs | RadioBearerSetup-r13-IEs ::= SEQUENCE { -- User equipment IEs integrityProtectionModeInfo IntegrityProtectionModeInfo-r7 OPTIONAL, cipheringModeInfo CipheringModeInfo-r7 OPTIONAL, sr-vcc-Info SR-VCC-Info OPTIONAL, activationTime ActivationTime OPTIONAL, dynamicActivationTime DynamicActivationTime OPTIONAL, new-U-RNTI U-RNTI OPTIONAL, new-C-RNTI C-RNTI OPTIONAL, -- The IE "new-DSCH-RNTI" should not be included in FDD mode, -- and if received the UE behaviour is unspecified new-DSCH-RNTI DSCH-RNTI OPTIONAL, new-H-RNTI H-RNTI OPTIONAL, newPrimary-E-RNTI E-RNTI OPTIONAL, newSecondary-E-RNTI E-RNTI OPTIONAL, rrc-StateIndicator RRC-StateIndicator, utran-DRX-CycleLengthCoeff UTRAN-DRX-CycleLengthCoefficient-r7 OPTIONAL, accessGroupIdentity AccessGroupIdentity OPTIONAL, -- UTRAN mobility IEs ura-Identity URA-Identity OPTIONAL, supportForChangeOfUE-Capability BOOLEAN OPTIONAL, -- Core network IEs cn-InformationInfo CN-InformationInfo-r6 OPTIONAL, -- Radio bearer IEs srb-InformationSetupList SRB-InformationSetupList-r11 OPTIONAL, rab-InformationSetupList RAB-InformationSetupList-r11 OPTIONAL, rab-InformationReconfigList RAB-InformationReconfigList-r8 OPTIONAL, rb-InformationReconfigList RB-InformationReconfigList-r11 OPTIONAL, rb-InformationAffectedList RB-InformationAffectedList-r8 OPTIONAL, dl-CounterSynchronisationInfo DL-CounterSynchronisationInfo-r5 OPTIONAL, pdcp-ROHC-TargetMode PDCP-ROHC-TargetMode OPTIONAL, -- Transport channel IEs ul-CommonTransChInfo UL-CommonTransChInfo-r12 OPTIONAL, ul-deletedTransChInfoList UL-DeletedTransChInfoList-r6 OPTIONAL, ul-AddReconfTransChInfoList UL-AddReconfTransChInfoList-r8 OPTIONAL, dl-CommonTransChInfo DL-CommonTransChInfo-r4 OPTIONAL, dl-DeletedTransChInfoList DL-DeletedTransChInfoList-r7 OPTIONAL, dl-AddReconfTransChInfoList DL-AddReconfTransChInfoList-r13 OPTIONAL, -- Retrievable configuration retrievableConfigInfo RetrievableConfigInfo OPTIONAL, -- Physical channel IEs frequencyInfo FrequencyInfo OPTIONAL, multi-frequencyInfo Multi-frequencyInfo-LCR-r7 OPTIONAL, dtx-drx-TimingInfo DTX-DRX-TimingInfo-r7 OPTIONAL, dtx-drx-Info DTX-DRX-Info-r12 OPTIONAL, hs-scch-LessInfo HS-SCCH-LessInfo-r7 OPTIONAL, mimoParameters MIMO-Parameters-r9 OPTIONAL, mimo4x4Parameters MIMO4x4-Parameters OPTIONAL, dch-Enhancements-Info-FDD DCH-Enhancements-Info-FDD OPTIONAL, maxAllowedUL-TX-Power MaxAllowedUL-TX-Power OPTIONAL, ul-DPCH-Info UL-DPCH-Info-r11 OPTIONAL, ul-EDCH-Information UL-EDCH-Information-r11 OPTIONAL, ul-SecondaryCellInfoFDD UL-SecondaryCellInfoFDD-r13 OPTIONAL, ul-MulticarrierEDCHInfo-TDD128 UL-MulticarrierEDCHInfo-TDD128 OPTIONAL, ul-CLTD-InfoFDD UL-CLTD-InfoFDD OPTIONAL, ul-OLTD-InfoFDD UL-OLTD-InfoFDD OPTIONAL, ul-OtherTTIConfiguration-Info UL-OtherTTIConfiguration-Information OPTIONAL, filteredUPHReportInfo FilteredUEPowerHeadroomReportInfo OPTIONAL, dl-HSPDSCH-Information DL-HSPDSCH-Information-r12 OPTIONAL, dl-CommonInformation DL-CommonInformation-r12 OPTIONAL, dl-InformationPerRL-List DL-InformationPerRL-List-r13 OPTIONAL, dl-SecondaryCellInfoFDD DL-SecondaryCellInfoFDD-r11 OPTIONAL, additionalDLSecCellInfoListFDD AdditionalDLSecCellInfoListFDD-r11 OPTIONAL, additionalDLSecCellInfoListFDD2 AdditionalDLSecCellInfoListFDD2 OPTIONAL, commonERGCHInfoFDD CommonERGCHInfoFDD OPTIONAL, controlChannelDRXInfo-TDD128 ControlChannelDRXInfo-TDD128-r8 OPTIONAL, sps-Information-TDD128 SPS-Information-TDD128-r8 OPTIONAL, mu-MIMO-Info-TDD128 MU-MIMO-Info-TDD128 OPTIONAL, non-rectResAllocInd-TDD128 Non-rectResAllocInd-TDD128 OPTIONAL, non-rectResSpecTSset-TDD128 Non-rectResSpecTSset-TDD128 OPTIONAL, -- MBMS IEs mbms-PL-ServiceRestrictInfo MBMS-PL-ServiceRestrictInfo-r6 OPTIONAL, -- Measurement IEs for LCR cellDCHMeasOccasionInfo-TDD128 CellDCHMeasOccasionInfo-TDD128-r9 OPTIONAL } RadioBearerSetup-LaterCriticalExtensions-r15-IEs ::= SEQUENCE { r15 SEQUENCE { radioBearerSetup-r15 RadioBearerSetup-r15-IEs, -- Container for adding non critical extensions -- after freezing REL-16 radioBearerSetup-r15-add-ext BIT STRING OPTIONAL, nonCriticalExtensions SEQUENCE {} OPTIONAL }, criticalExtensions SEQUENCE {} } RadioBearerSetup-r15-IEs ::= SEQUENCE { -- User equipment IEs integrityProtectionModeInfo IntegrityProtectionModeInfo-r7 OPTIONAL, cipheringModeInfo CipheringModeInfo-r7 OPTIONAL, sr-vcc-Info SR-VCC-Info OPTIONAL, activationTime ActivationTime OPTIONAL, dynamicActivationTime DynamicActivationTime OPTIONAL, new-U-RNTI U-RNTI OPTIONAL, new-C-RNTI C-RNTI OPTIONAL, -- The IE "new-DSCH-RNTI" should not be included in FDD mode, -- and if received the UE behaviour is unspecified new-DSCH-RNTI DSCH-RNTI OPTIONAL, new-H-RNTI H-RNTI OPTIONAL, newPrimary-E-RNTI E-RNTI OPTIONAL, newSecondary-E-RNTI E-RNTI OPTIONAL, rrc-StateIndicator RRC-StateIndicator, utran-DRX-CycleLengthCoeff UTRAN-DRX-CycleLengthCoefficient-r7 OPTIONAL, accessGroupIdentity AccessGroupIdentity OPTIONAL, -- UTRAN mobility IEs ura-Identity URA-Identity OPTIONAL, supportForChangeOfUE-Capability BOOLEAN OPTIONAL, -- Core network IEs cn-InformationInfo CN-InformationInfo-r6 OPTIONAL, -- Radio bearer IEs srb-InformationSetupList SRB-InformationSetupList-r11 OPTIONAL, rab-InformationSetupList RAB-InformationSetupList-r11 OPTIONAL, rab-InformationReconfigList RAB-InformationReconfigList-r8 OPTIONAL, rb-InformationReconfigList RB-InformationReconfigList-r11 OPTIONAL, rb-InformationAffectedList RB-InformationAffectedList-r8 OPTIONAL, dl-CounterSynchronisationInfo DL-CounterSynchronisationInfo-r5 OPTIONAL, pdcp-ROHC-TargetMode PDCP-ROHC-TargetMode OPTIONAL, -- Transport channel IEs ul-CommonTransChInfo UL-CommonTransChInfo-r12 OPTIONAL, ul-deletedTransChInfoList UL-DeletedTransChInfoList-r6 OPTIONAL, ul-AddReconfTransChInfoList UL-AddReconfTransChInfoList-r8 OPTIONAL, dl-CommonTransChInfo DL-CommonTransChInfo-r4 OPTIONAL, dl-DeletedTransChInfoList DL-DeletedTransChInfoList-r7 OPTIONAL, dl-AddReconfTransChInfoList DL-AddReconfTransChInfoList-r13 OPTIONAL, -- Retrievable configuration retrievableConfigInfo RetrievableConfigInfo OPTIONAL, -- Physical channel IEs frequencyInfo FrequencyInfo OPTIONAL, multi-frequencyInfo Multi-frequencyInfo-LCR-r7 OPTIONAL, dtx-drx-TimingInfo DTX-DRX-TimingInfo-r7 OPTIONAL, dtx-drx-Info DTX-DRX-Info-r12 OPTIONAL, hs-scch-LessInfo HS-SCCH-LessInfo-r7 OPTIONAL, mimoParameters MIMO-Parameters-r9 OPTIONAL, mimo4x4Parameters MIMO4x4-Parameters OPTIONAL, dch-Enhancements-Info-FDD DCH-Enhancements-Info-FDD OPTIONAL, maxAllowedUL-TX-Power MaxAllowedUL-TX-Power OPTIONAL, ul-DPCH-Info UL-DPCH-Info-r11 OPTIONAL, ul-EDCH-Information UL-EDCH-Information-r11 OPTIONAL, ul-SecondaryCellInfoFDD UL-SecondaryCellInfoFDD-r13 OPTIONAL, ul-MulticarrierEDCHInfo-TDD128 UL-MulticarrierEDCHInfo-TDD128 OPTIONAL, ul-CLTD-InfoFDD UL-CLTD-InfoFDD OPTIONAL, ul-OLTD-InfoFDD UL-OLTD-InfoFDD OPTIONAL, ul-OtherTTIConfiguration-Info UL-OtherTTIConfiguration-Information OPTIONAL, filteredUPHReportInfo FilteredUEPowerHeadroomReportInfo OPTIONAL, dl-HSPDSCH-Information DL-HSPDSCH-Information-r12 OPTIONAL, dl-CommonInformation DL-CommonInformation-r12 OPTIONAL, dl-InformationPerRL-List DL-InformationPerRL-List-r13 OPTIONAL, dl-SecondaryCellInfoFDD DL-SecondaryCellInfoFDD-r11 OPTIONAL, additionalDLSecCellInfoListFDD AdditionalDLSecCellInfoListFDD-r11 OPTIONAL, additionalDLSecCellInfoListFDD2 AdditionalDLSecCellInfoListFDD2 OPTIONAL, commonERGCHInfoFDD CommonERGCHInfoFDD OPTIONAL, controlChannelDRXInfo-TDD128 ControlChannelDRXInfo-TDD128-r8 OPTIONAL, sps-Information-TDD128 SPS-Information-TDD128-r8 OPTIONAL, mu-MIMO-Info-TDD128 MU-MIMO-Info-TDD128 OPTIONAL, non-rectResAllocInd-TDD128 Non-rectResAllocInd-TDD128 OPTIONAL, non-rectResSpecTSset-TDD128 Non-rectResSpecTSset-TDD128 OPTIONAL, simplified-HS-SCCH-type1-operation ENUMERATED { true } OPTIONAL, -- MBMS IEs mbms-PL-ServiceRestrictInfo MBMS-PL-ServiceRestrictInfo-r6 OPTIONAL, -- Measurement IEs for LCR cellDCHMeasOccasionInfo-TDD128 CellDCHMeasOccasionInfo-TDD128-r9 OPTIONAL } | 3 (Low) |  |
| 103 | Placing Rel-15 IE RadioBearerReconfiguration in correct order. Currently it exists on top before RadioBearerReconfiguration-vaa0ext. It should however appear at the end after the last added rel IE. RadioBearerReconfiguration-LaterCriticalExtensions-r15-IEs ::= SEQUENCE { r15 SEQUENCE { radioBearerReconfiguration-r15 RadioBearerReconfiguration-r15-IEs, -- Container for adding non critical -- extensions after freezing REL-16 radioBearerReconfiguration-r15-add-ext BIT STRING OPTIONAL, nonCriticalExtensions SEQUENCE {} OPTIONAL }, criticalExtensions SEQUENCE {} } RadioBearerReconfiguration-r15-IEs ::= SEQUENCE { -- User equipment IEs integrityProtectionModeInfo IntegrityProtectionModeInfo-r7 OPTIONAL, cipheringModeInfo CipheringModeInfo-r7 OPTIONAL, activationTime ActivationTime OPTIONAL, dynamicActivationTime DynamicActivationTime OPTIONAL, delayRestrictionFlag DelayRestrictionFlag OPTIONAL, new-U-RNTI U-RNTI OPTIONAL, new-C-RNTI C-RNTI OPTIONAL, -- The IE "new-DSCH-RNTI" should not be included in FDD mode, -- and if received the UE behaviour is unspecified new-DSCH-RNTI DSCH-RNTI OPTIONAL, new-H-RNTI H-RNTI OPTIONAL, newPrimary-E-RNTI E-RNTI OPTIONAL, newSecondary-E-RNTI E-RNTI OPTIONAL, rntiHandlingAtCellReselection ENUMERATED { true } OPTIONAL, rrc-StateIndicator RRC-StateIndicator, ueMobilityStateIndicator High-MobilityDetected OPTIONAL, utran-DRX-CycleLengthCoeff UTRAN-DRX-CycleLengthCoefficient-r7 OPTIONAL, accessGroupIdentity AccessGroupIdentity OPTIONAL, -- Core network IEs cn-InformationInfo CN-InformationInfo-r6 OPTIONAL, signallingConnectionRelIndication CN-DomainIdentity OPTIONAL, -- UTRAN mobility IEs ura-Identity URA-Identity OPTIONAL, supportForChangeOfUE-Capability BOOLEAN OPTIONAL, responseToChangeOfUE-Capability ENUMERATED { true } OPTIONAL, -- Specification mode information defaultConfigForCellFACH DefaultConfigForCellFACH OPTIONAL, specificationMode CHOICE { complete SEQUENCE { -- Radio bearer IEs rab-InformationSetupList RAB-InformationSetupList-r11 OPTIONAL, rab-InformationReconfigList RAB-InformationReconfigList-r8 OPTIONAL, rab-InformationMBMSPtpList RAB-InformationMBMSPtpList OPTIONAL, rb-InformationReconfigList RB-InformationReconfigList-r11 OPTIONAL, rb-InformationAffectedList RB-InformationAffectedList-r8 OPTIONAL, rb-InformationReleaseList RB-InformationReleaseList OPTIONAL, rb-PDCPContextRelocationList RB-PDCPContextRelocationList OPTIONAL, pdcp-ROHC-TargetMode PDCP-ROHC-TargetMode OPTIONAL, umRlc-ReEstablishmentRBList RB-IdentityList OPTIONAL, otherStateConfig OtherStateConfig OPTIONAL, -- Transport channel IEs ul-CommonTransChInfo UL-CommonTransChInfo-r12 OPTIONAL, ul-deletedTransChInfoList UL-DeletedTransChInfoList-r6 OPTIONAL, ul-AddReconfTransChInfoList UL-AddReconfTransChInfoList-r14 OPTIONAL, dl-CommonTransChInfo DL-CommonTransChInfo-r4 OPTIONAL, dl-DeletedTransChInfoList DL-DeletedTransChInfoList-r7 OPTIONAL, dl-AddReconfTransChInfoList DL-AddReconfTransChInfoList-r13 OPTIONAL, -- Retrievable configuration retrievableConfigInfo RetrievableConfigInfo OPTIONAL }, preconfiguration SEQUENCE { -- All IEs that include an FDD/TDD choice are split in two IEs for this message, -- one for the FDD only elements and one for the TDD only elements, so that one -- FDD/TDD choice in this level is sufficient. preConfigMode CHOICE { predefinedConfigIdentity PredefinedConfigIdentity, defaultConfig SEQUENCE { defaultConfigMode DefaultConfigMode, defaultConfigIdentity DefaultConfigIdentity-r6 } } } }, -- Physical channel IEs frequencyInfo FrequencyInfo OPTIONAL, multi-frequencyInfo Multi-frequencyInfo-LCR-r7 OPTIONAL, dtx-drx-TimingInfo DTX-DRX-TimingInfo-r7 OPTIONAL, dtx-drx-Info DTX-DRX-Info-r12 OPTIONAL, hs-scch-LessInfo HS-SCCH-LessInfo-r7 OPTIONAL, mimoParameters MIMO-Parameters-r9 OPTIONAL, mimo4x4Parameters MIMO4x4-Parameters OPTIONAL, dch-Enhancements-Info-FDD DCH-Enhancements-Info-FDD OPTIONAL, maxAllowedUL-TX-Power MaxAllowedUL-TX-Power OPTIONAL, ul-DPCH-Info UL-DPCH-Info-r11 OPTIONAL, ul-EDCH-Information UL-EDCH-Information-r11 OPTIONAL, ul-SecondaryCellInfoFDD UL-SecondaryCellInfoFDD-r14 OPTIONAL, ul-MulticarrierEDCHInfo-TDD128 UL-MulticarrierEDCHInfo-TDD128 OPTIONAL, ul-CLTD-InfoFDD UL-CLTD-InfoFDD OPTIONAL, ul-OLTD-InfoFDD UL-OLTD-InfoFDD OPTIONAL, ul-OtherTTIConfiguration-Info UL-OtherTTIConfiguration-Information OPTIONAL, filteredUPHReportInfo FilteredUEPowerHeadroomReportInfo OPTIONAL, ul-OtherTTIConfigurationInfo-Sec UL-OtherTTIConfigurationInfo-Sec OPTIONAL, filteredUPHReportInfo-Sec FilteredUEPowerHeadroomReportInfo OPTIONAL, dl-HSPDSCH-Information DL-HSPDSCH-Information-r12 OPTIONAL, dl-CommonInformation DL-CommonInformation-r12 OPTIONAL, dl-InformationPerRL-List DL-InformationPerRL-List-r13 OPTIONAL, dl-SecondaryCellInfoFDD DL-SecondaryCellInfoFDD-r11 OPTIONAL, additionalDLSecCellInfoListFDD AdditionalDLSecCellInfoListFDD-r11 OPTIONAL, additionalDLSecCellInfoListFDD2 AdditionalDLSecCellInfoListFDD2 OPTIONAL, commonERGCHInfoFDD CommonERGCHInfoFDD OPTIONAL, dPCCH2InfoFDD DPCCH2InfoFDD OPTIONAL, controlChannelDRXInfo-TDD128 ControlChannelDRXInfo-TDD128-r8 OPTIONAL, sps-Information-TDD128 SPS-Information-TDD128-r8 OPTIONAL, mu-MIMO-Info-TDD128 MU-MIMO-Info-TDD128 OPTIONAL, nonrectResAllocInd-TDD128 Non-rectResAllocInd-TDD128 OPTIONAL, nonrectResSpecTSset-TDD128 Non-rectResSpecTSset-TDD128 OPTIONAL, hs-scch-DrxCellfach-info HS-SCCH-DrxCellfach-info OPTIONAL, simplified-HS-SCCH-type1-operation ENUMERATED { true } OPTIONAL, -- MBMS IEs mbms-PL-ServiceRestrictInfo MBMS-PL-ServiceRestrictInfo-r6 OPTIONAL, -- Measurement IEs for LCR cellDCHMeasOccasionInfo-TDD128 CellDCHMeasOccasionInfo-TDD128-r9 OPTIONAL } RadioBearerReconfiguration-vaa0ext-IEs ::= SEQUENCE { um-RLC-ReEstablishmentRBList RB-IdentityList OPTIONAL } | RadioBearerReconfiguration-r14-IEs ::= SEQUENCE { -- User equipment IEs integrityProtectionModeInfo IntegrityProtectionModeInfo-r7 OPTIONAL, cipheringModeInfo CipheringModeInfo-r7 OPTIONAL, activationTime ActivationTime OPTIONAL, dynamicActivationTime DynamicActivationTime OPTIONAL, delayRestrictionFlag DelayRestrictionFlag OPTIONAL, new-U-RNTI U-RNTI OPTIONAL, new-C-RNTI C-RNTI OPTIONAL, -- The IE "new-DSCH-RNTI" should not be included in FDD mode, -- and if received the UE behaviour is unspecified new-DSCH-RNTI DSCH-RNTI OPTIONAL, new-H-RNTI H-RNTI OPTIONAL, newPrimary-E-RNTI E-RNTI OPTIONAL, newSecondary-E-RNTI E-RNTI OPTIONAL, rntiHandlingAtCellReselection ENUMERATED { true } OPTIONAL, rrc-StateIndicator RRC-StateIndicator, ueMobilityStateIndicator High-MobilityDetected OPTIONAL, utran-DRX-CycleLengthCoeff UTRAN-DRX-CycleLengthCoefficient-r7 OPTIONAL, accessGroupIdentity AccessGroupIdentity OPTIONAL, -- Core network IEs cn-InformationInfo CN-InformationInfo-r6 OPTIONAL, signallingConnectionRelIndication CN-DomainIdentity OPTIONAL, -- UTRAN mobility IEs ura-Identity URA-Identity OPTIONAL, supportForChangeOfUE-Capability BOOLEAN OPTIONAL, responseToChangeOfUE-Capability ENUMERATED { true } OPTIONAL, -- Specification mode information defaultConfigForCellFACH DefaultConfigForCellFACH OPTIONAL, specificationMode CHOICE { complete SEQUENCE { -- Radio bearer IEs rab-InformationSetupList RAB-InformationSetupList-r11 OPTIONAL, rab-InformationReconfigList RAB-InformationReconfigList-r8 OPTIONAL, rab-InformationMBMSPtpList RAB-InformationMBMSPtpList OPTIONAL, rb-InformationReconfigList RB-InformationReconfigList-r11 OPTIONAL, rb-InformationAffectedList RB-InformationAffectedList-r8 OPTIONAL, rb-InformationReleaseList RB-InformationReleaseList OPTIONAL, rb-PDCPContextRelocationList RB-PDCPContextRelocationList OPTIONAL, pdcp-ROHC-TargetMode PDCP-ROHC-TargetMode OPTIONAL, umRlc-ReEstablishmentRBList RB-IdentityList OPTIONAL, otherStateConfig OtherStateConfig OPTIONAL, -- Transport channel IEs ul-CommonTransChInfo UL-CommonTransChInfo-r12 OPTIONAL, ul-deletedTransChInfoList UL-DeletedTransChInfoList-r6 OPTIONAL, ul-AddReconfTransChInfoList UL-AddReconfTransChInfoList-r14 OPTIONAL, dl-CommonTransChInfo DL-CommonTransChInfo-r4 OPTIONAL, dl-DeletedTransChInfoList DL-DeletedTransChInfoList-r7 OPTIONAL, dl-AddReconfTransChInfoList DL-AddReconfTransChInfoList-r13 OPTIONAL, -- Retrievable configuration retrievableConfigInfo RetrievableConfigInfo OPTIONAL }, preconfiguration SEQUENCE { -- All IEs that include an FDD/TDD choice are split in two IEs for this message, -- one for the FDD only elements and one for the TDD only elements, so that one -- FDD/TDD choice in this level is sufficient. preConfigMode CHOICE { predefinedConfigIdentity PredefinedConfigIdentity, defaultConfig SEQUENCE { defaultConfigMode DefaultConfigMode, defaultConfigIdentity DefaultConfigIdentity-r6 } } } }, -- Physical channel IEs frequencyInfo FrequencyInfo OPTIONAL, multi-frequencyInfo Multi-frequencyInfo-LCR-r7 OPTIONAL, dtx-drx-TimingInfo DTX-DRX-TimingInfo-r7 OPTIONAL, dtx-drx-Info DTX-DRX-Info-r12 OPTIONAL, hs-scch-LessInfo HS-SCCH-LessInfo-r7 OPTIONAL, mimoParameters MIMO-Parameters-r9 OPTIONAL, mimo4x4Parameters MIMO4x4-Parameters OPTIONAL, dch-Enhancements-Info-FDD DCH-Enhancements-Info-FDD OPTIONAL, maxAllowedUL-TX-Power MaxAllowedUL-TX-Power OPTIONAL, ul-DPCH-Info UL-DPCH-Info-r11 OPTIONAL, ul-EDCH-Information UL-EDCH-Information-r11 OPTIONAL, ul-SecondaryCellInfoFDD UL-SecondaryCellInfoFDD-r14 OPTIONAL, ul-MulticarrierEDCHInfo-TDD128 UL-MulticarrierEDCHInfo-TDD128 OPTIONAL, ul-CLTD-InfoFDD UL-CLTD-InfoFDD OPTIONAL, ul-OLTD-InfoFDD UL-OLTD-InfoFDD OPTIONAL, ul-OtherTTIConfiguration-Info UL-OtherTTIConfiguration-Information OPTIONAL, filteredUPHReportInfo FilteredUEPowerHeadroomReportInfo OPTIONAL, ul-OtherTTIConfigurationInfo-Sec UL-OtherTTIConfigurationInfo-Sec OPTIONAL, filteredUPHReportInfo-Sec FilteredUEPowerHeadroomReportInfo OPTIONAL, dl-HSPDSCH-Information DL-HSPDSCH-Information-r12 OPTIONAL, dl-CommonInformation DL-CommonInformation-r12 OPTIONAL, dl-InformationPerRL-List DL-InformationPerRL-List-r13 OPTIONAL, dl-SecondaryCellInfoFDD DL-SecondaryCellInfoFDD-r11 OPTIONAL, additionalDLSecCellInfoListFDD AdditionalDLSecCellInfoListFDD-r11 OPTIONAL, additionalDLSecCellInfoListFDD2 AdditionalDLSecCellInfoListFDD2 OPTIONAL, commonERGCHInfoFDD CommonERGCHInfoFDD OPTIONAL, dPCCH2InfoFDD DPCCH2InfoFDD OPTIONAL, controlChannelDRXInfo-TDD128 ControlChannelDRXInfo-TDD128-r8 OPTIONAL, sps-Information-TDD128 SPS-Information-TDD128-r8 OPTIONAL, mu-MIMO-Info-TDD128 MU-MIMO-Info-TDD128 OPTIONAL, nonrectResAllocInd-TDD128 Non-rectResAllocInd-TDD128 OPTIONAL, nonrectResSpecTSset-TDD128 Non-rectResSpecTSset-TDD128 OPTIONAL, hs-scch-DrxCellfach-info HS-SCCH-DrxCellfach-info OPTIONAL, -- MBMS IEs mbms-PL-ServiceRestrictInfo MBMS-PL-ServiceRestrictInfo-r6 OPTIONAL, -- Measurement IEs for LCR cellDCHMeasOccasionInfo-TDD128 CellDCHMeasOccasionInfo-TDD128-r9 OPTIONAL } RadioBearerReconfiguration-LaterCriticalExtensions-r15-IEs ::= SEQUENCE { r15 SEQUENCE { radioBearerReconfiguration-r15 RadioBearerReconfiguration-r15-IEs, -- Container for adding non critical -- extensions after freezing REL-16 radioBearerReconfiguration-r15-add-ext BIT STRING OPTIONAL, nonCriticalExtensions SEQUENCE {} OPTIONAL }, criticalExtensions SEQUENCE {} } RadioBearerReconfiguration-r15-IEs ::= SEQUENCE { -- User equipment IEs integrityProtectionModeInfo IntegrityProtectionModeInfo-r7 OPTIONAL, cipheringModeInfo CipheringModeInfo-r7 OPTIONAL, activationTime ActivationTime OPTIONAL, dynamicActivationTime DynamicActivationTime OPTIONAL, delayRestrictionFlag DelayRestrictionFlag OPTIONAL, new-U-RNTI U-RNTI OPTIONAL, new-C-RNTI C-RNTI OPTIONAL, -- The IE "new-DSCH-RNTI" should not be included in FDD mode, -- and if received the UE behaviour is unspecified new-DSCH-RNTI DSCH-RNTI OPTIONAL, new-H-RNTI H-RNTI OPTIONAL, newPrimary-E-RNTI E-RNTI OPTIONAL, newSecondary-E-RNTI E-RNTI OPTIONAL, rntiHandlingAtCellReselection ENUMERATED { true } OPTIONAL, rrc-StateIndicator RRC-StateIndicator, ueMobilityStateIndicator High-MobilityDetected OPTIONAL, utran-DRX-CycleLengthCoeff UTRAN-DRX-CycleLengthCoefficient-r7 OPTIONAL, accessGroupIdentity AccessGroupIdentity OPTIONAL, -- Core network IEs cn-InformationInfo CN-InformationInfo-r6 OPTIONAL, signallingConnectionRelIndication CN-DomainIdentity OPTIONAL, -- UTRAN mobility IEs ura-Identity URA-Identity OPTIONAL, supportForChangeOfUE-Capability BOOLEAN OPTIONAL, responseToChangeOfUE-Capability ENUMERATED { true } OPTIONAL, -- Specification mode information defaultConfigForCellFACH DefaultConfigForCellFACH OPTIONAL, specificationMode CHOICE { complete SEQUENCE { -- Radio bearer IEs rab-InformationSetupList RAB-InformationSetupList-r11 OPTIONAL, rab-InformationReconfigList RAB-InformationReconfigList-r8 OPTIONAL, rab-InformationMBMSPtpList RAB-InformationMBMSPtpList OPTIONAL, rb-InformationReconfigList RB-InformationReconfigList-r11 OPTIONAL, rb-InformationAffectedList RB-InformationAffectedList-r8 OPTIONAL, rb-InformationReleaseList RB-InformationReleaseList OPTIONAL, rb-PDCPContextRelocationList RB-PDCPContextRelocationList OPTIONAL, pdcp-ROHC-TargetMode PDCP-ROHC-TargetMode OPTIONAL, umRlc-ReEstablishmentRBList RB-IdentityList OPTIONAL, otherStateConfig OtherStateConfig OPTIONAL, -- Transport channel IEs ul-CommonTransChInfo UL-CommonTransChInfo-r12 OPTIONAL, ul-deletedTransChInfoList UL-DeletedTransChInfoList-r6 OPTIONAL, ul-AddReconfTransChInfoList UL-AddReconfTransChInfoList-r14 OPTIONAL, dl-CommonTransChInfo DL-CommonTransChInfo-r4 OPTIONAL, dl-DeletedTransChInfoList DL-DeletedTransChInfoList-r7 OPTIONAL, dl-AddReconfTransChInfoList DL-AddReconfTransChInfoList-r13 OPTIONAL, -- Retrievable configuration retrievableConfigInfo RetrievableConfigInfo OPTIONAL }, preconfiguration SEQUENCE { -- All IEs that include an FDD/TDD choice are split in two IEs for this message, -- one for the FDD only elements and one for the TDD only elements, so that one -- FDD/TDD choice in this level is sufficient. preConfigMode CHOICE { predefinedConfigIdentity PredefinedConfigIdentity, defaultConfig SEQUENCE { defaultConfigMode DefaultConfigMode, defaultConfigIdentity DefaultConfigIdentity-r6 } } } }, -- Physical channel IEs frequencyInfo FrequencyInfo OPTIONAL, multi-frequencyInfo Multi-frequencyInfo-LCR-r7 OPTIONAL, dtx-drx-TimingInfo DTX-DRX-TimingInfo-r7 OPTIONAL, dtx-drx-Info DTX-DRX-Info-r12 OPTIONAL, hs-scch-LessInfo HS-SCCH-LessInfo-r7 OPTIONAL, mimoParameters MIMO-Parameters-r9 OPTIONAL, mimo4x4Parameters MIMO4x4-Parameters OPTIONAL, dch-Enhancements-Info-FDD DCH-Enhancements-Info-FDD OPTIONAL, maxAllowedUL-TX-Power MaxAllowedUL-TX-Power OPTIONAL, ul-DPCH-Info UL-DPCH-Info-r11 OPTIONAL, ul-EDCH-Information UL-EDCH-Information-r11 OPTIONAL, ul-SecondaryCellInfoFDD UL-SecondaryCellInfoFDD-r14 OPTIONAL, ul-MulticarrierEDCHInfo-TDD128 UL-MulticarrierEDCHInfo-TDD128 OPTIONAL, ul-CLTD-InfoFDD UL-CLTD-InfoFDD OPTIONAL, ul-OLTD-InfoFDD UL-OLTD-InfoFDD OPTIONAL, ul-OtherTTIConfiguration-Info UL-OtherTTIConfiguration-Information OPTIONAL, filteredUPHReportInfo FilteredUEPowerHeadroomReportInfo OPTIONAL, ul-OtherTTIConfigurationInfo-Sec UL-OtherTTIConfigurationInfo-Sec OPTIONAL, filteredUPHReportInfo-Sec FilteredUEPowerHeadroomReportInfo OPTIONAL, dl-HSPDSCH-Information DL-HSPDSCH-Information-r12 OPTIONAL, dl-CommonInformation DL-CommonInformation-r12 OPTIONAL, dl-InformationPerRL-List DL-InformationPerRL-List-r13 OPTIONAL, dl-SecondaryCellInfoFDD DL-SecondaryCellInfoFDD-r11 OPTIONAL, additionalDLSecCellInfoListFDD AdditionalDLSecCellInfoListFDD-r11 OPTIONAL, additionalDLSecCellInfoListFDD2 AdditionalDLSecCellInfoListFDD2 OPTIONAL, commonERGCHInfoFDD CommonERGCHInfoFDD OPTIONAL, dPCCH2InfoFDD DPCCH2InfoFDD OPTIONAL, controlChannelDRXInfo-TDD128 ControlChannelDRXInfo-TDD128-r8 OPTIONAL, sps-Information-TDD128 SPS-Information-TDD128-r8 OPTIONAL, mu-MIMO-Info-TDD128 MU-MIMO-Info-TDD128 OPTIONAL, nonrectResAllocInd-TDD128 Non-rectResAllocInd-TDD128 OPTIONAL, nonrectResSpecTSset-TDD128 Non-rectResSpecTSset-TDD128 OPTIONAL, hs-scch-DrxCellfach-info HS-SCCH-DrxCellfach-info OPTIONAL, simplified-HS-SCCH-type1-operation ENUMERATED { true } OPTIONAL, -- MBMS IEs mbms-PL-ServiceRestrictInfo MBMS-PL-ServiceRestrictInfo-r6 OPTIONAL, -- Measurement IEs for LCR cellDCHMeasOccasionInfo-TDD128 CellDCHMeasOccasionInfo-TDD128-r9 OPTIONAL } | 3 (Low) |  |
| 104 | Placing DL-PhysChCapabilityFDD-vf10ext at correct order Currently it exists along with UE capability UE-RadioAccessCapability-vf10ext-IEs ::= SEQUENCE { physicalChannelCapability PhysicalChannelCapability-vf10ext OPTIONAL, nonCriticalExtensions SEQUENCE {} OPTIONAL } PhysicalChannelCapability-vf10ext ::= SEQUENCE { fddPhysChCapability SEQUENCE { downlinkPhysChCapability DL-PhysChCapabilityFDD-vf10ext OPTIONAL } } DL-PhysChCapabilityFDD-vf10ext ::= SEQUENCE { supportofSimplifiedHS-SCCHType1Operation ENUMERATED { true } OPTIONAL } | DL-PhysChCapabilityFDD-ve30ext ::= SEQUENCE { supportofHS-SCCHDrxOperation ENUMERATED { true } OPTIONAL } DL-PhysChCapabilityFDD-vf10ext ::= SEQUENCE { supportofSimplifiedHS-SCCHType1Operation ENUMERATED { true } OPTIONAL } | 3 (Low) |  |
| 105 | Placing PhysChCapabilityFDD-vf10ext at correct order Currently it exists along with UE capability UE-RadioAccessCapability-vf10ext-IEs ::= SEQUENCE { physicalChannelCapability PhysicalChannelCapability-vf10ext OPTIONAL, nonCriticalExtensions SEQUENCE {} OPTIONAL } PhysicalChannelCapability-vf10ext ::= SEQUENCE { fddPhysChCapability SEQUENCE { downlinkPhysChCapability DL-PhysChCapabilityFDD-vf10ext OPTIONAL } } | PhysicalChannelCapability-ve30ext ::= SEQUENCE { fddPhysChCapability SEQUENCE { downlinkPhysChCapability DL-PhysChCapabilityFDD-ve30ext OPTIONAL, uplinkPhysChCapability UL-PhysChCapabilityFDD-ve30ext OPTIONAL } OPTIONAL } PhysicalChannelCapability-vf10ext ::= SEQUENCE { fddPhysChCapability SEQUENCE { downlinkPhysChCapability DL-PhysChCapabilityFDD-vf10ext OPTIONAL } } | 3 (Low) |  |

# Conclusion

Proposal 1                 RAN6 to agree on correction above issues number 101, 102, 103, 104 and 105.