3GPP TSG-WG SA2 Meeting #159	S2-2311580

# Xiamen, China, October 09 – 13, 2023		(revision of S2-2311419)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0904 | rev | 2 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on status report of FL Training |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2023-09-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In Nnwdaf_MLModelTraining_Notify service operation, there are the following definitions: - Status report of FL training: Accuracy of local model and Training Input Data Information (e.g. areas covered by the data set, sampling ratio, maximum/minimum of value of each dimension, etc.), which are generated by the FL Client NWDAF during FL procedure; In step 1e of Procedures for Maintaining Federated Learning Processes (see clause 6.2C.2.3), the description of status of FL training does not align with the definitions above. It is proposed to correct it. In addition, in the conclusions of KI#8 in TR 23.700-81, NWDAF containing MTLF as FL Server can monitor the accuracy level of the trained model and exclude a Client NWDAF from the FL group if the Accuracy feedback of the common model calculated by the Client NWDAF is much different from the Accuracy calculated from the AnLF. How the FL server receives the accuracy feedback from AnLF is following KI#1. It’s proposed to also clarify that in NOTE 2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Update step 1e of Procedures for Maintaining Federated Learning Processes to keep align with the definition of Status report of FL training. - Update NOTE 2 to include that FL Server NWDAF can initiate AnLF-assisted MTLF ML Models Accuracy Monitoring to check whether the accuracy of the global model calculated using the local training dataset is lower than ML Model accuracy information received from AnLF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - The description of status of FL training in step 1e of Procedures for Maintaining Federated Learning Processes does not align with the definitions in Nnwdaf_MLModelTraining_Notify service operation. - The conclusions of KI#8 in TR 23.700-81 are not fully implemented. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2C.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

6.2C.2.3	Procedures for Maintaining Federated Learning Processes

This clause specifies how to maintain a Federation Learning process in FL execution phase, including FL Server NWDAF triggers reselection, addition, or removal of FL Client NWDAF(s), discovers new FL Client NWDAF(s) via NRF, and FL Client NWDAF(s) joins or leaves Federated Learning process dynamically.

In Federated Learning execution phase, FL Server NWDAF monitors the status changes of FL Client NWDAF(s) and may reselects FL Client NWDAF(s) based on the updated status, availability, and/or capability, etc.

NOTE 1:	FL Server NWDAF checks if there is a need to carry on the FL execution phase and then reselects FL members for the next iteration if needed.

Figure 6.2C.2.3-1: Procedure of FL Server NWDAF reselects FL Client NWDAF(s), FL Client NWDAF(s) Join or Leave Federated Learning Process Dynamically in Federated Learning execution phase

The procedure for FL Server NWDAF reselecting FL Client NWDAF(s), FL Client NWDAF(s) joining or leaving Federated Learning process dynamically is as follows:

1a.	FL Server NWDAF may get the updated status of current FL Client NWDAF(s) via NRF by using Nnrf_NFManagement service (as in clause 5.2.7.2 of TS 23.502 [3]) in the Federated Learning execution phase.

FL Server NWDAF may subscribe to NRF for notifications of status changes of the current NWDAF(s) (FL Client NWDAFs 1…N) by invoking an Nnrf_NFManagement_NFStatusSubscribe service operation. NRF notifies the FL Server NWDAF the status changes of the current FL Client NWDAF(s) by invoking Nnrf_NFManagement_NFStatusNotify service operation(s).

The status of a current FL Client NWDAF could be availability changes, capability changes (e.g. it will not support FL anymore, etc.).

1b.	The current FL Client NWDAF(s) may inform FL Server NWDAF that it is leaving the Federated Learning process by invoking Nnwdaf_MLModelTraining_Notify service operation with Termination Request and cause code (reason for leaving, e.g. high NF load, time availability changes).

1c.	FL Server NWDAF may get the information of the new FL Client NWDAF(s) dynamically via NRF by subscribing to the event that a new FL Client NWDAF registers (Nnrf_NFManagement_NFStatusSubscribe service as in clause 5.2.7.2 of TS 23.502 [3]).

1d.	NWDAF may subscribe for NF load analytics of the FL Client NWDAF(s).

1e.	FL Client NWDAF(s) may send Status report of FL training and Global ML Model Accuracy Information by invoking Nnwdaf_MLModelTraining_Notify service.

2.	FL Server NWDAF checks FL Client NWDAF(s) status based on the received information, and may determine whether reselection of FL Client NWDAF(s) for the next round(s) of Federated Learning is needed based on the received information from step 1.

NOTE 2:	Several examples of the factors that the FL Server NWDAF can consider to reselect the FL Client NWDAF(s) are updated status of FL Client NWDAF reported by NRF is different than the criteria were initially used for selecting the client; characteristics of local training dataset is different than global validation dataset owned by FL Server NWDAF and/or the accuracy of the global model calculated using the local training dataset is much different from that calculated by other FL Client NWDAFs; the accuracy of the global model calculated using the local training dataset is lower than the accuracy calculated using the global validation dataset owned by FL Server NWDAF; the accuracy of the global model calculated using the local training dataset is lower than ML Model accuracy information received in Nnwdaf_MLModelMonitor_Notify when FL Server NWDAF using AnLF-assisted MTLF ML Models Accuracy Monitoring; the load of the FL Client NWDAF (from the NF load Analytics or from the FL Client NWDAF directly) is high; the FL Server NWDAF receives leave request from the FL Client NWDAF; the FL Client NWDAF did not report the status of FL Training within the maximum response time.

3.	[If re-selection is needed as judged in step 2] If step 1c is not performed, FL Server NWDAF may discover new candidate FL Client NWDAF(s) via NRF by using Nnrf_NFDiscovery services as in clause 5.2.7.3 of TS 23.502 [3]. FL Server NWDAF reselects FL Client NWDAF(s) from the current FL Client NWDAF(s) and the new candidate FL Client NWDAF(s) (found in steps 1c or 3). For the new candidate FL Client NWDAF(s), the interaction between FL Server NWDAF and FL Client NWDAF(s) is same as the selection procedure described in clause 6.2C.2.1. The adding / deleting FL Client NWDAF(s) may happen at the end of each iteration.

4.	FL Server NWDAF sends termination request by invoking Nnwdaf_MLModelTraining_Unsubscribe service operation or Nnwdaf_MLModelTrainingInfo_Request service operation with Correlation Termination Flag to the FL Client NWDAF(s), optionally indicating the reason, e.g. FL Client NWDAF is unselected by the FL Server NWDAF for the FL process, or the FL process is suspended, etc. And FL server may also send the updated global ML model information to the unselected FL client NWDAF. FL Client NWDAF(s) terminates operations for the Federated Learning process if receive termination request from the FL Server NWDAF and may perform further action to be qualified in participation of FL training in the next cycles.

NOTE 3:	In the case of high load, the FL Client NWDAF can e.g. decline new service request. In the case of low accuracy, the FL Client NWDAF can gather new local training data.

# * * * * End of changes * * * *