TSG SA Meeting #SP-102	SP-231285

11 - 15 December 2023, Edinburgh, Scotland

3GPP TSG-SA5 Meeting #152                    	S5-238106

# Chicago (US), 9 - 13 October 2023

Title:	LS on network energy related information exposure

Response to:

Release:	3GPP Rel-18

Work Item:	EE5GPLUS_Ph2

Source:	3GPP SA5

To:	3GPP SA2

Cc:	3GPP SA, 3GPP SA1

Contact Person:

#### Name:	Jean-Michel Cornily

E-mail Address:	jean <dot> michel <dot> cornily <at> huawei <dot> com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

SA5 has observed the TR 23.700-66 V0.1.0 Key Issue #1 (Network energy related information exposure), and would like to provide SA2 with the following information.

SA5 has been already working on energy efficiency and energy saving for several releases and has specified Energy Consumption (EC) KPIs, including (but not limited to):

- Energy Consumption of a gNB (ECgNB) – see TS 28.554 clause 6.7.3.4.2

- Energy Consumption of a 5G Network Function (ECNF) – see TS 28.554 clause 6.7.3.1.1

- Estimated Energy Consumption of a Virtualized Network Function (ECVNF) – see TS 28.554 clause 6.7.3.1.2

- Energy Consumption of a Physical Network Function (PNF) – see TS 28.552 clause 5.1.1.19 - as well as other Power, Energy, Environmental (PEE) measurements, including (but not limited to):

- Minimum, maximum, average power consumption of a PNF

- Minimum, maximum, average temperature of a PNF

- Voltage, humidity, etc. of a PNF

, as well as Energy Efficiency KPIs, including (but not limited to):

- Energy Efficiency of the NG-RAN (EEMN,DV) – see TS 28.554 clause 6.7.1.1

- Energy Efficiency of the 5GC (EE5GC,UO,UP,DV) – see TS 28.554 clause 6.7.4

- Energy Efficiency of a network slice – see TS 28.554 clause 6.7.2

With regard to how the measurements / KPIs can be collected and exposed by OAM: as per our Service Based Management Architecture (SBMA), any authorized consumer willing to collect such measurements / KPIs may use one of the following mechanisms:

Option#1:

- create an instance of a performance metric production job (i.e. an instance of the PerfMetricJob information object class – see TS 28.622 clause 4.3.31) by invoking the createMOI operation of the Provisioning Management Service (MnS) (see TS 28.532 clause 11.1.1.1). The consumer must then specify:

- Which measurement(s) / KPI(s) are expected to be collected, such as e.g. ECNF, ECgNB, etc.;

- On which network entities (represented by managed objects) such as e.g. a SMF x , UPF y, etc.). in this option, it is assumed that the consumer may have detailed knowledge of the network and hence may identify the exact object instance(s) producing the required management data;

- Which granularity period is expected to produce measurements / KPIs (in seconds);

- Which reporting method is expected, mainly:

  - file-based: in this method the performance data is accumulated for certain time before it is reported; the data is delivered as a file; the file content encoding technique can be either XML or ASN.1,

  - stream-based: in this method, the producer, when the performance data are ready, sends the performance data to the consumer. The volume of the performance data reported by streaming is expected to be small, and the granularity period of the performance data stream and is expected to be short. The stream content encoding technique can be either GPB or ASN.1.

Depending on the reporting method, the consumer collects the measurements / KPIs as follows:

- In case of file-based reporting, the producer sends the notification NotifyFileReady to subscribed consumer(s) when a new file becomes available on the producer for upload by consumer(s);

- In case of stream-based reporting, the producer sends units of streaming data to the consumer by using the reportStreamData operation.

Option#2:

- create an instance of a management data collection (i.e. an instance of the ManagementDataCollection information object class – see TS 28.622 clause 4.3.47) by invoking the createMOI operation of the Provisioning MnS (see TS 28.532 clause 11.1.1.1). The consumer must then specify:

- the management data which shall be reported (can be of different kind: Trace, MDT (Minimization of Drive Test), RLF (Radio Link Failure) report, RCEF (RRC Connection Establishment Failure) report, PM (performance measurements), KPI (end-to-end key performance indicators) or a combination of these);

- the "targetNodeFilter" to target object instance(s) producing the required management data. It is assumed that the consumer may not have detailed knowledge of the network and hence may not identify the exact object instance(s) producing the required management data. In this case consumer can request management data, specified by 3GPP, produced by certain object instance (s) based on a particular location, the domain (CN or RAN) of the object instances, and the handled traffic (CP or UP) of the object instances;

- the collectionTimeWindow, i.e. the time window for which the management data should be reported;

- the method and associated control parameters for reporting the produced management data to MnS consumers – see reporting method here above;

- the "dataScope", i.e. whether the management data should be reported per S-NSSAI or per 5QI, if applicable.

NOTE: for both above methods, if the consumer opts for file-based reporting, the consumer, to be informed of the availability of new measurement data files, must subscribe for notifications by creating a notification subscription (i.e. an instance of the NtfSubscriptionControl information object class – see TS 28.622 clause 4.3.22, with attribute notificationTypes containing notifyFileReady) by invoking the createMOI operation of the Provisioning MnS (see TS 28.532 clause 11.1.1.1).

2. Reference documentation:

- 3GPP TS 28.310: Management and orchestration; Energy efficiency of 5G

- 3GPP TS 28.554: Management and orchestration; 5G end to end Key Performance Indicators (KPI)

- 3GPP TS 28.552: Management and orchestration; 5G performance measurements

- 3GPP TS 28.622: Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)

- 3GPP TS 28.532: Management and orchestration; Generic management services

3. Actions:

To SA2: please take the above information into consideration.

5. Date of Next TSG-SA WG5 Meetings:

SA5#153	29 Jan. – 2 Feb. 2024	Sevilla, Spain

SA5#154	15 – 19 Apr. 2024	TBC, China