3GPP TSG-SA5 Meeting #151 	S5-237047

Xiamen, China, 9 - 13 October 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.533 | CR | 0120 | rev | 1 | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR TS 28.533 Add example of RAN domain management capabilities mapped with ZSM |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Telefónica, Deustche Telekom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eSBMA |  |  |  |  |  | Date: |  |  | 2023-09-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Based on the study in clause 5.8 TR 28.925, some management capabilities provided by domain management functions are represented by IOCs defined in 3GPP. We propose to add description in clause 5.3 to further illustrate the domain management capabilities represented by 3GPP IOCs, mapping with ZSM domain capability categories |  |  |  |  |  |  |  |  |
| Summary of change: |  | This proposal adds an example of deployment scenario for RAN domain management capabilities. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There is lack of detail description on how ZSM architectual framework is related. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex A.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

A.x	Management domain provided management services mapped with ZSM

In ETSI GS ZSM 002 [29], management services provided by management domain includes the following categorization:

- Domain data collection

- Domain analytics

- Domain intelligence

- Domain orchestration

- Domain control

Figure A.x-1 shows an example of potential RAN domain management capabilities and how these different 3GPP RAN management capabilities fit into the ZSM architecture framework. These capabilities, produced by one or more management functions, are offered via corresponding MnSs through the RAN domain integration fabric:

- Domain data collection

- RAN domain threshold monitoring control capabilities are provided by ThresholdMonitor IOC defined in TS 28.622 [32].

- RAN domain data collection control capabilities are provided by ManagementDataCollection IOC defined in TS 28.622 [32].

- Domain analytics

- RAN domain MDA capabilities are provided by MDAFunction IOC defined in 28.104 [57].

- Domain intelligence

- RAN domain Intent handling capabilities are provided by IntentHandlingFunction IOC defined in TS 28.312 [46].

- Domain orchestration

- Domain control

- RAN domain Assurance Closed Loop capabilities are provided by AssuranceClosedControlLoop IOC defined in TS 28.536 [38].

- RAN domain Centralized SON Energy Saving capabilities are provided by CESManagementFunction IOC defined in TS 28.541 [4].

- RAN domain Capacity and Coverage optimization capabilities are provided by CCOFunction IOC defined in TS 28.541 [4].

- RAN domain PCI configuration capabilities are provided by CPCIConfigurationFunction IOC defined in TS 28.541 [4].

- RAN domain PM control capabilities are provided by PerfMetricJob IOC defined in TS 28.622 [32].

- RAN domain FM control capabilities are provided by AlarmList IOC defined in TS 28.622 [32].

- RAN domain CM control capabilities are represented by NR NRM defined in TS 28.541 [4].

NOTE: Domain integration fabric and Cross-domain integration fabric are ZSM defined management functions (see ZSM 002 [29] for further details). These functions provide, among other capabilities, MnS discovery functionality (see further details in clause 4.7 and TS 28.622 [32]).

Figure A.x-1: An example of deployment scenario for RAN management functions

End of Change