3GPP TSG-SA5 Meeting #151 	S5-237041

Xiamen, China, 9 - 13 October 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.312 | CR | 0150 | rev | 1 | Current version: | 18.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR TS 28.312 add intent conflict resolution based on intent preemption |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | IDMS_MN_ph2 |  |  |  |  |  | Date: |  |  | 2023-09-29 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The potential solution of intent conflicts based on preemption has been discussed and agreed in TR 28.912. This CR is to normatively propose the proceduer of intent conflict resolution based on intent pre-emption. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The proceduer of intent conflict resolution based on intent preemption have been added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The intent conflict resolution based on intent preemption shall not be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

### 6.3.6	Intent Conflict Resolution

Figure 6.3.6-1: Procedure for Intent Conflict Resolution

1)	Consumer subscribes to receive notiyMOICreation and notifyMOIDeletion using NtfSubscriptionCtrl as defined in TS 28.622 [6].

2)	The producer detects a conflict

3)	Producer instantiate IntentReportInfo IOC with attributes specifying the intent conflict information.

4)	Producer notify the creation of IntentReport IOC using notifyMOICreation request as defined in TS 28.532 [3]. The notification may include measures to solve the conflict including intent deletion and/or intent modification.

5)	Consumer send a response

6)	Consumer may send a deleteMOI request to delete the intent identified by the producer in report.

7)	Producer notify consumer about the deletion of the intent using notifyMOIDeletion request as defined in TS 28.532 [3].

8)	Consumer sends a response

9)	Consumer may send a ModifyMOIAttribute request to modify the Expectation and/or Target based on the information provided in the report.

10)	Producer notify consumer about the modification of the Expectation and/or Target using notifyMOIAttributeValueChange request as defined in TS 28.532 [3].

11)	Consumer sends a response.

### 6.3.6.X	Resolution of an intent conflict based on  pre-emption

In procedure for intent conflict resolution, it is necessary to specify an intent that is target of intent deletion or intent modification. MnS producer specify the intent based on intent priority level. Intent that is low-intent priority will become the target of intent deletion or intent modification. However, if two intents with the same priority conflict, it is impossible for MnS producer to specify the intent that is target of intent deletion or intent modification. In this case, the intent that is target of intent deletion or intent modification is identified using preemption. Based on preemption, the judgment is made using the attributes of "intent pre-emption capbility". The "intent pre-emption capbility" specify whether pre-emption shall be applied. The role of MnS Consumer and MnS Producer is as follows; MnS Conusmer can set intent pre-emption capbility as "T" for priorited intent during pre-emption. MnS Producer decides the target of intent deletion or intent modification during pre-emption. Here, there are existing intent and new intent. When MnS Producer creates new intent, it happnes conflict between existing intent and new intent with same priority. If intent pre-emption capbility="T" for existing Intent, target of intent deletion or intent modification is new intent. If intent pre-emption capbility="F” for existing Intent and intent pre-emption capbility="T" for new intent, target of intent deletion or intent modification will be existing Intent. If both existing Intent and new intent has same parameter of intent pre-emption capability, new Intent will be target of intent deletion or intent modification.

End of changes