3GPP TSG-SA5 Meeting #151 	S5-237111

Xiamen, China, 09 - 13 October 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0482 | rev | 1 | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR TS 28.552 Add PLMN granularity for performance measurements for NG-RAN MOCN network sharing scenario |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Unicom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | MANS_ph2 |  |  |  |  |  | Date: |  |  | 2023-09-26 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | This submission implements the solution for “PLMN granularity requirement of performance measurements for MOCN” as recommended in TR 28.835 (Issue #8). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add PLMN granularity for performance measurements for NG-RAN MOCN network sharing scenario |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1.4.4, 5.1.1.23.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

##### 5.1.1.4.4	Max number of stored inactive RRC Connections

a)	This measurement provides the max number of users in RRC inactive mode during each granularity period. The measurement is optionally split into subcounters per PLMN ID.

b)	SI

c)	This measurement is defined according to measurement "Max number of stored inactive UE contexts" in TS 38.314 [29]. Each measurement is optionally performed per PLMN ID.

d)	Each measurement is a single integer value. If the optional measurement is performed, the number of measurements is equal to the number of supported PLMNs.

e)	The measurement name has the form RRC.InactiveConnMax or optionally RRC.InactiveConnMax.PLMN, where PLMN identifies the PLMN ID.

f)	NRCellCU.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this measurement is for monitoring the memory allocation due to storage of inactive RRC connections.

2nd Change

##### 5.1.1.23.4	Max number of Active UEs in the UL per cell

a)	This measurement provides the max number of active DRBs for UEs in an NRCellDU. The measurement is optionally split into subcounters per QoS level (mapped 5QI or/and QCI in NR option 3), subcounters per S-NSSAI and subcounters per PLMN ID.

b)	DER (n=1)

c)	This measurement is defined by the measurement " Max number of Active UEs in the UL per DRB per cell " (see clause 4.2.1.3.5 in TS 38.314 [29]). Separate counters are optionally maintained for each mapped 5QI (or/and QCI for option 3), for each S-NSSAI and for each PLMN ID.

d)	The number of measurements is equal to one. If the optional QoS level measurement is perfomed, the number of measurements is equal to the number of mapped 5QIs (or/and number of QCI for option 3), and the number of S-NSSAIs, and the number of PLMN IDs.

e)	The measurement name has the form DRB.MaxActiveUeUl, 
DRB.MaxActiveUeUl.QOS where QOS identifies the target quality of service class, and
DRB.MaxActiveUeUl.SNSSAI, where SNSSAI identifies the S-NSSAI, and

DRB.MaxActiveUeUl.PLMN, where PLMN identifies the PLMN ID.

f)	NRCellDU.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this measurement is for performance assurance within integrity area (user plane connection quality).

End of Change