3GPP TSG-SA5 Meeting #152	S5-238094

Chicago,US, 13-17 November 2023											Revision of SP-22XXXX

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.255 | CR | 0484 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR 32.255 Clarification on EPS to 5GS handover for roaming in Home routed scenario |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18 |  |  |  |  |  | Date: |  |  | 2023-11-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In the message flows about PDU session charging for interworking with EPC, the triggers and charging identifier used for the message flows should be clarified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the triggers description for the EPS to 5GS handover for roaming in Home routed scenario. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The description for the message flow of EPS to 5GS handover for roaming in Home routed scenario is unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2.11.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is revision of S5-237845. |  |  |  |  |  |  |  |  |

First change

5.2.2.11.7	EPS to 5GS handover for roaming in Home routed scenario

The following figure 5.2.2.11.7.1 describes the handover from EPS to 5GS for roaming in home routed scenario, focusing on the Charging Identifier generation mechanism.

Figure 5.2.2.11.7.1: EPS to 5GS mobility without N26 in Home-Routed Roaming

1.	The UE is attached in EPC and initiates a new PDU connection (IP-CAN session).

1ch-a. A Charging Data Request [Initial] is sent to CHF via PGW-C+SMF in HPLMN, triggered by "Start of PDU Session". The Charging Identifier included is generated by PGW-C +SMF in HPLMN.

1ch-b. The H-CHF opens a CDR 1ch-c. The H-CHF acknowledges by sending Charging Data Response [Initial] to the PGW-C+SMF.

2. UE initiates registration procedure to the 5GS and indicates that it is moving from EPC. UE requests PDU Session Establishment.

2ch-a. SMF in VPLMN creates a Charging Identifier (VPLMN created Charging Identifier) for the PDU session and sends the Charging Data Request [Initial] to CHF in VPLMN triggered by "Start of PDU Session"

2ch-b. The CHF in VPLMN opens a CDR.

2ch-c. The CHF in VPLMN acknowledges by sending Charging Data Response [Initial] to the SMF and optionally supplies a VPLMN selected “Roaming Charging Profile” to the V-SMF which override the default one.

3. UPF selection and V-SMF sends the PDU session establishment request to H-SMF.

3ch-a. A Charging Data Request [Update] is sent to CHF in HPLMN triggered by "Handover start", indicating the "Roaming Charging Profile" received from the VPLMN. The same Charging Identifier (home provided Charging Identifier) in step 1ch-a will be the only one Charging Identifier used for any subsequent charging data request sent from H-SMF to CHF i.e. , the Charging Identifier generated in PGW-C +SMF in HPLMN.

3ch-b. The CHF in HPLMN updates the CDR.

3ch-c. The CHF in HPLMN acknowledges by sending Charging Data Response [Update] to the PGW-C+SMF and supplies the HPLMN selected "Roaming Charging Profile" to the PGW-C+SMF.

4. H-SMF sends the PDU session establishment response with Charging Identifier (home provided Charging Identifier).

4ch-a. A Charging Data Request [Update] is sent to CHF in VPLMN triggered by "Handover complete", with the "Roaming Charging Profile", Charging Identifier (visited created Charging Identifier), and Home Provided Charging Identifier.

4ch-b. The CHF updates the CDR.

4ch-c. The CHF acknowledges by sending Charging Data Response [Update].

In subsequent charging data request sent from V-SMF to V-CHF, the Charging Identifier has the value of Home Provided Charging Identifier, and the Home Provided Charging Identifier is not present.

End of change