3GPP TSG-SA5 Meeting #152	S5-238091

Chicago,US, 13-17 November 2023

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.256 | CR | 0026 | rev | 1 | Current version: | 18.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR 32.256 Update on Triggers for AMF charging |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18 |  |  |  |  |  | Date: |  |  | 2023-11-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP o. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Triggers used for the IEC, PEC, ECUR charging scenarios should not be included in the Charging Data Response. The trigger setting for IEC, PEC, ECUR from CHF (disable/enable the triggers) can be done by Charging profile Mechanism. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove the Common Trigger in the charging data request |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Nchf_Convergedcharging is not aligned in the stage 2 and stage 3. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.1.1, 6.1.1.2, 6.1.1.3, 6.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This is a CR related to DP S5-237838. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is revision of S5-237841. |  |  |  |  |  |  |  |  |

First change

#### 5.2.1.1	General

5G connection and mobility converged charging, when activated, may be performed by the AMF interacting with CHF using Nchf specified in TS 32.290 [57] and TS 32.291 [58]. In order to provide the data required for the management activities outlined in TS 32.240 [1] (Credit-Control, accounting, billing, statistics, etc.), the AMF shall be able to perform converged charging for each of the following:

-	Charging data related to registration.

-	Charging data related to N2 connection between the AN and the AMF.

-	Charging data related to user location.

The AMF shall use the AMF charging profile defined in clause 5.2.1.2. The default AMF charging profile is provided by Charging Characteristics applicable to AMF are described in Annex A.

The AMF shall be able to perform converged charging by interacting with CHF, for charging data related to registration, connection and location management. The Charging Data Request and Charging Data Response are exchanged between the AMF and the CHF, based on IEC, PEC or ECUR scenarios as specified in TS 32.290 [57]. The Charging Data Request is issued by the AMF towards the CHF when certain conditions (chargeable events) are met.

The contents and purpose of each charging event that triggers interaction with CHF, as well as the chargeable events that trigger them, are described in the following clauses.

A detailed formal description of the converged charging parameters defined in the present document is to be found in TS 32.291 [58].

A detailed formal description of the CDR parameters defined in the present document is to be found in TS 32.298 [51].

Next change

#### 6.1.1.2	Charging Data Request message

Table 6.1.1.2.1 illustrates the basic structure of a Charging Data Request message from the AMF as used for 5G connection and mobility converged charging.

Table 6.1.1.2.1: Charging Data Request message contents

| Information Element | Category for converged charging | Description |
| --- | --- | --- |
| Session Identifier | OC | Described in TS 32.290 [57] |
| Subscriber Identifier | OM | Described in TS 32.290 [57] In case Subscriber Identifier (i.e. SUPI) is not present (for emergency service), the User Equipment Info in table 6.2.1.2.1. shall be present for identifying the user. |
| NF Consumer Identification | M | Described in TS 32.290 [57] |
| NF Functionality | M | Described in TS 32.290 [57] |
| NF Name | OC | Described in TS 32.290 [57] |
| NF Address | OC | Described in TS 32.290 [57] |
| NF PLMN ID | OC | Described in TS 32.290 [57] |
| Charging Identifier | - | This field is not applicable. |
| Invocation Timestamp | M | Described in TS 32.290 [57] |
| Invocation Sequence Number | M | Described in TS 32.290 [57] |
| Retransmission Indicator | - | This field is not applicable. |
| One-time Event | OC | Described in TS 32.290 [57] |
| One-time Event Type | OC | Described in TS 32.290 [57] |
| Notify URI | - | This field is not applicable. |
| Supported Features | OC | Described in TS 32.290 [57] |
| Service Specification Information | OC | Described in TS 32.290 [57] |
| Triggers | - | This field is not applicable. |
| Multiple Unit Usage | OC | Described in TS 32.290 [57]. |
| Rating Group | M | Described in TS 32.290 [57] |
| Requested Unit | OC | Described in TS 32.290 [57] |
| AMF Identifier | OC | This field holds the AMF identifier. |
| AMF Charging Profile | OC | This field holds the applicable AMF Charging Profile |
| Registration Charging Information | OM | This field holds the registration specific information described in clause 6.2.1.2 |
| N2 connection Charging Information | OM | This field holds the N2 connection specific information described in clause 6.2.1.3 |
| Location Reporting Charging information | OM | This field holds the Location Reporting specific information described in clause 6.2.1.4 |

Next change

#### 6.1.1.3	Charging data response message

Table 6.1.1.3.1 illustrates the basic structure of a Charging Data Response message from the CHF as used for 5G connection and mobility converged charging.

Table 6.1.1.3.1: Charging Data Response message contents

| Information Element | Category for converged charging | Description |
| --- | --- | --- |
| Session Identifier | OC | Described in TS 32.290 [57] |
| Invocation Timestamp | M | Described in TS 32.290 [57] |
| Invocation Result | OC | Described in TS 32.290 [57] |
| Invocation Sequence Number | M | Described in TS 32.290 [57] |
| Session Failover | OC | This field is not applicable. |
| Supported Features | OC | Described in TS 32.290 [57] |
| Triggers | - | This field is not applicable. |
| Multiple Unit Information | OC | This field is applicable for ECUR. |
| Result Code | OC | Described in TS 32.290 [57] |
| Rating Group | OM | Described in TS 32.290 [57] |
| Granted Unit | OC | Described in TS 32.290 [57] |
| Validity Time | OC | Described in TS 32.290 [57] |
| AMF Charging Profile | OC | This field holds the AMF Charging Profile to be applied |
| Location Reporting Charging information | Oc | This field holds the Location Reporting specific information described in clause 6.2.1.4 |

Next change

### 6.2.3	Detailed message format for converged charging

The following clause specifies per Operation Type the charging data that are sent by AMF for 5G connection and mobility converged charging.

The Operation Types are listed in the following order: I (Initial)/T (Termination)/E (Event). Therefore, when all Operation Types are possible it is marked as ITE. If only some Operation Types are allowed for a node, only the appropriate letters are used (i.e. IT or E) as indicated in the table heading. The omission of an Operation Type for a particular field is marked with "-" (i.e. I-E). Also, when an entire field is not allowed in a node the entire cell is marked as "-".

Table 6.2.3.1 defines the basic structure of the supported fields in the Charging Data Request message for AMF converged charging.

Table 6.2.3.1: Supported fields in Charging Data Request message

| Information Element |  | Functionality of AMF | Reg. |  | N2 cnt |  | Loc. Report. |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Supported Operation Types | I/T/E |  | E |  | E |  |
| Session Identifier |  |  | ITE |  | E |  | E |  |
| Subscriber Identifier |  |  | ITE |  | E |  | E |  |
| NF Consumer Identification |  |  | ITE |  | E |  | E |  |
| Charging Identifier |  |  | - |  | - |  | - |  |
| Invocation Timestamp |  |  | ITE |  | E |  | E |  |
| Invocation Sequence Number |  |  | ITE |  | E |  | E |  |
| Retransmission Indicator |  |  | - |  | - |  | - |  |
| One-time Event |  |  | --E |  | E |  | E |  |
| One-time Event Type |  |  | --E |  | E |  | E |  |
| Notify URI |  |  | - |  | - |  | - |  |
| Supported Features |  |  | I-E |  | E |  | E |  |
| Service Specification Information |  |  | ITE |  | E |  | E |  |
| Triggers |  |  | - |  | - |  | - |  |
| Multiple Unit Usage |  |  | ITE |  | - |  | - |  |
| AMF Identifier |  |  | ITE |  | E |  | E |  |
| AMF Charging Profile |  |  | IT-E |  | E |  | E |  |
| Registration Charging Information |  |  |  |  |  |  |  |  |
| Registration Message type |  |  | ITE |  | - |  | - |  |
| User Information |  |  | ITE |  | - |  | - |  |
| UE 5GMM Core Network Capability |  |  | ITE |  | - |  | - |  |
| MICO Mode Indication |  |  | ITE |  | - |  | - |  |
| SMS Supported Indication |  |  | ITE |  | - |  | - |  |
| Access Type |  |  | ITE |  | - |  | - |  |
| RAT Type |  |  | ITE |  | - |  | - |  |
| TAI List |  |  | ITE |  | - |  | - |  |
| User Location Info |  |  | ITE |  | - |  | - |  |
| PSCell Information |  |  | ITE |  | - |  | - |  |
| UE Time Zone |  |  | ITE |  | - |  | - |  |
| Mobility Restrictions |  |  | ITE |  | - |  | - |  |
| Requested NSSAI |  |  | ITE |  | - |  | - |  |
| Allowed NSSAI |  |  | ITE |  | - |  | - |  |
| Rejected NSSAI |  |  | ITE |  | - |  | - |  |
| NSSAI mapping list |  |  | ITE |  | - |  | - |  |
| AMF UE NGAP ID |  |  | ITE |  | - |  | - |  |
| RAN UE NGAP ID |  |  | ITE |  | - |  | - |  |
| RAN Node Id |  |  | ITE |  | - |  | - |  |
| SNPN ID |  |  | ITE |  | - |  | - |  |
| CAG ID List |  |  | ITE |  | - |  | - |  |
| N2 Connection Charging Information |  |  |  |  |  |  |  |  |
| N2 Connection Message type |  |  | - |  | E |  | - |  |
| User Information |  |  | - |  | E |  | - |  |
| User Location Info |  |  | - |  | E |  | - |  |
| PSCell Information |  |  | - |  | E |  | - |  |
| UE Time Zone |  |  | - |  | E |  | - |  |
| RAT Type |  |  | - |  | E |  | - |  |
| AMF UE NGAP ID |  |  | - |  | E |  | - |  |
| RAN UE NGAP ID |  |  | - |  | E |  | - |  |
| RAN Node Id |  |  | - |  | E |  | - |  |
| Mobility Restrictions |  |  | - |  | E |  | - |  |
| Allowed NSSAI |  |  | - |  | E |  | - |  |
| NSSAI mapping list |  |  | - |  | E |  | - |  |
| RRC Establishment Cause |  |  | - |  | E |  | - |  |
| Location Reporting Charging information |  |  |  |  |  |  |  |  |
| Location reporting Message type |  |  | - |  | - |  | E |  |
| User Information |  |  | - |  | - |  | E |  |
| User Location Info |  |  | - |  | - |  | E |  |
| PSCell Information |  |  | - |  | - |  | E |  |
| UE Time Zone |  |  | - |  | - |  | E |  |
| Presence Reporting Area Information |  |  | - |  | - |  | E |  |
| RAT Type |  |  | - |  | - |  | E |  |

Table 6.2.3.2 defines the basic structure of the supported fields in the Charging Data Response message for AMF converged charging.

Table 6.2.3.2: Supported fields in Charging Data Response message

| Information Element | Functionality of AMF | Reg. | N2 cnt | Loc. Report. |
| --- | --- | --- | --- | --- |
|  | Supported Operation Types | ITE | E | E |
| Session Identifier |  | ITE | E | E |
| Invocation Timestamp |  | ITE | E | E |
| Invocation Result |  | ITE | E | E |
| Invocation Sequence Number |  | ITE | E | E |
| Session Failover |  | I-- | - | - |
| Supported Features |  | I-E | E | E |
| Triggers |  | - | - | - |
| Multiple Unit information |  | - | - | - |
| AMF Charging Profile |  | ITE | E | E |
| Location Reporting Charging information. |
| Location reporting Message type |  | - | - | - |
| User Information |  | - | - | - |
| User Location Info |  | - | - | - |
| PSCell Information |  | - | - | - |
| UE Time Zone |  | - | - | - |
| Presence Reporting Area Information |  | - | - | E |
| RAT Type |  | - | - | - |

End of change