3GPP TSG SA Plenary #102	SP-231503

Edinburgh, Scotland, 11 - 15 December 2023	(revision of S2-2313750)

# Source:	Deutsche Telekom AG (Moderator)

# Title:	New WID on Architecture support of Roaming Value Added Services

# Document for:	Approval

# Agenda Item:	6.4.2

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title:	Architecture support of roaming value-added services

Acronym:	RVAS_ARCH

Unique identifier:

Potential target Release:	Rel-19

# 1	Impacts

{For Normative work, identify the anticipated impacts. For a Study, identify the scope of the study}

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  |  | X |  |
| No | X | X | X |  |  |
| Don't know |  |  |  |  | X |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a …

|  | Study |
| --- | --- |
|  | Normative – Stage 1 |
| X | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

## 2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| RVAS | S1 | 990051 | Roaming value added services |

### 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
|  |  | {optional free text} |

Dependency on non-3GPP (draft) specification: None

# 3	Justification

Roaming Value Added Services (RVAS) form part of the roaming services ecosystem and have been discussed within SA1 and the GSMA. RVAS have traditionally been provided by either the PLMN or outsourced to a fully trusted entity using non-standard solutions. The focus of this work is on RVAS enabled by the PLMN for 5GS roaming.

With the introduction of e2e encryption for roaming in 5GS, it is in some cases not possible to provide RVAS in a proprietary way and these services therefore need to be standardized.

There are 2 use cases of RVAS in 5GS enabled by the PLMN as described in TR22.877 for which requirements have been specified in TS 22.261 and that have SA2 impact:

Work Task1: Welcome SMS

The “Welcome SMS” service sends a SMS to a roaming subscriber’s UE when the UE is registered in a new network for the first time (or re-entering the network after a longer absence). The SMS typically follows a predefined template and is sent on behalf of the home operator and may contain relevant information related to the visited country, e.g., the cost to call home, how to reach the operator’s customer service, local emergency services number, etc.

When a successful UE registration in a VPLMN is identified by the HPLMN, the HPLMN notifies the “Welcome SMS” application server. The notification needs to include equipment and subscription identifier (e.g. in order to be able to distinguish an IoT device or subscription intended to be used by a human user). As a result of  the notification, the application server can indicate specific actions to the HPLMN (e.g., send an SMS to the UE).

Event Exposure via NEF supports “Roaming Status” as a Monitoring Event (see TS 23.502, cl.4.15.3.1). The trusted AF can subscribe to ANY UE to be notified by the UDM about the Roaming Status. A registration of a UE would trigger a notification and provide the roaming status, the VPLMN ID, the PEI and the MSISDN.

Based on that information and additional information the AF can create a text that is sent via SMS towards the roaming UE.

Currently, the “Roaming Status” event does not provide equipment identifier, PEI, as part of the notification as required by TS 22.261 and therefore no distinction can be made as to which kind of device is currently being used.

Work Task2: Subscriber-based routing to a particular core network (e.g. in a different country)

The “Subscriber-based routing to a particular core network” service forwards signaling and user plane traffic targeting HPLMN of the users to a target PLMN. Which signaling and/or user plane traffic is forwarded to a target PLMN depends on the scenario and is determined by the HPLMN e.g. based on the user subscription.

Some operators use more than one PLMN ID, e.g., multi-national operators. Due to certain business and operational demands, it might be necessary to route certain signaling and user plane traffic of a certain customer segment (set of subscribers), e.g. from a certain SUPI range of USIMs, of a PLMN to another PLMN and to further handle the subscriber there. This means the UE is not handled by the "real" HPLMN (according to MNC and MCC) but by some alternative PLMN (called partner PLMN).

This subscriber-based routing enables the case where several national subsidiaries of a multi-national operator offer various services for different customer segments but for operational efficiency the actual service for a certain customer segment (set of subscribers), is provided by only one dedicated network.

Two scenarios are targeted:

Scenario 1) MM + SM signaling and user plane traffic of a user (belonging to a specific SUPI range) is forwarded to a target PLMN (as required in TS 22.261)

Scenario 2) Some PDU sessions of a user (belonging to a specific SUPI range and with certain subscription) have their signaling and traffic routed to a partner PLMN or to the HPLMN on a per HPLMN policy decision.

# 4	Objective

The objectives of this work item are the stage 2 aspects for the following RVAS in 5GS, enabled by the PLMN according to the requirements specified in TS 22.261:

Work Task1:

Welcome SMS feature support at HPLMN when the user is registered for the first time (or after long period) in a VPLMN. 5GC shall expose subscription information (e.g. MSISDN) and information about the device currently being used (e.g. a device category, an equipment identifier or other information that enables to determine whether a welcome SMS is needed to be sent or not) to the AF.

Work Task2:

Support of subscriber-based routing to a particular core network (e.g. in a different country, e.g. to a particular partner PLMN) when the user is registering to the network, either from HPLMN or from VPLMN

  - Close cooperation between HPLMN and partner PLMN is assumed.

  - UDM/AUSF may be deployed in partner PLMN but would use MCC MNC of HPLMN; in that case

    - SUCI de-concealment and credentials for the AKA authentication is stored in the partner PLMN for the specific SUPI range.

    - signaling to AUSF is forwarded to the partner PLMN based on Routing Indicator (i.e. NRF based AUSF discovery result based on Routing Indicator).

    - signaling to UDM is forwarded to the partner PLMN based on SUPI (i.e. NRF based UDM discovery result based on SUPI range), Routing Indicator.

    - Potential clarifications  for vNRF – hNRF interaction as well as for partner NRF.

  - And for the scenarios (1 and 2) related SM, i.e all PDU sessions of a user have their signaling and user traffic handled by a partner PLMN or some PDU sessions of a user have their signaling and user traffic handled by a partner PLMN or the HPLMN on a per HPLMN policy; the decision for SM signaling and User plane forwarding to the partner PLMN is done on a per DNN+S-NSSAI basis and/or SUPI range and/or subscription.

Impacts to VPLMN entities should be minimized (if impact is unavoidable to support the scenario), including no configuration.

Note 1: The feature is defined to keep the principle that at most 2 SMFs are involved for the establishment of a PDU Session handled by a partner PLMN

Note 2: This mechanism is not visible for the UE and therefore UEs do not need any additional features to support this RVAS.

Note 3: Any needed functionality on security e.g. on SEPPs for routing signaling either to HPLMN SEPP or partner PLMN SEPP (which would still use MCC MNC of HPLMN) needs to be taken into account by SA3.

## TU estimates and dependencies

| Work Task ID | TU Estimate (Study) | TU Estimate (Normative) | RAN Dependency (Yes/No/Maybe) | Inter Work Tasks Dependency Editor’s Note: This column should highlight if WT#x is self-contained, or it depends on the completion of other WTs |
| --- | --- | --- | --- | --- |
| WT1 | - | 0.5 | No | self-contained |
| WT2 | - | 1 | No | self-contained |

Total TU estimates for the study phase: -

Total TU estimates for the normative phase: 1.5

Total TU estimates: 1.5

# 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 23.501 | 5GC Architectural support of roaming value-added services |  |  |
| 23.502 | 5GC procedures update to support roaming value-added services |  |  |

# 6	Work item Rapporteur(s)

TBD

# 7	Work item leadership

SA2

# 8	Aspects that involve other WGs

Stage 2 5GS security specifications.

# 9	Supporting Individual Members

Supporting IM name

AT&T

BT

Deutsche Telekom

Ericsson

Huawei

KPN

Nokia

Nokia Shanghai Bell

OPPO

Orange

Telecom Italia

T-Mobile US

Verizon

Vodafone

ZTE