TSG SA Meeting #SP-102	SP-231533

11 - 15 December, 2023, Edinburgh, Scotland

# Title:	Reply LS to GSMA on Monitoring of Encrypted 5GS Signalling Traffic

# Response to:	LS (S5-237343) to 3GPP re Monitoring of Encrypted 5GS Signalling Traffic, Reply LS (S3-235009/S5-238097) on Monitoring of Encrypted 5GS Signalling Traffic

# Release:	Rel-18

# Work Item:	-

Source:	TSG SA

To:	GSMA 5GPKIWP

CC:	3GPP SA2, SA5, SA3

Contact person:	Zou Lan, 3GPP SA WG5 Chair (zoulan@huawei.com)

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

3GPP SA thanks GSMA 5GPKIWP for the incoming LS to 3GPP SA2, SA3 and SA5 on Monitoring of Encrypted 5GS Signalling Traffic.

3GPP SA coordinated the views across SA2, SA3 and SA5 on the potential GSMA 5GPKIWP solutions aimed at addressing mobile operators' implementation issues with the monitoring of their 5GS signalling traffic and provide our consolidated view below.

SA agrees with GSMA 5GPKIWP evaluation of Solution 2 for its operational and security merits.

The solution 2 identified by GSMA 5GPKIWP is known in 3GPP as Subscriber and Equipment Trace (see 3GPP TSs 32.421, 32.422 and 32.423) falling under the scope of 3GPP SA5. The Subscriber and Equipment Trace was originally introduced in 3GPP Rel-6 and has been actively maintained and evolved since. Starting from 3GPP Rel-10 the Trace functionality is re-used for reporting of per-UE measurements (collected either at the UE and/or at the BTS) currently known as Minimization of Drive Testing (MDT). In 3GPP Rel-15, with the introduction of Service Based Management Architecture (SBMA), Trace has been extended with the real-time high volume streaming reporting (see 3GPP TSs 28.532 and 32.423). As a high-level overview, the 5G Trace relies on Network Functions collecting a copy of the signalling traffic matching the trace job activation criteria (e.g. all subscribers or only certain identities, all or certain interfaces, all or certain Network Functions), the collected data is streamed to the Management Service (MnS) consumer via TLS protected WebSocket with ASN.1 or GPB serialization. Subscriber and Equipment Trace applies to both virtualization and non-virtualization scenarios. A known implementation of 3GPP Trace MnS consumer in ONAP DCAE receives the streaming Trace data from a WebSocket and publishes it on an internal Kafka bus for multiple applications interested in said data.  The SBMA specifications do not restrict what entities can play the role(s) of MnS consumer - any authorized entity (e.g. "GSMA monitoring system") can play said role.

3GPP would also like to inform GSMA 5GPKIWP about a few key evolutions of 3GPP charging framework from 3GPP Rel-15 (see TS 32.290) relevant to solution 4: in 5GS, a dedicated NF (namely the CHF) is responsible for generating the Charging Data records (CDRs) associated with active elements (e.g. SMF) and their services, i.e. CDRs generation by NF is no longer applicable. This is achieved by CHF interacting with NFs using SBI, in addition to the control of service(s) usage in real-time (via quota management). Although the produced CHF CDRs are designed for “post-processing”, they include details captured during the service(s) usage control (and not only at service or call release), and could be used as a complement to solution 2.

# 2	Actions

To GSMA

ACTION: 	3GPP SA kindly asks GSMA 5GPKIWP to take into account the information above.

# 3	Dates of next TSG SA meetings

SA#103			19 - 22 March 2024	Maastricht, NL

SA#104			18 - 21 June 2024	China (TBC)