3GPP TSG-SA WG6 Meeting #58	S6-234068

Chicago, USA, 13th – 17th November 2023	(revision of S6-233680, 233199, 233965)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.436 | CR | 0004 | rev | 4 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Extend the edge performance analytics to support transmission quality analytics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | ADAES |  |  |  |  |  | Date: |  |  | 2023-10-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As required by the discovery of SEALDD server and VAL server in clause 9.4.3.2.3 of TS 23.433: NOTE: It is assumed that the ADAE sever can collect load and transmission quality information from EAS. Regarding of how to support transmission qualitity analytics, SEALDD server can act as the data producers, providing the measured transmission quality results to the ADAE server, which can be used for transmission quality analytics by ADAE server. As specified in clause 9.7.2.2 of TS 23.433, SEALDD server exposes data transmission query service to support the other consumers to receive the measured transmission results. Therefore, ADAE server can act as the consumer to retrieve the transmission quality data by using the data transmission quality query message. By this clarification, it is can be avoided to define the duplicated message in ADAE server to collect the transmission quality data from SEALDD server. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This paper is proposed to make the transmission quality analytics in ADAE specification more clear, and further clarifies how the ADAE server collects the transmission quality data from SEALDD server. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | How to provide the transmision quality analytics is not clear. There is misalignment between transmission quality data collection and transmission quality query in TS 23.436 and TS 23.433. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.8.2, 8.2.2, 2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision after SA6#57 based on the agreed CR S6-233199 hilighted in green. |  |  |  |  |  |  |  |  |

* * * First Change * * * *

8.2.2	Procedure on VAL server performance analytics

Figure 8.2.2-1 illustrates the procedure where the VAL server performance analytics are performed based on data collected from the ongoing VAL sessions as well as data from the DN (VAL server, DN database or networking stack at the DN).

Pre-conditions:

1.	ADAE Client (ADAEC) is connected to ADAES.

2.	Data producers (e.g. A-ADRF, VAL Client) may be pre-configured with data producer profiles for the data they can provide. ADAES and ADAEC have discovered available data producers and their data producer profiles.

Figure 8.2.2-1: ADAES support for VAL server performance analytics

1.	The consumer of the ADAES analytics service sends a VAL performance analytics subscription request to ADAES.

2.	The ADAES sends a subscription response as a positive or negative acknowledgement to the consumer of the analytics service.

3.	The ADAES maps the analytics event ID to a list of data collection event identifiers, and a list of data producer IDs. Such mapping may be preconfigured by OAM or may be determined by ADAES based on the analytics event type / vertical type and/or data producer profile.

4.	The ADAES sends a data collection subscription request to the Data Producers (at the DN side or UE side) with the respective Data Collection Event ID and the requirement for data collection. Such data producers include the A-ADRF, the A-DCCF, the VAL server, SEALDD server, or the VAL UEs.

5.	The Data Producer(s) sends a subscription response as a positive or negative acknowledgement to the ADAES.

NOTE:	The ADAES acting as AF may also subscribe to NEF/SMF/PCF/NWDAF to monitor network/UE situation or network data analytics required for the application data analytics event.

6.	The ADAES based on subscription, may receive offline stats/data from A-ADRF on the VAL server performance based on the analytics/data collection event ID. Such offline data can be average/peak throughput, average/maximum e2e delay, jitter, av. PER, availability, VAL server load, number of failed transactions, and can be for a given area and time of the day (based on the time/area of the request).

A session starts between the VAL server #1 and a UE (this could happen for more than one UEs)

7.	The Data Producer at DN side, starts collecting data from the data generating entities, e.g. real-time networking or application data (from networking start at DN or VAL server itself), such as RTT, PER, throughput.

8a.	The Data Producer sends the real-time data to the ADAES, where the data correspond to the data collection ID or the analytics event ID for which the ADAES subscribed.

8b.	The ADAES may receive also data (periodically or if a threshold is reached based on configuration) from the application of the UE within the ongoing session (via ADAEC). Such data can be about the RTT, average/peak throughput, jitter, QoE measurements (MOS, stalling events, stalling ratios, etc), QoS profile load, VAL server load, etc.

9.	When the VAL UE session with VAL server finishes, the ADAEC notifies the ADAEC of the completion of the reporting.

10.	The ADAES abstracts or correlates the data based on the analytics event and the data collection configuration. Such correlation can be filtering of data for the same metrics but with different granularities or be combining/aggregating the data of segments of the end-to-end path (end to end is between VAL client and server). The outcome is an abstracted/correlated/filtered set of data.

11.	The ADAES derives application layer analytics on VAL server #1 performance, based on the analytics ID and type of request. Such analytics can be stats or prediction for a given area/time and based on the event type for a given network configuration.

12.	The ADAES sends the analytics to the consumer, where these analytics include the VAL server 1 predicted or statistic performance for a given area and time horizon, including also the confidence level, whether offline/online analytics were used.

NOTE:	If the Data Producer in steps 4-5 and 8a is SEALDD server, procedure in clause 9.7.2.1 of 3GPP TS 23.433 [x]) is used for the collection of the E2E transmission quality measurement results to ADAES.

* * * Next Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]		3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows".

[3]	3GPP TS 26.531: "Data Collection and Reporting; General Description and Architecture"

[4]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[5]	3GPP TS 28.104: "Management and orchestration; Management Data Analytics".

[6]		3GPP TS 23.435: "Procedures for Network Slice Capability Exposure for Application Layer Enablement Service".

[7]		3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[8]		3GPP TS 23.222: "Common API Framework for 3GPP Northbound APIs".

[9]	3GPP TS 23.501: “System architecture for the 5G System”.

[10]		GSMA NG.116 - Generic Network Slice Template.

[11]	3GPP TS 22.261: “Service requirements for the 5G system”.

[12]		3GPP TS 28.545: "Management and orchestration; Fault Supervision (FS)".

[x]		3GPP TS 23.433: "Service Enabler Architecture Layer for Verticals (SEAL); Data Delivery enabler for vertical applications".