3GPP TSG-SA WG6 Meeting #57	S6-233053

Xiamen, China, 09th – 13th October, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.436 | CR | 0011 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to Procedure for supporting edge load analytics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | ADAES |  |  |  |  |  | Date: |  |  | 2023-09-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Errors exist in the procedure for supporting edge load analytics. This CR proposes to correct the errors. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The changes include: - Corrected the errors in clauses 8.8.1, 8.8.2.2, 8.8.3.2, and 8.8.3.5. - Removed the parameter “Analytics filter information” in Table 8.8.3.2-1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistancy in stage 2 specification of ADAES services. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.8.1, 8.8.2.2, 8.8.3.2, 8.8.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| () |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Additional discussion(if needed):

# Proposed changes:

* * * First Change * * * *

### 8.8.1	General

This clause describes two procedures (covering both subscribe-notify and request-response models in 8.8.2.1 and 8.8.2.2 respectively) for supporting edge load analytics, where the edge analytics are performed based on data collected from the EDN (EAS and/or EES) and A-ADRF.

* * * Next change * * * *

#### 8.8.2.2	Request-response model

Figure 8.8.2.2-1 illustrates the procedure for the analytics consumer to request analytics data of the application server(s) from the ADAE server.

Figure 8.8.2.2-1: ADAES support for edge analytics

1.	The analytics consumer sends a request message to the ADAE server to receive analytics data for one or more application servers. The request message includes the identity of the analytics consumer, security credential(s) for authorization and verification, identity of all the application server for which analytics data is requested, type of analytics data, time duration since when analytics data is required.

2.	Upon receiving the request, the ADAE server authenticates and authorizes the analytics consumer. If the analytics consumer is authorized, the ADAE server may get the analytics data by performing step 3 to 10 of clause 8.8.2.1. The ADAE server sends a response message including the statistical and predictive analytics data of the application servers for the requested duration period (if the time duration is available).

* * * Next change * * * *

#### 8.8.3.2	Edge analytics subscription request

Table 8.8.3.2-1 describes information elements for the edge analytics subscription request from the VAL server / Consumer to the ADAE server.

Table 8.8.3.2-1: Edge analytics subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Analytics Consumer ID | M | The identifier of the analytics consumer (VAL server, EAS). |
| Analytics ID | M | The identifier of the analytics event. This ID can be for example “edge performance analytics”. |
| Analytics type | M | Whether analytics event is about prediction or statistics. |
| Destination EAS ID | O (NOTE) | This identifier shows the destination EAS for which the analytics subscription applies. |
| Destination EES ID | O (NOTE) | This identifier shows the destination EES for which the analytics subscription applies. |
| DNN/DNAI | O (NOTE) | DNN or DNAIs information for which the subscription applies. |
| Target data producer profile criteria | O | Characteristics of the data producers to be used. |
| Preferred confidence level | O | The level of accuracy for the analytics service (in case of prediction). |
| Area of Interest | O | The geographical or service area for which the subscription request applies. |
| Time validity | O | The time validity of the subscription request. |
| NOTE: At least one of these shall be present. |  |  |

* * * Next change * * * *

#### 8.8.3.5	Edge data collection subscription response

Table 8.8.3.5-1 describes information elements for the Data collection subscription response from the edge Data Producer at the EDN or the A-DCCF to the ADAE server, or from ADAE client to ADAE server.

Table 8.8.3.5-1: Data collection subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The result of the edge data collection subscription request (positive or negative acknowledgement). |

* * * End of changes * * * *