3GPP TSG-SA WG6 Meeting #58	S6-233822

Chicago, USA, 13th – 17th November 2023	(revision of S6-233679)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.436 | CR | 0014 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification and correction for edge load analytics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | ADAES |  |  |  |  |  | Date: |  |  | 2023-10-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Accroding to the EAS ID definition in TS 23.558: “The EASID is a globally unique identifier which identifies a particular application for e.g. SA6Video, SA6Game etc. All EAS instances (e.g. of SA6Video application) will share the same EASID.” The EASID is the appplication identifier, and cannot identify the specific edge server. Therefore, regarding of the edge load analytics, the “Destination EAS address” shoud be added to support the edge load analytics for the specific EAS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This paper is proposed to add “Destination EAS address” to support the edge load analytics for the specific EAS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The current edge load analytics procedure is incomplete and unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.8.3.2, 8.8.3.4, 8.8.3.6, 8.8.3.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

8.8.3.2	Edge analytics subscription request

Table 8.8.3.2-1 describes information elements for the edge analytics subscription request from the VAL server / Consumer to the ADAE server.

Table 8.8.3.2-1: Edge analytics subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Analytics Consumer ID | M | The identifier of the analytics consumer (VAL server, EAS,..) |
| Analytics ID | M | The identifier of the analytics event. This ID can be for example “edge performance analytics”. |
| Analytics filter information | M | Filter information for the analytics event |
| Analytics type | M | Whether analytics event is about prediction or statistics |
| Destination EAS information | O (NOTE) | This identifier shows the destination EAS information including destination EAS ID and destination EAS address for which the analytics subscription applies. |
| Destination EES ID | O (NOTE) | This identifier shows the destination EES for which the analytics subscription applies. |
| DNN/DNAI | O (NOTE) | DNN or DNAIs information for which the subscription applies. |
| Target data producer profile criteria | O | Characteristics of the data producers to be used. |
| Preferred confidence level | O | The level of accuracy for the analytics service (in case of prediction. |
| Area of Interest | O | The geographical or service area for which the subscription request applies. |
| Time validity | O | The time validity of the request. |
| NOTE: At least one of these shall be present |  |  |

* * * Next Change * * * *

8.8.3.4	Edge data collection subscription request

Table 8.8.3.4-1 describes information elements for the edge data collection subscription request from the ADAE server to the Data Producer at the EDN or the A-DCCF, or from ADAE server to ADAE client.

Table 8.8.3.4-1: Data collection subscription request

| Information element | Status | Description |
| --- | --- | --- |
| ADAE server ID | M | The identifier of the ADAE server |
| Data Collection Event ID | M | The identifier of the data collection event |
| Data Collection requirements | M | The requirements for data collection, including the format of data, frequency of reporting, level of abstraction of data, level of accuracy of data. |
| Analytics ID | O | The identifier of the analytics event, for which the data collection is needed. |
| List of Data Producer IDs | O | In case when this request is performed via A-DCCF, then the list of Data Producer IDs is needed. |
| Destination EAS information | O (NOTE) | This identifier shows the destination EAS information including destination EAS ID and destination EAS address for which the analytics subscription applies. |
| Destination EES ID | O (NOTE) | This identifier shows the target EES for which the analytics subscription applies. |
| DNN/DNAI | O (NOTE) | DNN or DNAIs information for which the subscription applies. |
| Target data producer profile criteria | O | Characteristics of the data producers to be used. |
| Area of Interest | O | The geographical or service area for which the requirement request applies. |
| Time validity | O | The time validity of the request. |
| NOTE: At least one of these shall be present. |  |  |

* * * Next Change * * * *

8.8.3.6	Data Notification

Table 8.8.3.6-1 describes information elements for the Data Notification from the Data Producer to the ADAE server.

Table 8.8.3.6-1: Data notification

| Information element | Status | Description |
| --- | --- | --- |
| Data Collection Event ID | M | The identifier of the data collection event. |
| Data Producer ID | M | The identity of Data Producer. |
| Destination EAS information | O (NOTE) | This identifier shows the destination EAS information including destination EAS ID and destination EAS address for which the analytics subscription applies. |
| Destination EES ID | O (NOTE) | This identifier shows the destination EES for which the analytics subscription applies. |
| DNN/DNAI | O (NOTE) | DNN or DNAIs information for which the subscription applies. |
| Analytics ID | O | The identifier of the analytics event. |
| Data Type | M | The type of reported data samples which can be network data, application data, edge data, or different granularities / abstraction of data (e.g. real time, non-real time). This also indicates whether data are offline (from A-ADRF or not). |
| Data Output | M | The reported data, which can be inform of measurements or offline/historical data on the requested parameter based on subscription. Such data can be per EDN or per DNAI or per EAS/EES load statistics and edge computational resource utilization stats for a given time and area of interest. |
| NOTE: At least one of these shall be present based on the data collection event. |  |  |

* * * Next Change * * * *

8.8.3.8	Get analytics data request

Table 8.8.3.8-1 describes information elements for the Get analytics data request from the analytics consumer to the ADAE server.

Table 8.8.3.8-1: Get analytics data request

| Information element | Status | Description |
| --- | --- | --- |
| Analytics Consumer ID | M | The identifier of the analytics consumer (VAL server, EAS, EES). |
| Analytics ID | M | The identifier of the analytics event. This ID can be for example “edge performance analytics”. |
| Analytics filter information | M | Filter information for the analytics event |
| Analytics type | M | Whether analytics event is about prediction or statistics. |
| Destination EASs information | O (NOTE) | This identifier provides the list of destination EASs information including destination EAS ID and destination EAS address for which the analytics request applies. |
| Destination EES IDs | O (NOTE) | This identifier provides the list of destination EESs for which the analytics request applies. |
| Preferred confidence level | O | The level of accuracy for the analytics service (in case of prediction). |
| Time duration | O | Time duration since when analytics data is required. |
| NOTE: At least one of these shall be present |  |  |