3GPP TSG-SA WG6 Meeting #57	S6-233305

Xiamen, China 9th – 13th October 2023	(revision of S6-232924)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0496 | rev | 1 | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add missing EES endpoint in ECS-ER interaction |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, InterDigital |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph2 |  |  |  |  |  | Date: |  |  | 2023-09-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The associated EES info of the common EAS needs to be returned to the consumer EES so that EEC can connect to the EES serving the common EAS for any service consumption (e.g. ACR event subscription). Note: In EAS discovery response, EES endpoint is already added in Table 8.5.3.3-1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add EES endpoint in EAS info store request and response. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Mis-aligned procedures in the TS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.20.2.3, 8.20.3.4, 8.20.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR … CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This’CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 8.20.2.3	Common EAS information storage

Pre-condition:

#### 1.	The EES is registered in the ECS supporting repository function.

Figure 8.20.2.3-1: Common EAS information storage

1.	The EES sends Common EAS information store request message to the ECS-ER. The request message includes EDN information, EES ID, EAS ID, EAS endpoint and Application Group ID.

2.	Upon receiving the request, the ECS-ER checks if there is any stored EAS serving the application group for the EAS ID within the same EDN. If no common EAS is identified, the ECS-ER stores the received information for subsequent binding requests.

3.	The ECS-ER responds the EES with Common EAS information store response message. The ECS-ER may reject the request with common EAS information identified in step 2 or accept the request. For enabling the EEC to further communicate with common EES, the common EES endpoint is also provided with common EAS information.

* * * Next Change * * * *

#### 8.20.3.5	Common EAS information store response

Table 8.20.3.5-1 describes the information elements for common EAS information store response from the ECS-ER to the EES.

Table 8.20.3.5-1: Common EAS information store response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response (NOTE) | O | Indicates that the request was successful. |
| Failure response (NOTE) | O | Indicates that the request has failed. |
| > Cause | O | Indicates the failure cause. Only included when Failure response is included. |
| > Common EAS information | O | This IE includes common EAS endpoint. |
| > Common EES endpoint | O | This IE includes common EES endpoint address. It shall be provided when common EAS information is provided. |
| NOTE: One of these IEs shall be present in the message. |  |  |

* * * END of Change * * * *