3GPP TSG-SA WG6 Meeting #58	S6-233921

Chicago, USA, 13th – 17th November 2023	(revision of S6-233881)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0250 | rev | 2 | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to Location-based group creation request Information flow |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung, Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eSEAL |  |  |  |  |  | Date: |  |  | 2023-11-07 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Location based group creation can be performed by VAL server also. Identify of requesting VAL server is missing in the IE description. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR adds the VAL server identity as part of the requester identity Rev1 : Changes to clause 10.3.7 from S6-233869 is merged |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.3.2.34, 10.3.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 10.3.2.34	Location-based group creation request

Table 10.3.2.34-1 describes the information flow location-based group creation request from the group management client or VAL server to the group management server.

Table 10.3.2.34-1: location-based group creation request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the group management client or VAL server performing the request. |
| Location criteria | M | Criteria to combine Users or UEs in a location. |
| VAL service ID list (see NOTE 1) | O | List of VAL services whose service communications are to be enabled on the group. |
| VAL service specific information (see NOTE 2) | O | Placeholder for VAL service specific information |
| NOTE 1: This information element shall be included in the message for creating a group configured for multiple VAL services. NOTE 2: The details of this information element are specified in VAL service specific specification and are out of scope of the present document. |  |  |

* * * * Next Change * * * *

### 10.3.7	Location-based group creation

Figure 10.3.7-1 below illustrates the location-based group creation.

Pre-conditions:

1.	The group management client, group management server, VAL server, location management server and the VAL group members belong to the same VAL system.

2.	The authorized VAL user/UE/administrator or VAL server is not aware of the users' or UE identities which will be combined to form the VAL group.

Figure 10.3.7-1: Location-based group creation

1.	The group management client or the VAL server requests location-based group create operation to the group management server. The location criteria for determining the identities of the users or UEs to be combined shall be included in this message and the location criteria may be the VAL service area identified by the VAL service area ID.

2.	The group management server requests the location management server for obtaining the users or UEs corresponding to the location information or the VAL service area identified by the VAL service area ID.

3.	The location management server composes the list of users or UEs within the requested location.

4.	The group management server receives the composed list of users or UEs from the location management server. The location management server may include VAL service IDs associated with the list of users or UEs.

5.	During the group creation, the group management server creates and stores the information of the location-based group. The group management server performs the check on the policies e.g. maximum limit of the total number of VAL group members for the VAL group(s). If an external group identifier, identifying the member UEs of the VAL group at the 3GPP core network is available, then the external group ID is stored in the newly created VAL group's configuration information. If VAL service ID list was received in step 1 as part of group formation criteria and not in step 4 for the composed list of users or UEs, the group management server shall retrieve the VAL service data from the configuration management server as described in clause 11.3.2.13 and form the group with relevant users or UEs.

NOTE:	The exact policies are out of scope of the present document.

6.	The group management server provides a location-based group creation response to the group management client or the VAL server.

* * * End Change * * * *