3GPP TSG-SA WG6 Meeting #57	S6-233167

Xiamen, China 09th – 23th October 2023	(revision of S6-233143 of S6-232866)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.280 | CR | 0429 | rev | 2 | Current version: | 19.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of functional entities for supporting MCShAC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | BDBOS, Netherlands Police |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCShAC |  |  |  |  |  | Date: |  |  | 2023-10-02 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Sharing administrative configuration between interconnected MC systems require enhancements/changes on the functional model for supporting this mechanism. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR provides enhancement to current on-network functional model (clause 7.3.1) and add new functional entities (clause 7.4) and new reference points (clause 7.5) for supporting sharing administrative configuration between interconnected MC systems. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Sharing administrative configuration functionality would not be supported by MC system. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.1, 7.4.2.2.X (new), 7.4.2.2.Y(new), 7.5.2.22, 7.5.2.X (new), 7.5.2.Y (new) and 7.5.2.Z (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 7.3.1	On-network functional model

Each MC service can be represented by an application plane functional model. The functional model across MC services may be similar but is described by the individual functional entities and reference points that belong to that MC service. Within the application plane for an MC service there is a common set of functions and reference points. The common set is shared across services. This common set of functions and reference points is known as the common services core.

Figure 7.3.1-1 shows the functional model for the application plane for an MC system.

Figure 7.3.1-1: Functional model for application plane for an MC system

The common services core functions and reference points shown in figure 7.3.1-1 are shared across each MC service. The description of the functions and reference points specific to an MC service is contained in the corresponding MC service TS.

NOTE 1:  The ACM client and ACM server are not essential entities to enable MC services. The ACM client and ACM server are optional and only required if the administrative configuration management feature is deployed. In administrative configuration management case, only the UE of an authorized MC service user requires such an ACM client.

In the model shown in figure 7.3.1-1, the following apply:

-	A specific MC service server is an instantiation of a GCS AS in accordance with 3GPP TS 23.468 [18].

-	The functional alias management client is an integrated functional entity of the configuration management client. The functional alias management client is described in subclause 7.4.2.2.12.

-	The functional alias management server is an integrated functional entity of the configuration management server. The functional alias management server is described in subclause 7.4.2.2.13.

Figure 7.3.1-2 shows the functional model for the signalling control plane.

Figure 7.3.1-2: Functional model for signalling control plane

Figure 7.3.1-3 shows the relationships between the reference points of the application plane of an MC service server and the signalling plane.

Figure 7.3.1-3: Relationships between reference points of MC service application plane and signalling control planes

NOTE 2:	Application plane reference point CSC-7 makes use of SIP-2 reference point when the group management servers are connected by a single SIP core. Where they are joined by more than one SIP core, CSC-7 also makes use of the SIP-3 reference point.

NOTE 3:	For simplicity, the HTTP proxy, which provides the interconnection between HTTP-1, HTTP-2 and HTTP-3 reference points, is not shown in figure 7.3.1-3.

NOTE 4:	CSC-5, CSC-9, and CSC-15 make use of SIP-1 and SIP-2 reference points. For simplicity, this mapping relationship is not shown in figure 7.3.1-3.

* * * Second Change * * * *

##### 7.4.2.2.X	ACM client

The ACM client is a functional entity that acts as the application user agent for an authorized user that is responsible for requesting, approving, and applying configurations between a primary MC system and partner MC system. These configurations include MC service user profile, group management, and associated system parameters. The administrative configuration management client interacts with the administrative configuration management server.

The ACM client functional entity is supported by the signalling user agent and by the HTTP client functional entities of the signalling control plane.

##### 7.4.2.2.Y	ACM server

The ACM server is a functional entity that provides the necessary capabilities to control and perform required transactions to exchange administrative configurations and/or information between partner MC system(s). ACM server has the following functions:

-	Provides the temporary storage of configuration changes when requests are received from a partner MC system. The configuration changes include MC service user profile, group management configuration, and associated system parameters.

-	Supports requests from the ACM client that result in exchanges with the partner MC system to share administrative configurations and information.

-	Controls, processes, validates, accepts, forwards or rejects administrative configuration exchange based on a set of pre-defined rules and policies.

The administrative configuration management server functional entity is supported by the SIP AS and by the HTTP server functional entities of the signalling control plane.

* * * Third Change * * * *

#### 7.5.2.22	Reference point CSC-21 (between MC gateway servers in different MC systems)

The CSC-21 reference point, which exists between MC gateway servers in different MC systems in different security domains, is used to share subscription and notification related signalling for group configuration, user configuration management, administrative configuration management and location management to permit interconnection and migration between MC systems.

The CSC-21 reference point uses the SIP-3 reference point for transport and routing of subscription/notification related signalling.

#### 7.5.2.X	Reference point ACM-1 (between administrative configuration management  client and administrative configuration management server)

The ACM-1 reference point, which exists between the admininstrative configuration management client and the administrative configuration management server, provides exchange of administrative configuration data between interconnected MC systems while the administrative configuration management client is on-network.

The ACM-1 reference point supports:

- retrieving the administrative management configuration client user configuration data for authorization

- sending requests for exchanging administrative configuration data with an partner MC system

- receiving responses from partner MC system upon requests sent

- receiving requests of administrative configuration management client of partner MC system for exchanging administrative configuration data

- decision taking on how to handle received requests and to reply with status response to the requestor of the partner MC system

The ACM-1 reference point shall use the HTTP-1 and HTTP-2 reference points for transport and routing of non-subscription/notification related signalling. The ACM-1 reference point shall use SIP-1 and SIP-2 reference points for transport and routing of subscription/notification related signalling.

#### 7.5.2.Y	Reference point ACM-2 (between ACM server of primary MC system and ACM server of partner MC system)

The ACM-2 reference point, which exists between the admininstrative configuration management servers in different security domains, is used by administrative configuration management servers to exchange administrative configuration data between interconnected MC systems.

The ACM-2 reference point uses the HTTP-2 and HTTP-3 reference points for transport and routing of non-subscription/notification related signalling. The ACM-2 reference point uses SIP-2 and SIP-3 reference points for transport and routing of subscription/notification related signalling.

#### 7.5.2.Z	Reference point ACM-3 (between ACM server and MC gateway server)

The ACM-3 reference point, which exists between administrative configuration management server and MC gateway server is used for signalling related to administrative configuration management that is shared to provide preconditions for the cooperation between interconnected MC systems.

The ACM-3 reference point uses SIP-2 and SIP-3 reference points for transport and routing of signalling. The SIP-3 reference point is used when an administrative configuration management server and a MC gateway server are served by different SIP cores.

* * * End of Change * * * *