3GPP TSG-SA WG6 Meeting #58	S6-233882

Chicago, USA, 13th – 17th November 2023	(revision of S6-233533)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0251 | rev | 1 | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to the Information flows and APIs for Group Management |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung, Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | SEAL |  |  |  |  |  | Date: |  |  | 2023-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Some of the descriptions related to the information flows and information elements are in-correct and it may lead to confusion for the readers especially for stage-3. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR corrects the information flows and information elements Rev1 : Changes from S6-233869 (Clause 10.3.6.2, 10.3.2.18) are merged. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.3.2.1, 10.3.2.2, 10.3.2.4, 10.3.2.5, 10.3.2.6, 10.3.2.7,10.3.2.18, 10.3.2.19, 10.3.2.20, 10.3.2.21, 10.3.2.8, 10.3.6.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 10.3.2.1	Group creation request

Table 10.3.2.1-1 describes the information flow group creation request from the group management client or VAL server to the group management server.

Table 10.3.2.1-1: Group creation request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the group management client or VAL server performing the request. |
| Identity list | M | List of VAL user IDs or VAL UE IDs that are part of the group to be created corresponding to the list of the configured services |
| VAL service ID list (see NOTE 1) | O | List of VAL services whose service communications are to be enabled on the group. |
| VAL service specific information (NOTE 2) | O | Placeholder for VAL service specific information |
| NOTE 1: This information element shall be included in the message for creating a group configured for multiple VAL services. NOTE 2: The details of this information element are specified in VAL service specific specification and are out of scope of the present document. |  |  |

#### 10.3.2.2	Group creation response

Table 10.3.2.2-1 describes the information flow group creation response from the group management server to the group management client or VAL server.

Table 10.3.2.2-1: Group creation response

| Information element | Status | Description |
| --- | --- | --- |
| VAL group ID | M (see NOTE) | VAL group ID of the group |
| Result | M | Indicates the success or failure for the operation |
| NOTE: If the Result information element indicates failure then the value of VAL group ID information element has no meaning. |  |  |

* * * Next Change * * * *

#### 10.3.2.4	Group information query request

Table 10.3.2.4-1 describes the information group information query request from group management client or VAL server to group management server.

Table 10.3.2.4-1: Group information query request

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | The identity of the VAL server, VAL user or VAL UE performing the query. |
| VAL group ID | M | The identity of the VAL group to be queried. |
| Query type | M | It indicates the query type, i.e., membership information. |

#### 10.3.2.5	Group information query response

Table 10.3.2.5-1 describes the information flow group information query response from group management server to group management client or VAL server.

Table 10.3.2.5-1: Group information query response

| Information element | Status | Description |
| --- | --- | --- |
| VAL group ID | M (see NOTE) | The identity of the VAL group to be queried. |
| Query type | M (see NOTE) | It indicates the query type, e.g. membership information. |
| Query result | M (see NOTE) | The group information retrieved from the group management server based on the query type, i.e., a list of group members. |
| Result | M | Indicates the success or failure for the operation |
| NOTE: If the Result information element indicates failure then the values of other information elements have no meaning. |  |  |

* * * Next Change * * * *

#### 10.3.2.6	Group membership update request

Table 10.3.2.6-1 describes the information flow group membership update request from the group management client or VAL server to the group management server.

Table 10.3.2.6-1: Group membership update request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the group management client or VAL server performing the request. |
| VAL group ID | M | Identity of the VAL group |
| Identity | M | List of identities of the VAL users and VAL UEs affected by this operation |
| Operations | M | Add to or delete from the group |

#### 10.3.2.7	Group membership update response

Table 10.3.2.7-1 describes the information flow group membership update response from the group management server to the group management client or VAL server.

Table 10.3.2.7-1: Group membership update response

| Information element | Status | Description |
| --- | --- | --- |
| VAL group ID | M | Identity of the VAL group |
| Result | M | Indicates the success or failure for the operation |

* * * Next Change * * * *

#### 10.3.2.18	Store group configuration request

Table 10.3.2.18-1 describes the information flow store group configuration request from the group management client or VAL server to the group management server.

Table 10.3.2.18-1: Store group configuration request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the group management client or VAL server performing the request. |
| VAL group ID | M | VAL group ID of the group |
| VAL group configuration data | M | VAL group configuration data |

#### 10.3.2.19	Store group configuration response

Table 10.3.2.19-1 describes the information flow store group configuration response from the group management server to the group management client or VAL server.

Table 10.3.2.19-1: Store group configuration response

| Information element | Status | Description |
| --- | --- | --- |
| VAL group ID | M | VAL group ID of the group |
| Result | M | Indicates the success or failure for the result |

* * * Next Change * * * *

#### 10.3.2.20	Get group configuration request

Table 10.3.2.20-1 describes the information flow get group configuration request from the group management client or VAL server to the group management server.

Table 10.3.2.20-1: Get group configuration request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the group management client or VAL server performing the request. |
| VAL group ID | M | VAL group ID of the group |
| VAL group information reference | M | Reference to configuration data for the VAL group |
| VAL services requested (see NOTE) | O | Service(s) for which group configuration is requested |
| NOTE: If 'VAL services requested' is not present, group configuration is requested for all services defined for the VAL group |  |  |

#### 10.3.2.21	Get group configuration response

Table 10.3.2.21-1 describes the information flow get configuration response from the group management server to the group management client or VAL server.

Table 10.3.2.21: Get group configuration response

| Information element | Status | Description |
| --- | --- | --- |
| VAL group ID | M | VAL group ID of the group |
| VAL group configuration data | M (see NOTE) | VAL group configuration data |
| Result | M | Indicates the success or failure for the operation |
| NOTE: If the Result information element indicates failure then the value of the VAL group configuration data information element has no meaning. |  |  |

* * * Next Change * * * *

#### 10.3.6.2	Retrieve group configurations

The procedure for retrieve group configurations at the group management client or the VAL server is described in figure 10.3.6.2-1. This procedure can be used following service authorisation when the configuration management client has received the list of groups and the group management client needs to obtain the group configurations, or following a notification from the group management server that new group configuration information is available.

Pre-conditions:

-	The group management server has received configuration data for groups, and has stored this configuration data;

-	The VAL UE has registered for service and the group management client or the VAL server needs to download group configuration data applicable to the user/UE.

Figure 10.3.6.2-1: Retrieve group configurations

1.	The group management client or the VAL server requests the group configuration data.

2.	The group management server provides the group configuration data to the group management client or the VAL server.

3.	The group management client or the VAL server stores the group configuration information.

* * * End Change * * * *