3GPP TSG-SA WG6 Meeting #58	S6-233554

Chicago, USA, 13th – 17th November 2023	(revision of S6-23xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0257 | rev | - | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Subscription procedure for group membership changes is missing |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SA6 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Samsung |  |  |  |  |  |  |  |  |  |
| Work item code: | SEAL |  |  |  |  |  | Date: |  |  | 2023-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Group membership changes notification procedure is available in TS but the corresponding subscription procedure is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR adds the missing subscription procedure for group membership change |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect specification and misalignement with stage 3. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.3.5.0 (new), 10.3.2.14, 10.3.2.15, 10.3.2.16, 10.3.2.17, 10.4.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 10.3.5	Group membership

#### 10.3.5.0	Group membership subscription

The procedure for subscription for group membership data as described in figure 10.3.5.0-1 is used by the group management client and VAL server to indicate to the group management server that it wishes to receive updates of group membership data for groups for which it is authorized.

Pre-conditions:

1.	The group management server and VAL server serve the same VAL system;

2.	The initiator of this operation is aware of the current group membership of the VAL group;

Figure 10.3.5.0-1: Subscription for group membership

1.	The group management client or VAL server subscribes to the group membership information stored at the group management server using the group information subscribe request.

2.	The group management server provides a group information subscribe response to the group management client or VAL server indicating success or failure of the request.

* * * Next Change * * * *

#### 10.3.2.14	Group information subscribe request

Table 10.3.2.14-1 describes the information flow group information subscribe request from the group management client or VAL server to the group management server.

Table 10.3.2.14-1: Group information subscribe request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the group management client or VAL server performing the request. |
| VAL group ID | M | VAL group ID of the group |
| Minimum time between consecutive notifications | O | Defaults to 0 if not provided, otherwise indicates the minimum time interval between consecutive notifications |

#### 10.3.2.15	Group information subscribe response

Table 10.3.2.15-1 describes the information flow group information subscribe response from the group management server to the group management client or the VAL server.

Table 10.3.2.15-1: Group information subscribe response

| Information element | Status | Description |
| --- | --- | --- |
| VAL group ID | M | VAL group ID of the group |
| Result | M | Indicates success or failure of the subscribe request |

#### 10.3.2.16	Group information notify request

Table 10.3.2.16-1 describes the information flow group information notify request from the group management server to the group management client or VAL server.

Table 10.3.2.16-1: Group information notify request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the group management server performing the request. |
| VAL group ID | M | VAL group ID of the group |
| VAL group information | M | Group information for the VAL group |

#### 10.3.2.17	Group information notify response

Table 10.3.2.17-1 describes the information flow group information notify response from the group management client or VAL server to the group management server.

Table 10.3.2.17-1: Group information notify response

| Information element | Status | Description |
| --- | --- | --- |
| VAL group ID | M | VAL group ID of the group |
| Result | M | Indicates success or failure of the notification request |

* * * Next Change * * * *

#### 10.3.5.1	Group membership notification

Figure 10.3.5.1-1 illustrates the group membership notification operations to the VAL server(s) and group management clients upon the group membership change at group management server.

Pre-conditions:

1.	VAL group is created on the group management server.

2.	The group management client or VAL server has subscribed to the group configuration information.

Figure 10.3.5.1-1: group membership notification

1.	The membership of a specific VAL group is changed at group management server.

2.	The group management server notifies the VAL server(s) regarding the group membership change with the information of the updated group members.

3.	The group management server updates the group management clients of the VAL users/UEs who have been added to or removed from the group.

4.	The group management client requests to retrieve the relevant group configurations from group management server, if the user or UE is added to the group. If the user or UE is deleted from the group, the locally stored group configurations in the VAL UE may be removed.

* * * Next Change * * * *

### 10.4.5	SS_Group_Management_Event API

#### 10.4.5.1	General

API description: This API enables the VAL server to communicate with the group management server to subscribe and receive subsequent notification events over GM-S.

#### 10.4.5.2	Subscribe_ Group_Info_Modification operation

API operation name: Subscribe_ Group_Info_Modification

Description: Subscribing to changes to group membership and configuration information.

Known Consumers: VAL server.

Inputs: See subclause 10.3.2.14, 10.3.2.22

Outputs: See subclause 10.3.2.15, 10.3.2.23

See subclause 10.3.5.0 and 10.3.6.3 for the details of usage of this API operation.

#### 10.4.5.3	Notify_Group_Info_Modification operation

API operation name: Notify_Group_Info_Modification

Description: Notification for changes to group membership and configuration information.

Known Consumers: VAL server.

Inputs: See subclause 10.3.2.8

Outputs: See subclause 10.3.2.8

See subclause 10.3.5.1 and 10.3.6.3 for the details of usage of this API operation.