3GPP TSG-SA WG6 Meeting #58	S6-233912

Chicago, USA, 13th – 17th November 2023	(revision of S6-233569)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.222 | CR | 0145 | rev | 1 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Service API retrieval for CAPIF interconnection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19, eCAPIF |  |  |  |  |  | Date: |  |  | 2023-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the current version of the specification it is expected that the CCF reuses the same service API operations as the AEF. However, in the description of the API operations the CCF is not listed as a known consumer and some references to the right clauses are missing. Additionally, the retrieve service APIs for CAPIF interconnection procedure is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - CCF as known consumer of the (already available) Publish_Service_API operation - Information flow and IEs to retrieve service APIs for CAPIF interconnection procedure - Extend referencing to clauses involving CAPIF interconnection |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missalignment with Stage 3 Specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.3.2, 8.25.2.X (New), 8.25.2.Y (New), 8.25.3.Z (New), 10.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision 1 includes eCAPIF in the Work item code |  |  |  |  |  |  |  |  |

* * * * First Change * * * *

### 10.3.2	Publish_Service_API operation

API operation name: Publish_Service_API

Description: Publish the service API information.

Known Consumers: API publishing function, CAPIF core function.

Inputs: Refer subclause 8.3.2.1, 8.25.2.1.

Outputs: Refer subclause 8.3.2.2, 8.25.2.2.

See subclause 8.3.3 and 8.25.3.1 for the details of usage of this API operation.

* * * * Next Change * * * *

#### 8.25.2.X	Interconnection get service API request

Table 8.25.2.X-1 describes the information flow interconnection get service API request from a CAPIF core function to another CAPIF core function.

Table 8.25.2.X-1: Interconnection get service API request

| Information element | Status | Description |
| --- | --- | --- |
| CCF information | M | The information of the CAPIF core function that wants to retrieve service APIs information. It may include identity, authentication and authorization information |
| Service API published information reference (see NOTE) | M | The information which can be used for referencing the information (set) about the published service API by the CCF which publishes service APIs |
| NOTE: Obtained during the interconnection API publish procedure in clause 8.25.3.1. |  |  |

* * * * Next Change * * * *

#### 8.25.2.Y	Interconnection get service API response

Table 8.25.2.Y-1 describes the information flow interconnection get service API response from a CAPIF core function to another CAPIF core function.

Table 8.25.2.Y-1: Interconnection get service API response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of unpublishing the service API information |
| Service API information | O (see NOTE) | The service API information includes the service API name, service API type, communication type, description, Serving Area Information (optional), interface details (e.g. IP address, port number, URI), protocols, version numbers, and data format. |
| NOTE: Shall be present if the Result information element indicates that the interconnection get service API request is successful. Otherwise, service API information shall not be present. |  |  |

* * * * Next Change * * * *

#### 8.25.3.Z	Retrieve service APIs for CAPIF interconnection

This clause describes the procedure to retrieve service APIs information in a CAPIF interconnection scenario.

Pre-condition:

1.	CCF-A and CCF-B connect to each other, and either belong to the single trust domain of the same CAPIF provider or trust domains of different CAPIF providers. When CCF-A and CCF-B belong to trust domains of different CAPIF providers, the two CAPIF providers have business agreement for service API sharing.

2.	CCF-B is configured as the designated CAPIF core function in the trust domain of CAPIF provider A.

Figure 8.25.3.Z-1: Retrieve service APIs for CAPIF interconnection

1.	CCF-A sends an interconnection get service API request to CCF-B with service API published information reference provided by the CAPIF core function when the service API was published.

2.	Upon receiving interconnection get service API request, CCF-B checks whether CCF-A is authorized to get the published service APIs information. If the check is successful, the corresponding service API information is retrieved from the CAPIF core function (API registry).

3.	CCF-B sends a service API get response to CCF-A which includes the requested service API information if the retrieval operation was successful.

* * * * Next Change * * * *

### 10.3.5	Get_Service_API operation

API operation name: Get_Service_API

Description: Retrieve the published service API information.

Known Consumers: API publishing function, CAPIF core function.

Inputs: Refer subclause 8.12.2.1, 8.25.2.X.

Outputs: Refer subclause 8.12.2.2, 8.25.2.Y.

See subclause 8.12.3 and 8.25.3.Z for the details of usage of this API operation.

* * * * End of Changes * * * *