3GPP TSG-SA WG6 Meeting #58	S6-233864

Chicago, USA, 13th - 17th November 2023	(revision of S6-233667)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0269 | rev | 1 | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | RRC inactive support |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19, SEAL_Ph3 |  |  |  |  |  | Date: |  |  | 2023-10-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In Rel-18, the UE is able to receive the multicast MBS data in RRC-inactive state. However, in current 23.434, the description is only restricted to recive the multicast MBS data in CONNECTED state. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the description of receiving multicast MBS data in both RRC-CONNECTED and RRC-INACTIVE state. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The NRM client is not able to receive the multicast MBS data in RRC-inactive state. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 14.3.4A.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# /******************** First Changes ********************/

14.3.4A.1	General

This subclause defines information flows and procedures for 5G MBS usage that applies to VAL services. 5G MBS session can be used by any VAL service for any VAL service group.

The main purpose of using 5G Multicast-Broadcast Service (MBS) by verticals is to provide efficient downlink delivery of user traffic in VAL service group communications or in a certain area.

Multicast and broadcast communication services in 5G for vertical applicationsrely on the creation and establishment of MBS sessions to deliver user data in downlink. Shared and individual delivery from the VAL server to multiple VAL users is supported either as point-to-point or point-to-multipoint over the radio. The MBS session which consist of one or multiple QoS flows for different service requirements are either broadcast or multicast type. For the broadcast MBS session or local MBS session, the MBS service area is configured with the MBS session.

NOTE 1:	Support of MBS and specific session types is an implementation choice.

Within this arrangement, the VAL server decides whether to create broadcast or multicast MBS sessions to be associated with certain VAL service groups or area. The 5GC adaptively decides whether to deliver the MBS traffic from the MB-UPF in the form of shared delivery or individual delivery, where the latter is applicable to multicast MBS sessions. The NG-RAN decides to utilize point-to-point or point-to-multipoint delivery methods applicable for shared delivery only. MBS provides reliability enhancements and minimizes loss of information, e.g., due to mobility and handover.

MBS group scheduling mechanism enables simultaneous reception of MBS and unicast user traffic by the VAL UEs. The UEs can receive broadcast MBS sessions irrespective of their RRC state (i.e., connected, inactive or idle) and multicast sessions in RRC-CONNECTED state and RRC_INACTIVE state.

The following capabilities (non-exhaustive list) provided by MBS could be used by NRM server:

-	MBS session creation;

-	MBS session update;

-	MBS session release;

-	MBS session ID allocation;

-	Dynamic PCC control for MBS session.

The first phase to utilize MBS sessions for VAL media transmission is to reserve the network resource by creating a MBS sessions. The MBS session creation is initiated by the VAL server towards the NRM server, and the NRM server further interacts with the 5GS to complete the whole process. The UE’s capabilities and service related information e.g., UE’s MBS capabilities, location, MBS listening status report, UE session join notification sent by group members are considered when deciding to create or use MBS sessions. During the interaction with NRM server, the necessary information related to the requested session is determined, e.g., MBS session mode (either a broadcast or a multicast session) and the required service profile. This interaction between the NRM server and the 5GS depends on the configuration option under consideration, i.e., whether the NRM server is in trusted domain (limited operations), and whether the session creation is done with or without a dynamic PCC rule.

NOTE 1:	It is implementation specific whether the VAL server decides to use multicast or broadcast MBS sessions.

NOTE 2:	It is implementation specific whether the VAL decides to create (one or multiple) MBS sessions for VAL media for VAL group communications associated to a certain VAL group or create (one or multiple) dynamic MBS sessions once the need has emerged, e.g., dynamic MBS sessions to be associated for an ad hoc group.

NOTE 3:	It is implementation specific whether an MBS session is associated to one or multiple VAL service groups, and whether it is re-assigned to other VAL service groups.

NOTE 4:	How the NRM server uses the UE’s capabilities and service related information in order to create and use the MBS session is implementation specific.

The information elements describing the MBS session under consideration is then sent to the NRM clients via MBS session announcement, where the latter need to react according to the announced session mode.

If eMBMS and 5G MBS co-exist for VAL services, the NRM service server may decide to trigger the establishment of an eMBMS bearer to deliver the VAL media associated to the VAL service group communications, if the target VAL service group(s) consists of members with MBMS capable RAT. As a result, the NRM server subsequently needs to send an eMBMS bearer announcement towards the clients camping on LTE.

NOTE 5:	It is implementation specific whether the NRM server triggers an eMBMS bearer or a unicast bearer to serve VAL clients camping on LTE.