3GPP TSG SA Meeting #102	SP-231766

Edinburgh, Scotland, December 11 – 15, 2023	(revision of S2-23131504)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
| 68 |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5175 | rev | 6 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification of the Unmarked PDUs with SN Offset and PSI Value |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Xiaomi |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM |  |  |  |  |  | Date: |  |  | 2023-12-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | as highlighted by LS from SA4 in S2-2310131/S4-231592, - When the UPF marked PDU Set of one or more unmarked PDUs, the UPF should offset the PDU Set Sequence Number to ensure correct sequencing at GTP-U header level. - For the PDU Set importance, same value for all unmarked lonely PDUs will mix the different category e.g. audio, metadata and RTCP PDUs. The defult importance values e.g. specified in the TS 26.522 as the general guidelines, is proposed as the PSI value determination for the alone PDUs. It is updated to capture the clarification of related handling. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to clarify the related handling of unmarked PDUs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The handling of unmarked PDUs is not correct. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.37.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev5: Clean up the changes on change. (Rev4 was raised and taken out from block approval and marked as postponed, because of the changes on change) |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

5.37.5.2	PDU Set Information and Identification

To support PDU Set based QoS handling, the PSA UPF identifies PDUs that belong to PDU Sets and determines the below PDU Set Information which it sends to the NG-RAN in the GTP-U header. The PDU Set information is used by the NG-RAN for PDU Set based QoS handling as described above.

The PDU Set Information comprises:

-	PDU Set Sequence Number.

-	Indication of End PDU of the PDU Set.

-	PDU Sequence Number within a PDU Set.

-	PDU Set Size in bytes.

-	PDU Set Importance, which identifies the relative importance of a PDU Set compared to other PDU Sets within a QoS Flow.

The NG-RAN may use the Priority Level (see clause 5.7.3.3) across QoS Flows and PDU Set Importance within a QoS Flow for PDU Set level packet discarding in presence of congestion.

NOTE 1:	In addition to considering the PDU Set Importance within a QoS Flow, NG-RAN could also consider the relative PDU Set Importance across QoS Flows of the same Priority Level when determining which PDU Set needs to be discarded, which is up to implementation and configuration of operator.

NOTE 2:	The PDU Set Information can be different for different PDU Sets within a QoS Flow.

The SMF instructs PSA UPF to perform PDU Set marking and may provide the PSA UPF the Protocol Description used by the service data flow. The Protocol Description may be received in the PCC rule, based on information provided by the AF or by PCF local policies as described in clause 5.37.5.1.

PSA UPF can identify the PDU Set Information using the Protocol Description and the received transport protocol headers and payload or using implementation specific means. The details of the RTP/SRTP headers, header extensions and/or payloads used to identify PDU Set Information are defined in TS 26.522 [179].

For each DL PDU received on N6 for which PDU Set based QoS handling is indicated from the SMF, the PSA UPF applies the rules for PDU Set identification and provides PDU Set Information which is available to the RAN in the GTP-U header.

NOTE 3:	The PSA UPF is expected to assign a unique PDU Set Sequence Number in the GTP-U header to each PDU Set of the QoS Flow.

# * * * * End of changes * * * *