3GPP TSG-WG SA2 Meeting #161	S2-2402146

26 Feb – 1 March, 2024, Greece, Athens	          (revison of S2-XXXXXXX)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0511 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update GMLC local coordinate functionality |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eLCS_Ph2 |  |  |  |  |  |  | Date: |  | 2024-02-16 |
| Category: | A |  |  |  |  |  |  | Release: |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |  |  |
| Reason for change: |  | Clause 6.1.2 step 24 defines that GMLC is capable for transform universal location co-ordinates to local geographic reference system but this is not mentioned under GMLC functionalities. If the external LCS client or AF requires it, the (H)GMLC may first transform the universal location co-ordinates provided by the AMF into some local geographic reference system. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding to GMLC the functionality of translation into some local geographic reference system. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | GMLC functionality is not aligned with procudure description. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  | TS/TR ... CR ... |  |
| affected: |  |  | x | Test specifications |  |  |  |  | TS/TR ... CR ... |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  | TS/TR ... CR ... |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of Change * * * *

### 4.3.3	Gateway Mobile Location Centre, GMLC

The Gateway Mobile Location Centre (GMLC) contains functionality required to support LCS. In one PLMN, there may be more than one GMLC.

A GMLC is the first node an external LCS client accesses in a PLMN (i.e. the Le reference point is supported by the GMLC). AFs and NFs may access GMLC directly or via NEF. The GMLC may request routing information and/or target UE privacy information from the UDM via the Nudm interface. After performing authorization of an external LCS Client or AF and verifying target UE privacy, a GMLC forwards a location request to either a serving AMF using Namf interface or to a GMLC in another PLMN using the Ngmlc interface in the case of a roaming UE.

The target UE's privacy profile settings shall always be checked in the UE's home PLMN prior to delivering a location estimate.

The "Visited GMLC" (VGMLC) is the GMLC, which is associated with the serving node of the target UE.

The "Home GMLC" (HGMLC) is the GMLC residing in the target UE's home PLMN, which is responsible for the control of privacy checking of the target UE.

Additional functions which may be performed by a GMLC to support location services include the following:

-	At an HGMLC, determine the serving AMF for a target UE when there is more than one serving AMF.

-	At an HGMLC, determine whether to attempt a second location request for a target UE from a different AMF when location information returned by a first AMF does not meet QoS requirements and there is more than one serving AMF.

-	At an HGMLC, support location requests from an external LCS client or NEF for a 5GC-MT-LR and deferred 5GC-MT-LR for periodic, triggered and UE available location events.

-	At an HGMLC, support additional check of whether the UE is located within the requested target area for deferred 5GC-MT-LR for periodic, triggered and UE available location events.

-	At an HGMLC, forward location requests for a roaming UE to a VGMLC or serving AMF in the VPLMN based on deployment configurations.

-	At an HGMLC, receive event reports from a VGMLC or LMF for a deferred 5GC-MT-LR for periodic or triggered location and return to an external LCS Client or NEF.

-	At an HGMLC, support cancelation of a periodic or triggered location.

-	At an HGMLC, receive location information from an VGMLC for a 5GC-MO-LR and forwards to an LCS Client or an AF (via NEF) if requested by the UE.

-	At a VGMLC, receive location requests from an HGMLC for a roaming UE and forward to a serving AMF.

-	At a VGMLC, receive event reports from an LMF for a deferred 5GC-MT-LR for periodic or triggered location for a roaming UE and forward to an HGMLC.

-	At a VGMLC, receive location information from an AMF for a 5GC-MO-LR and forwards to an HGMLC.

-	At an HGMLC, reject the LCS request coming from a LCS client, e.g. when the number of Target UEs in the LCS request exceeds the Maximum Target UE Number of such client.

-	At an HGMLC, allocate the reference number for each location request from an external LCS client for LDR.

-	At an HGMLC, assign the pseudonym if pseudonym indicator is received in the service request and transfer it to external LCS client, e.g. when core network provides the UE's location information to LCS client. Resolve the verinym from the pseudonym, if it is received from the LCS client.

-	At an HGMLC, resolve group identifier to identifier of individual UEs and aggregate responses to LCS Client or NEF during bulk operation procedure.

-	At an HGMLC, verify a request for user plane reporting by an LCS Client or AF for a periodic or triggered 5GC-MT-LR and verify or assign parameters to the request to enable the target UE to establish a secure user plane connection to the LCS Client or AF. Subsequently, support the transfer of cumulative event reports from the target UE via control plane back to the LCS Client or AF.

-	During UE mobility between 5GS and EPS, the GMLC transfers the LCS request between 5GS and EPS, may map the LCS QoS and (re-)initiate the LCS sessions towards the 5GS to/from EPS.

-	At an HGMLC, determine a reporting indication on how a target UE sends event reports when the UE is outside the event report allowed area based on LCS privacy profile data, LCS Client type, the type of periodic or triggered location reporting and associated parameters.

-	Support aggregation of a single UE's location estimates over a period of time for a periodic or triggered 5GC-MT-LR event.

-	At an HGMLC or a VGMLC, for a regulatory location service, support multiple location reports if requested by the LCS client.

-	At an HGMLC, transform the universal location co-ordinates provided by the AMF into some local geographic reference system.

* * * * End of Changes * * * *