3GPP TSG-SA5 Meeting #153 	S5-240860

Sevilla, Spain, 29 January - 02 February 2024							    Revision of S5-240074

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0507 | rev | 1 | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR TS 28.552 Add satellite backhaul category for performance measurements for UPF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSATB_OAM |  |  |  |  |  | Date: |  |  | 2024-01-19 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | While using satellites as the backhaul, a gNB may be subject to variable backhaul delay if the backhaul connection involves varying inter-satellite links or there may be different types of satellite backhaul. The change of satellite backhaul delay has impact to PCC/QoS control, hence, with the satellite backhaul category indication, the SMF or PCF can determine to enforce QoS monitoring to measure the packet delay between UE and PSA UPF considering the varience introduced by the satellite backhaul category. With the background information above, we proposed to clarify the measurements which can be applied to the satellite backhaul scenario. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a new use case to clarify that in the satellite backhaul scenario, when the UPF is deployed on a GEO satellite, multiple performance measurements which has already defined for UPF can be applied, |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The measurement applicable to satellite backhaul scenarios is not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change all new text

A.X	Use case of measurements on one way delay between PSA UPF and UE when 5GS supports satellite backhaul

While using satellites as the backhaul, a NG-RAN node may be subject to variable backhaul delay if the backhaul connection involves varying inter-satellite links or the backhaul connect is of the different satellite backhaul category. The change of satellite backhaul delay has impact to user plane delay performance. In case the PSA UPF and NG-RAN are time synchronised, the DL/UL delay between PSA UPF and UE can be measured at PSA UPF.

The measurements on the DL/UL delay between PSA UPF and UE can be used to evaluate the user plane delay performance in 5GS supporting satellite backhaul scenario where the UPF is deployed on a GEO satellite, if the PSA UPF and NG-RAN are time synchronised.