3GPP TSG-SA5 Meeting #153 	S5-240788

# Sevilla, Spain, 29 January - 02 February 2024                             revision of S5-240232

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.104 | CR | 0081 | rev | 1 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR 28.104 correction on MDA request and reporting workflow |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eMDAS_Ph2 |  |  |  |  |  | Date: |  |  | 2024-01-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | According to description of subscribe operation used for file data reporting in TS 28.532 and NtfSubscriptionControl as defined in TS 28.622, the subscribe request shall be sent from MnS consumer to MnS producer. The subscribe relationship extablishment procedure as described in the MDA request/reporting workflow is not correct. The MDAS producer can not subscribe for the reporting target. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the subscribe relationship extablishment procedure in the MDA request/reporting workflow. Correct the reference (from TS 28.531 to TS 28.532) usage in the MDA request/reporting workflow. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Wrong workflow may lead to implementation issue. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.2.1 and 11.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

9.3.2	MDARequest

9.3.2.1	Definition

The IOC MDARequest represents the MDA output request created by an MnS consumer.

The attribute requestedMDAOutputs contains one or multiple MDAOutputPerMDAType elements, and each MDAOutputPerMDAType element supports filtering of MDA output for a certain MDA type.

The attribute reportingTarget may contain the target address, which instructs the MDA MnS producer to create, on behalf of the MnS consumer, a subscription of MDA report.

9.3.2.2	Attributes

Table 9.3.2.2-1

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| requestedMDAOutputs | M | T | T | F | T |
| reportingMethod | M | T | T | F | T |
| reportingTarget | M | T | T | F | T |
| analyticsScope | M | T | T | F | T |
| startTime | CM | T | T | F | T |
| stopTime | CM | T | T | F | T |

2nd Change

11.1	MDA request and reporting workflow

Figure 11.1-1: Generic MDA request/reporting workflow

1	MDAS Producer creates MOI for MDARequest IOC (see createMOI operation defined in TS 28.532 [11]) for the MDAS Consumer with MDA request related information.

Note:	Void

2.	The MDAS producer subscribes to the relevant notifications or setup the streaming connections, per the selected reporting method (identified by reportingMethod attribute in the MDARequest MOI):

-	If the reportingMethod designated in the MDARequest MOI is "File":

2a.	if subscription for the reporting target (specified by the reportingTarget attribute in the MDARequest MOI) do not exist, the MDAS producer subscribes to the file data reporting related notifications  (see TS 28.532 [11]) for the reporting target;

-	If the reportingMethod designated in the MDARequest MOI is "Streaming":

2b/2c.	if the streaming connection with the reporting target does not exist, the MDAS producer invokes the establishStreamingConnection operation (see TS 28.532 [11]) to setup the streaming connection with the streaming target;

2d/2e.	if the streaming connection with the reporting target exists, the MDAS producer invokes the addStream operation (see TS 28.532 [11]) to add the stream for the expected MDA reports. And,

2f/2g.	if the newly added stream is to replace an existing one, the MDAS producer invokes the deleteStream operation (see TS 28.532 [11]) to delete the stream.

NOTE 1: 	the order of 2d/2e and 2f/2g is not significant and could be swapped too.

-	If the reportingMethod designated in the MDARequest MOI is "Notification":

2h.	if subscription for the reporting target do not exist, the MDAS producer subscribes to the provisioning related notifications (see TS 28.532 [11]) for the reporting target.

NOTE 2: 	Although, the workflow assumes that different entities are playing the role of "MDAS Consumer" and "Reporting target", it is possible and allowed to have single entity playing the role of both "MDAS Consumer" and "Reporting target".

3.	While the MDARequest is active, the MDAS Producer keeps performing MDA,  and making the MDA report (see the MDAReport IOC defined in clause 9) according to the MDARequest MOI.

3a.	the MDAS producer makes the MDA report ready and sends the MDA report to the reporting target per the selected reporting method (identified by reportingMethod attribute in the MDARequest MOI):

-	If the reportingMethod designated in the MDARequest MOI is "File":

3b.	the MDAS producer makes the MDA report into a file;

3c.	the MDAS producer emits the notifyFileReady notification (see TS 28.532 [11]) to the reporting target for the MDA report.

-	If the reportingMethod designated in the MDARequest MOI is "Streaming":

3d.	the MDAS producers makes the MDA report into a stream data unit;

3e.	invokes the reportStreamData operation (see TS 28.532 [11]) to the reporting target for the MDA report.

-	If the reportingMethod designated in the MDARequest MOI is "Notification":

3f.	the MDAS producer creates and MDAReport MOI (see clause 9) for the MDA report;

3g.	if notifyMOICreation is used, the MDAS producer emits the notifyMOICreation notification (see TS 28.532 [11]) to the reporting target for the MDA report.

3h.	if notifyMOIChanges is used, the MDAS producer emits the notifyMOIChanges notification (see TS 28.532 [11]) to the reporting target for the MDA report.

End of Change