3GPP TSG-SA5 Meeting #153 	S5-240070

Sevilla, Spain, 29 January – 02 Feburary 2024

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.316 | CR | 0001 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Fix DHCPv4 option 124 format |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17 |  |  |  |  |  | Date: |  |  | 2024-01-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The format of DHCPv4 option 124 is missing the length of Vendor Class Data field according to RFC 3925. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct DHCPv4 option 124 format. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The error in the specification may lead wrong implementation |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

# 4	Data formats for Plug and Connect

## 4.1	Client identification in DHCP requests

### 4.1.1	DHCPv4

This clause describes DHCP options for use with DHCP for IPv4 (DHCPv4) that NE uses to identify itself in DHCP requests in Plug and Connect (PnC).

The NE performing the Initial IP Autoconfiguration procedure specified in clause 5.2 of 3GPP TS 28.315 [3], using IPv4 based network stack, may identify itself using Vendor Class Identifier DHCPv4 option 60 as specified in RFC 2132 [6]. The format of Vendor Class identifier shall follow the rules specified in clause 9.13 of RFC 2132 [6], which is illustrated in table 4.1.1.1:

Table 4.1.1.1: Format of Vendor Class Identifier

| Code | Length | Vendor Class Identifier |  |  |
| --- | --- | --- | --- | --- |
| 60 | n | i1 | i2 | … |
| octet | octet | octet | octet | octet |

The NE may identify itself as Multi-Vendor Plug and Connect (MvPnC) compatible DHCPv4 client using DHCPv4 option 60 in the following way:

- 	DHCPv4 option code 60;

- 	Length 5 bytes;

- 	Vendor class identifier "MvPnC".

The use of Vendor Class Identifier DHCPv4 option 60 with specific value MvPnC is illustrated in table 4.1.1.2.

Table 4.1.1.2: Use of Vendor Class Identifier

| Code | Length | Vendor Class Identifier |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 60 | 5 | M | v | P | n | C |

The NE may alternatively identify itself by using the Vendor-Identifying Vendor Class DHCPv4 option 124 as specified RFC 3925 [4]. This option contains one or more Vendor-Identifying Vendor class information each identified by Enterprise Number as registered with IANA [7]. The format of Vendor-Identifying Vendor Class shall follow the rules specified in clause 3 of RFC 3925 [4], which is illustrated in table 4.1.1.3:

Table 4.1.1.3: Format of Vendor-Identifying Vendor Class

| Code | Length 1-255 | Enterprise Number 1 | Data Len1 | Vendor Class Data 1 |  | Enterprise Number 2 | Data Len2 | Vendor Class Data 2 |  | … |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Data Length 1 | Opaque data 1 |  |  | Data Length 2 | Opaque data 2 | … |
| 124 | n | x | p | p1 | Data | y | q | q1 | Data | ... |
| octet | octet | 4 octets | octet | octet | n octets | 4 octets | octet | octet | n octets |  |

The NE may identify itself as MvPnC compatible DHCPv4 client using DHCPv4 option 124 in the following way:

- 	DHCPv4 option code 124;

- 	Vendor enterprise number 10415 for "3GPP";

- 	Opaque vendor class data length 5 bytes;

-	Length of Vendor Class Data field is 6 bytes;

- 	Vendor class data "MvPnC".

The use of Vendor- Identifying Vendor Class DHCPv4 option 124 with specific value MvPnC is illustrated in table 4.1.1.4.

Table 4.1.1.4: Use of Vendor-Identifying Vendor Class

| Code | Length 1-255 | Enterprise Number 1 | Data Len1 | Vendor Class Data 1 |  |  |  |  |  | … |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Data Length 1 | Opaque data 1 |  |  |  |  | … |
| 124 | n | 10415 | 6 | 5 | M | v | P | n | C | ... |
| octet | octet | 4 octets | octet | octet | octet | octet | octet | octet | octet | … |

The order of vendor-identifying vendor class contained in option 124 does not matter, and any other vendor-identifying vendor class data with a different IANA enterprise number, if required by the vendor, may appear before or after the 3GPP vendor class.

End of changes