3GPP TSG-SA5 Meeting #153	S5-240066

# Sevilla, Spain, 29th Jan 2024 - 2nd Feb 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.160 | CR | 0050 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR 32.160 Clarify YANG revisions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson Hungary Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17 |  |  |  |  |  | Date: |  |  | 2024-01-17 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The current text seems to indicate to some people that a new YANG revision is always needed if a YANG module is included in a new 3GPP release. However, there is no need to create a new revision unless there is some real change in the content of the YANG module. This needs to be clarified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that a new revision is not needed just because the module is included in a new release. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misunderstanding and potential creation of unneeded YANG module versions. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1.13 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

#### 6.2.1.13	YANG module revisions

A YANG module version is identified by its name and the date in the latest revision statement in it. When a module is changed in any way a new revision statement/date shall be added to it. Different versions of the same module shall contain a different (latest) revision date.

In order to minimize changes to the YANG interface it is recommended to use the same module revision (same YANG file) in multiple 3GPP releases as long as there is no interface effecting change between the different releases for that YANG module.

A separate "revision" statement shall be present for each new published version of a module. The revision statement shall contain a reference substatement listing the numbers of all 3GPP change requests and any other documents that resulted in the creation of the new revision.

Example:

revision 1956-10-13 {

reference “CR-0258, CR-0267”;}

NOTE:	Void.

If multiple change requests modify the new revision of a YANG module, the content of the reference substatements should be merged.

In case a YANG module revision (same YANG file) is used in multiple releases and needs similar updates in multiple releases (e.g. corrections mapped between the different releases) creating separate module revisions just because the different 3GPP releases use different CR numbers should be avoided. In such case a single new YANG module revision should be created and used in each release. This should contain the CR number from each release.

In order to avoid reusing the same revision date in multiple releases, when a new YANG module revision is needed the revision date should be set as follows. Instead of setting the exact revision date when the module was last edited, the date nearest to that day that is not in the future and that follows the rule below should be used.

When divided by 6, the day in the date should have the same remainder as the release number: (DAY modulo 6 == releaseNumber modulo 6).

Examples:

- Release 17 modulo 6 is 5 ; so day numbers 5, 11, 17, 23, 29 are acceptable while days e.g., 2 or 7 are not.

- Release 18 modulo 6 is 0 ; so day numbers 6, 12, 18, 24, 30 are acceptable while days e.g., 8 or 31 are not.

End of changes