3GPP TSG-SA1 Meeting #105	S1-240131

# Athens, Greece, 26th Feb 2024 - 1st Mar 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.261 | CR | 0781 | rev | - | Current version: | 19.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support for Multiple Spanning Tree Protocol |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Siemens, Robert Bosch GmbH, Mitsubishi Electric |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA1 |  |  |  |  |  |  |  |  |  |
| Work item code: | cyberCAV, TEI19 |  |  |  |  |  | Date: |  |  | 2024-02-16 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Many industrial automation networks use IEEE 802.1 virtual LANs extensively. The Multiple Spanning Tree Protocol (MSTP) is required to prevent loops in networks with VLANs. MSTP includes support of Rapid Spanning Tree Protocol (RSTP), but RSTP as such cannot support loop prevention in case of multiple VLANs. The existing requirement in TS 22.261 only mentions RSTP as an example. MSTP as a major protocol for loop prevention should be added so that it gets the necessary attention in downstream groups. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added MSTP to corresponding requirement on Ethernet transport services. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Limited applicability of 5G in industrial automation networks. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | 2, 3.2, 6.24.2 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	NGMN 5G White Paper v1.0, February 2015.

[3]	3GPP TS 22.011: "Service accessibility".

[4]	Void

[5] 	3GPP TS 22.278: "Service requirements for the Evolved Packet System (EPS)".

[6] 	3GPP TS 22.101: "Service aspects; Service principles".

[7]	3GPP TS 22.146: "Multimedia Broadcast/Multicast Service (MBMS)".

[8]	3GPP TS 22.246: "Multimedia Broadcast/Multicast Service (MBMS) user services".

[9] 	3GPP TS 22.186: "Enhancement of 3GPP support for V2X scenarios".

[10]	NGMN, "Recommendations for NGMN KPIs and Requirements for 5G", June 2016

[11]	3GPP TS 22.115: "Service aspects; Charging and billing".

[12] 	Void

[13]	Soriano, R., Alberto, M., Collazo, J., Gonzales, I., Kupzo, F., Moreno, L., & Lorenzo, J. OpenNode. Open Architecture for Secondary Nodes of the Electricity Smartgrid. In Proceedings CIRED 2011 21st International Conference on Electricity Distribution, CD1. June 2011.

[14] 	North American Electric Reliability Council. Frequently Asked Questions (FAQs) Cyber Security Standards CIP–002–1 through CIP–009–1. Available: http://www.nerc.com/docs/standards/sar/Revised_CIP-002-009_FAQs_06Mar06.pdf. 2006.

[15]	McTaggart, Craig, et al. "Improvements in power system integrity protection schemes". Developments in Power System Protection (DPSP 2010). Managing the Change, 10th IET International Conference on. IET, 2010.

[16]	IEEE Power Engineering Society – Power System Relaying Committee – System Protection Subcommittee Working Group C-6. Wide Area Protection and Emergency Control.

[17]	Begovic, Miroslav, et al. "Wide-area protection and emergency control". Proceedings of the IEEE 93.5, pp. 876-891, 2005.

[18]	ITU-T Recommendation G.1000 "Communications quality of service: A framework and definitions".

[19]	IEC 61907, "Communication network dependability engineering".

[20]	NIST, "Framework for Cyber-Physical Systems", 2016.

[21]	3GPP TS 22.104: "Service requirements for cyber-physical control applications in vertical domains".

[22]	3GPP TS 22.262: "Message Service within the 5G System".

[23]	3GPP TS 22.289: "Mobile Communication System for Railways".

[24]	3GPP TS 22.071: "Location Services".

[25]	3GPP TS 23.122: "Non-Access-Stratum (NAS) functions related to Mobile Station (MS) in idle mode".

[26]	3GPP TS 22.125: "Unmanned Aerial System (UAS) support in 3GPP".

[27]	Void

[28]	3GPP TS 22.263: "Service requirements for Video, Imaging and Audio for Professional Applications (VIAPA)".

[29]	Void

[30]	3GPP TS 22.179: "Mission Critical Push to Talk (MCPTT)".

[31]	IEEE 1588-2019, IEEE Standard for a Precision Clock Synchronization Protocol for Networked Measurement and Control Systems.

[32]	IEC 61850-9-3-2016 - IEC/IEEE International Standard - Communication networks and systems for power utility automation – Part 9-3: Precision time protocol profile for power utility automation.

[33]	3GPP TS 38.305: "NG Radio Access Network (NG-RAN); Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN"

[34]	ATIS-0900005: "Technical Report on GPS Vulnerability", https://access.atis.org/apps/group_public/download.php/36304/ATIS-0900005.pdf

[35]	European Commission, Regulatory Technical Standard 25. Level of accuracy of business clocks
https://ec.europa.eu/finance/securities/docs/isd/mifid/rts/160607-rts-25_en.pdf (annex https://ec.europa.eu/finance/securities/docs/isd/mifid/rts/160607-rts-25-annex_en.pdf)

[36]	5G-ACIA, "Exposure of 5G capabilities for Connected Industries and Automation Applications", 5G-ACIA white pater, February 2021, https://5g-acia.org/wp-content/uploads/2021/04/5G-ACIA_ExposureOf5GCapabilitiesForConnectedIndustriesAndAutomationApplications.pdf

[37]	3GPP TS 22.173: "IP Multimedia Core Network Subsystem (IMS) Multimedia Telephony Service and supplementary services".

[38]	ITU-T, "Technology Watch Report: The Tactile Internet", August 2014.

[39]	D. Soldani, Y. Guo, B. Barani, P. Mogensen, I. Chih-Lin, S. Das, "5G for ultra-reliable low-latency communications". IEEE Network. 2018 Apr 2; 32(2):6-7.

[40]	O. Holland et al., "The IEEE 1918.1 "Tactile Internet" Standards Working Group and its Standards," Proceedings of the IEEE, vol. 107, no. 2, Feb. 2019.

[41]	Altinsoy, M. E., Blauert, J., & Treier, C., "Inter-Modal Effects of Non-Simultaneous Stimulus Presentation," A. Alippi (Ed.), Proceedings of the 7th International Congress on Acoustics, Rome, Italy, 2001.

[42]	Hirsh I.J., and Sherrrick C.E, 1961. J. Exp. Psychol 62, 423-432

[43]	Altinsoy, M.E. (2012). "The Quality of Auditory-Tactile Virtual Environments," Journal of the Audio Engineering Society, Vol. 60, No. 1/2, pp. 38-46, Jan.-Feb. 2012.

[44]	M. Di Luca and A. Mahnan, "Perceptual Limits of Visual-Haptic Simultaneity in Virtual Reality Interactions," 2019 IEEE World Haptics Conference (WHC), 2019, pp. 67-72, doi: 10.1109/WHC.2019.8816173.

[45]	K. Antonakoglou et al., “Toward Haptic Communications Over the 5G Tactile Internet”, IEEE Communications Surveys & Tutorials, 20 (4), 2018.

[46]	ETSI GS OEU 020 (v1.1.1): "Operational energy Efficiency for Users (OEU); Carbon equivalent Intensity measurement; Operational infrastructures; Global KPIs; Global KPIs for ICT Sites".

[47]	3GPP TS 28.310: "Management and orchestration; Energy efficiency of 5G".

[48]	ETSI EN 303 472: "Environmental Engineering (EE); Energy Efficiency measurement methodology and metrics for RAN equipment".

[49]	3GPP TS 32.299: " Telecommunication management; Charging management; Diameter charging applications".

[50]	N. Nonaka et al., "Experimental Trial aboard Shinkansen Test Train Running at 360 km/h for 5G Evolution," 2022 IEEE 95th Vehicular Technology Conference: (VTC2022-Spring), Helsinki, Finland, 2022.

[51]	3GPP TS 22.137: " Service requirements for Integrated Sensing and Communication ".

[52]	3GPP TS 22.369: " Service requirements for ambient power-enabled IoT ".

[53]	3GPP TS 22.156: "Mobile Metaverse Services”.

[54x]	IEEE Std 802.1Q: "IEEE Standard for Local and Metropolitan Area Networks---Bridges and Bridged Networks".

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

5G LAN-VN	5G LAN-Virtual Network

A/S	Actuator/Sensor

CPN	Customer Premises Network

eFMSS	Enhancement to Flexible Mobile Service Steering

eRG	Evolved Residential Gateway

eV2X	Enhanced V2X

FL	Federated Learning

FMSS	Flexible Mobile Service Steering

GEO	Geostationary satellite Earth Orbit

ICP	Internet Content Provider

ID	Identification

IMU	Inertial Measurement Unit

IOPS	Isolated E-UTRAN Operation for Public Safety

ISL	Inter-Satellite Link

LEO	Low-Earth Orbit

MBS	Metropolitan Beacon System

MCS	Mission Critical Services

MCX	Mission Critical X, with X = PTT or X = Video or X = Data

MEO	Medium-Earth Orbit

MIoT	Massive Internet of Things

MMTEL	Multimedia Telephony

MOCN	Multi-Operator Core Network

MPS	Multimedia Priority Service

MSGin5G	Message Service Within the 5G System

MSTP	Multiple Spanning Tree Protocol

NGSO	Non-Geostationary Satellite Orbit

NPN	Non-Public Network

PIN	Personal IoT Network

PRAS	Premises Radio Access Station

RSTP	Rapid Spanning Tree Protocol

RVAS	Roaming Value-Added Service

RSTP	Rapid Spanning Tree Protocol

SEES	Service Exposure and Enablement Support

S&F	Store and Forward

SST	Slice/Service Type

TBS	Terrestrial Beacon System

TTFF	Time To First Fix

UAV	Uncrewed Aerial Vehicle

UTC 	Coordinated Universal Time

XR	eXtended Reality

### 6.24.2	Requirements

The 3GPP system shall be able to support an Ethernet transport service.

The 5G network shall support the routing of non-IP packet (e.g. Ethernet frame) efficiently for private communication between UEs within a 5G LAN-type service.

The 5G network shall be able to provide the required QoS (e.g. reliability, latency, and bandwidth) for non-IP packet (e.g. Ethernet frame) for private communication between UEs within a 5G LAN-type service.

The Ethernet transport service shall support routing based on information extracted from Virtual LAN (VLAN) ID by the 3GPP system.

The Ethernet transport service shall support the transport of Ethernet frames between UEs that Ethernet devices are connected to.

The Ethernet transport service shall support the transport of Ethernet frames between a UE that an Ethernet device is connected to and an Ethernet network in DN (Data Network).

NOTE:	If more than one Ethernet devices need to be connected to a UE, they can be connected using an Ethernet switch between the devices and the UE.

The Ethernet transport service shall support the transport of Ethernet broadcast frames.

The Ethernet transport service shall support traffic filtering and prioritization based on source and destination MAC addresses.

The Ethernet transport service shall support traffic filtering and prioritization based on Ethertype (including multiple Ethertypes in double tagging).

The Ethernet transport service shall support traffic filtering and prioritization based on IEEE 802.1Q VLAN tags (including double tagging).

The Ethernet transport service shall support routing based on information extracted by the 3GPP system from the Bridge Protocol Data Units created in the Ethernet network based on a Spanning Tree Protocol (e.g. RSTP, MSTP [54x]).