SA WG6 Meeting #59	S6-240544

Athens, Greece, 26th February – 1st March 2024	(revision of S6-240090)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.542 | CR | 0053 | rev | 1 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of Additional PEMCs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | PINAPP |  |  |  |  |  | Date: |  |  | 2024-02-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In PIN creation procedure, the PEMC can involve other several PEMCs as the individual PEMC, that are allowed to manage the PIN: In order to save the procedure of several PEMCs to be involved into the certain PIN as individual PEMC, the PEMC can have the additional PEMC GPSIs/PIN client ID in the PIN create request, to indicate additional PEMCs that are allowed to manage the PIN. This procedure doesn’t have conflict with that other PEMC requests to join the certain PIN and becomes PEMC separately. But after PIN creation, there is no method to add/remove other PEMCs into the PIN. So it is to limited that only during PIN creation procedure to add additional PEMC into the PIN. It is reasonable that during PIN modification procedure that can also add additional PEMC into the PIN. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add clarification of the details of adding additional PEMC into the PIN during PIN modification procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is too limited that only during PIN creation procedure to add additional PEMC into PIN. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.5.10.3.2, 8.5.10.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change 1 * * *

8.5.10.3.2	PIN configuration request

Table 8.5.10.3.2-1 shows the informational elements of the PIN configuration request sent by a PIN Element to the PIN server to obtain authorization for the modification of a PIN.

Table 8.5.10.3.2-1: PIN configuration request

| Information element | Status | Description |
| --- | --- | --- |
| PIN ID | M | The identifier of the PIN |
| Requester PINE ID | M | The identifier of the PIN Element making the request |
| Authorization type | M | Request for the authorization to modify the configuration of a PIN: PEMC role change, PEGC role change |
| PINE ID of predecessor in the role | M | PINE ID of the element serving in the indicated role and needs to be changed e.g. due to being unavailable. |
| PINE ID of proposed successor in the role | O | PINE ID of the element proposed to assume the role |
| Additional PEMCs | O | Indicate additional PEMCs (with identifier of PEMC) that are requested to manage the PIN. |

* * * Start of Change 2 * * *

8.5.10.3.3	PIN configuration response

Table 8.5.10.3.3-1 shows the informational elements of the PIN configuration response provided by the PIN server to authorize the modification of a PIN.

Table 8.5.10.3.3-1: PIN configuration response

| Information element | Status | Description |
| --- | --- | --- |
| Response | M | The response (authorize or not authorize) from the PIN Server. |
| PIN profile information | M | The IEs from the PIN profile information that the PIN Server has updated for modifying the PIN |
| Dynamic PIN profile information | M | The IEs from the dynamic PIN profile information that the PIN Server has authorized for modifying the PIN |
| Additional PEMCs | O | Indicate additional PEMCs (with identifier of PEMC) that are allowed to manage the PIN. |

* * * End of Change * * *