3GPP TSG-SA WG6 Meeting #59	S6-240249

Athens, Greece, 26th February – 1st March 2024	(revision of S6-24xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.280 | CR | 0538 | rev |  | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Reporting success or failure of an ad hoc group emergency alert modify criteria request (multiple MC systems) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | enhMC |  |  |  |  |  | Date: |  |  | 2024-02-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | If the initiator of an ad hoc group emergency alert modify request is not authorised, current standard does not allow the MC service server to return a report in case of failure of the request. Similar changes were agreed in S6-233787/S6-233779 (adhoc alert establishment) and S6-233780/S6-233781 (adhoc alert cancel). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding new steps in clause 10.10.3.4.4 to report authorisation result of the ad hoc group emergency alert modify request. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The initiator will not be notified if the ad hoc group emergency alert modify request has failed. The reason for denial will not be known by the initiator. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.10.3.4.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

##### 10.10.3.4.4	Modifying the criteria for determining the participants during an ongoing ad hoc group emergency alert

Figure 10.10.3.4.4-1 illustrates when the alerting user modifies the criteria for determining the list of participants of an ongoing ad hoc group emergency alert between multiple MC systems. This includes a scenario of MC service users entering and leaving an ongoing ad hoc emergency alert.

Preconditions:

-	The MC service user at the MC service client 1 is authorized to modify the criteria.

-	The MC service server A and MC service server B are aware of the criteria related to the ongoing ad hoc group emergency alert.

-	MC system A and MC system B are interconnected.

Figure 10.10.3.4.4-1: Modifying the criteria for determining the participants during an ongoing ad hoc group emergency alert between multiple MC systems

1.	An ad hoc group emergency alert has been established based on criteria sent by an authorized MC service client upon initiating the ad hoc emergency alert.

2.	The MC service user at the MC service client 1 is authorized and wishes to modify the criteria for determining the list of ad hoc group emergency alert participants.

3.	The MC service server A receives the ad hoc group emergency alert modify request.

4.	The MC service server A checks whether MC service client 1 is authorized to modify the criteria which determines the list of ad hoc group participants.

5.	The MC service server A sends the ad hoc group emergency alert modify request return to MC service client 1 containing the result of whether modifying the criteria for determining the participants of the ad hoc group is authorized or not. If the ad hoc group emergency alert modify request is not authorized, the MC service server and the MC service client 1 shall not proceed with the rest of the steps.

6.	If MC service server A identifies that the criteria require to add participants from another MC system then MC service server A sends an ad hoc group emergency alert request to MC service server B at MC system B.

7a/b.	The MC service server A receives the ad hoc group emergency alert modify response and forwards the response to MC service client 1 in MC system A.

8.	MC service server B detects that MC service client 3 meets the criteria for the ongoing ad hoc group emergency alert initiated at MC system A.

8a.	MC service server B sends an ad hoc group emergency add user notification message towards MC service server A.

8b.	MC service server A sends an ad hoc group emergency alert request towards MC service client 3.

8c.	The MC service user 3 is notified of entering an ongoing ad hoc group emergency alert.

8d.	MC service client 3 sends an ad hoc group emergency alert response towards MC service server A.

8e.	MC service server B sends an ad hoc group emergency remove user notification message towards MC service server A.

8f.	MC service server A sends an ad hoc group emergency alert cancel request towards MC service client 4.

8g.	The MC service user 4 is notified of leaving an ongoing ad hoc group emergency alert.

8h.	MC service client 4 sends an ad hoc group emergency alert cancel response towards MC service server A.

9.	The authorized user on MC service client 1 is notified with the MC service IDs of those MC service users who are currently in the ad hoc group. This notification may be sent to the authorised MC service user by the MC service server A more than once during the call when MC service users join or leave the ad hoc group.

10.	The MC service server A adds MC service client 3 to the ad hoc group emergency alert and removes MC service client 4 from the ad hoc group emergency alert.

The MC service servers continuously check whether other MC service clients meet or if participating MC service clients no longer meet the criteria for the ad hoc group emergency alert.

NOTE:	If the ad hoc group emergency alert is associated with an ad hoc group call and the change of criteria caused the modification of the ad hoc group emergency alert participant list then the ongoing ad hoc group call is modified accordingly.

# * * * * End of changes * * * *