3GPP TSG-SA WG6 Meeting #59	S6-240436

Athens, Greece, 26th February – 01st March 2024	(revision of S6-240116)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.280 | CR | 0509 | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | ACM updating MC service user profiles assigned from partner MC system |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | BDBOS |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCShAC |  |  |  |  |  | Date: |  |  | 26.01.2024 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The study TR 23.700-38 has provided a solution on updating MC service user profiles assigned from a partner MC system where an authorized MC service user of primary MC system can request update for those user profiles through its ACM client. Introducing this procedure to chapter 10.17 will add a means on this subject to MC architecture. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR adds clauses to chapter 10.17 specifying information flows and the procedure of updating MC service user profiles assigned from partner MC system. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There will be no clarifation on how MC service user profiles assigned from partner MC systems can be updated |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.17.2.15 (new), 10.17.2.16 (new), 10.17.7 (new), 10.17.7.1 (new), 10.17.7.2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 10.17.2.15	MC service user profile update request

Table 10.17.2.15-1 describes the information flow sent from ACM client to ACM server in the primary MC system, from ACM server of primary MC system to ACM server of partner MC system and from ACM server to ACM client in partner MC system.

Table 10.17.2.15-1: MC service user profile update request

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID | M | The identity of the MC service user at the primary MC system which triggers the request. |
| Functional alias | O | The functional alias of the MC service user at the primary MC system which triggers request. |
| MC service ID | O (see NOTE) | The identity of the MC service user at the partner MC system towards which the request is sent |
| Functional alias | O (see NOTE) | The functional alias of the MC service user at the partner MC system towards which the request is sent |
| MC service ID | M | MC service ID of MC service user permitted to migrate, created by the partner MC system. |
| MC service user profile | M | The suggested updated MC service user profile on the partner MC system. |
| Additional Information | O | Additional information regarding the suggested updated MC service user profile on the partner MC system. |
| NOTE: Either the MC service ID or the functional alias must be present. |  |  |

#### 10.17.2.16	MC service user profile update response

Table 10.17.2.16-1 describes the information flow sent from ACM client to ACM server in the partner MC system, from ACM server in partner MC system to ACM server in primary MC system and from ACM server to ACM client in primary MC system.

Table 10.17.2.16-1: MC service user profile update response

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID | M | The identity of the MC service user at the partner MC system which handled the received request. |
| Functional alias | O | The functional alias of the MC service user at the partner MC system which handled the received request. |
| MC service ID | M | The identity of the authorized user in the primary MC system which is target of the response. |
| MC service ID | M | MC service ID of MC service user permitted to migrate, created by the partner MC system. |
| Result | M | Result information of the MC service user profile update request, |

* * * Second Change * * * *

### 10.17.7	ACM updating MC service user profiles assigned from partner MC system

#### 10.17.7.1	General

A primary MC system may request through ACM entities to add a list of its MC service user for migration to a connected partner MC system as specified in clause 10.17.5 ACM user migration management. In case the partner MC system agrees to accept these users for migration it will reply with a response containing assigned MC service user identities and MC service user profiles. Those will be applicable and valid while the requested primary MC service users are migrated into the partner MC system,

In case, the primary MC system may want to update the received MC service user profiles from the partner MC system it can send a request for updating those as described in the procedure below.

#### 10.17.7.2	Procedure

The procedure for an authorized MC service user in a primary MC system to request MC service user profile update from a partner MC system is shown in figure 10.17.7.2-1.

Pre-conditions:

-	Primary MC system and partner MC system are interconnected.

-	Both MC systems have implemented ACM functionality.

-	When a functional alias is used as the target of the request by an ACM client in the primary MC system, it is required that the primary ACM server has subscribed to the MC service functional alias controlling server within the primary MC system, and that the partner ACM server has subscribed to the MC service functional alias controlling server within the partner MC system.

Figure 10.17.7.2-1: MC service user profile update

1.	The primary ACMC sends MC service user profile update request to the primary ACMS providing the required  changes for the MC service user that is migrated to the partner MC System.

2.	The primary ACMS performs an authorization check to verify that the MC service user is authorized to perform this action. If the authorization check fails, the procedure is stopped.

3.	The primary ACMS sends the MC service user profile update request to the partner ACMS.

4. 	The ACMS of the partner MC system sends a process indication to primary MC system, and stores, verifies and assesses the incoming request.

Based on local policy,

-	the ACMS of the partner MC system may automatically handle the request as described in clause 10.17.3.2 and continues with step 8,

or

-	the ACM server requests verification by an ACM client: If the user is not logged on, the pending request procedure as described in clause 10.17.3.1 is followed. If the request targets a functional alias, the ACM server of the partner MC system resolves the functional alias (based on the current state of the functional alias) and proceeds as follows:

-	if the functional alias is active for one ACM client, the request is sent to the ACM client;

-	if the functional alias is active for more than one ACM client, the ACM server sends the request to one of these ACM clients based on the local policy;

-	if the functional alias is not currently active, the ACM server of the partner MC system stores the request.

5.	The partner ACMS forwards the stored MC service user profile update request to the partner ACMC.

6.	The authorized MC service user of partner MC system checks the content of the MC service user profile update request and decides whether to approve it or not. The authorized user in the partner MC system takes whatever actions are needed to apply the MC service user profile update. The actions within the partner MC system to apply this configuration are outside the scope of the 3GPP specifications.

7. The partner ACMC sends MC service user profile update response to the partner ACMS.

8. The partner ACMS sends MC service user profile update response to the primary ACMS.

9.	The primary ACMS stores the MC service user profile update response. If the primary ACMC is not logged on the primary ACMS will follow the pending request procedure as described in clause 10.17.3.1.

10.	The primary ACMS forwards the MC service user profile update response to the primary ACMC

* * * End of Change * * * *