3GPP TSG-WG SA2 Meeting #160-Ad Hoc-e	S2-2400118

# E-meeting, January 22 – 29, 2024		(revision of S2-2312334)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.586 | CR | 0078 | rev | 1 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update parameter provisioning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Sony |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Ranging_SL |  |  |  |  |  | Date: |  |  | 2024-01-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The clause describing the policy/parameter provisioning lists information that is provisioned to the UE. The list does not mention any parameters that is needed for e.g. discovery, control and exposure service. The equivalent clause related to UE provisioning for 5G ProSe and V2x, includes references to subclauses that includes more details. SA3 specification TS 33.533 specifies e.g. that the SLPKMF shall be able to provision e.g. security materials for discovery, unicast and groupcast communication. Furthermore clause 5.6.2.3 states that Onboarding information is provisioned. To make clause 5.1.1.2 complete then also policy/parameter information related to Ranging/SL positioning services shall be documented as part of what the UE may be provisioned with. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add policy/parameter information related to Ranging/SL positioning services shall be documented as part of what the UE mayl be provisioned with. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The list of information provisioned to the UE is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

#### 5.1.1.2	Policy/Parameter provisioned to UE

The following sets of information for Ranging/SL positioning service is provisioned to the UE:

-	When the UE is served by NG-RAN, the PLMNs in which the UE is authorized to perform Ranging/SL positioning service over PC5 reference point for acquiring relative distance, relative direction, high accuracy relative distance and high accuracy relative direction.

-	When the UE is not served by NG-RAN, indicates whether the UE is authorized to perform Ranging/SL positioning service over PC5 reference point for acquiring relative distance, relative direction, high accuracy relative distance and high accuracy relative direction.

-	The mapping between Ranging/SL positioning services (e.g. ProSe identifiers, V2X service types) and Ranging/SL positioning QoS parameters as defined in clause 5.7.2.

-	The mapping between Ranging/SL positioning services (e.g. ProSe identifiers, V2X service types) and PQI values for RSPP transport QoS as defined in clause 5.7.3.

-	The authorized Ranging/SL Positioning role per PLMN, e.g. Target UE, SL Reference UE, Located UE, SL Positioning Client UE, and SL Positioning Server UE.

-	When the UE is not served by NG-RAN, indicates the authorized Ranging/SL positioning role, e.g. Target UE, SL Reference UE, Located UE and SL Positioning server UE.

-	When the UE is served by NG-RAN and when Network-based Operation is not supported by the 5GC network, whether it is allowed to use UE-only operation to perform Ranging/ SL Positioning.

-	The PCF provisions one or more of the following Ranging/SL Positioning Policy/parameters:

-	 Policy/parameters for Ranging/Sidelink Positioning UE Discovery & Selection as specified in clause 5.2.

-	 Policy/parameters for Ranging/Sidelink Positioning UE Control as specified in clause 5.3.

NOTE: The V2X capable UE and/or 5G-ProSe capable UE may get Policy/parameters as part of the V2X and/or 5G-ProSe authorization and provisioning.

-	 Policy/parameters for Ranging/Sidelink Positioning service exposure as specified in clause 5.6.

# * * * * End of changes * * * *