TSG SA Meeting #103	SP-240125

19-22 March 2024, Maastricht, Netherlands

# Source:	SA WG2

# Title:	New WID: Deferred 5GC-MT-LR Procedure for Periodic Location Events based NRPPa Periodic Measurement Reports

# Document for:	Approval

# Agenda Item:	6.5.2

3GPP TSG-WG SA2 Meeting #161	S2-2403553

26 Feb – 1 March, 2024, Greece, Athens

Source:	Ericsson

Title:	New WID: Deferred 5GC-MT-LR Procedure for Periodic Location Events based NRPPa Periodic Measurement Reports

Document for:	Approval

Agenda Item:	30.2

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title:	Deferred 5GC-MT-LR Procedure for Periodic Location Events based NRPPa Periodic Measurement Reports

Acronym:	TEI19_DLPMR

Unique identifier:	1030019

Potential target Release:	Rel-19

# 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  |  | X |  |
| No | X | X | X |  |  |
| Don't know |  |  |  |  |  |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a …

|  | Study |
| --- | --- |
|  | Normative – Stage 1 |
| x | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

## 2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
|  | N/A |  |  |

### 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |

Dependency on non-3GPP (draft) specification:

# 3	Justification

RAN provides solution on NRPPa in NR to receive periodic location measurements from gNB. LMF could use this periodical location information to generate location estimates periodically. However, there is no SA2 procedure which could leverage this periodical received location information to generate location estimates periodically.

SA2 currently provides a solution (Deferred 5GC-MT-LR Location for Periodic Location Events) to generate location estimates periodically, but this procedure does not leverage NRPPa periodical measurement report.

It would be beneficial to have a 3GPP procedure for periodic location event using NRPPa periodical measurement report by LMF to generate location estimates periodically and transfer those to estimates GMLC for commercial services similar as those already been addressed by the Deferred Location Procedure (defined in clause 6.3 in TS 23.273).

# 4	Objective

The objective is to introduce a new solution for Periodic Location Report by leveraging the RAN existing NRPPa periodic measurement for commercial services.

The solution adds following changes:

- LMF controls the NRPPa Periodic Measurement Inititation;

- Handling of all UE mobility scenarios in case NRPPa periodic measurement has been initiated by the LMF;

- Cancellation of the procedure by the LMF in case NRPPa periodic measurement has been initiated.

# 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 23.273 | LMF function impacted, minor impact on AMF and GMLC logic. | SA#106 December 2024 | TBD |

Estimated TU: 0.5

# 6	Work item Rapporteur(s)

Richard, Batorfi, Ericsson, richard.batorfi@ericsson.com

# 7	Work item leadership

SA WG2

# 8	Aspects that involve other WGs

CT WG4 impact if CR is approved.

# 9	Supporting Individual Members

Supporting IM name

Ericsson

T-Mobile USA

Huawei

China Mobile

AT&T

Nokia

CATT