TSG SA Meeting #103	SP-240128

19-22 March 2024, Maastricht, Netherlands

# Source:	SA WG2

# Title:	New WID: Providing per-subscriber VLAN instructions from UDM and DN-AAA

# Document for:	Approval

# Agenda Item:	6.5.2

3GPP TSG SA WG2 Meeting #161			S2-2403556

Athens, Greece, February 26 – March 1, 2024	(revision of S2-240xxxx)

# Source:	Ericsson, Verizon, Nokia, Nokia, Nokia Shanghai Bell, Oracle

# Title:	New WID on providing per-subscriber VLAN instructions from UDM and DN-AAA

# Document for:	Information

# Agenda Item:	30.2

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: 	Providing per-subscriber VLAN instructions from UDM and DN-AAA

Acronym:	TEI19_VLANSUB

Unique identifier:	1030022

Potential target Release:	Rel-19

# 1	Impacts

{For Normative work, identify the anticipated impacts. For a Study, identify the scope of the study}

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  |  | X |  |
| No | X | X | X |  | X |
| Don't know |  |  |  |  |  |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a …

|  | Study |
| --- | --- |
|  | Normative – Stage 1 |
| X | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

## 2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
|  |  |  | N/A |

### 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |

# 3	Justification

The support for VLAN handling in 5GS for Ethernet PDU Sessions is currently rather limited. As specified in TS 23.501, clause 5.6.10.2:

- 	There is support for a DN-AAA Server to provide a list of allowed VLAN IDs (for a maximum of 16 VLAN tags). This allows the UPF to forward packets to/from a PDU Session only if it has one of the allowed VLAN tags. Ethernet frames with other VLAN tags will be dropped.

- 	There is also support for SMF to be configured with instructions on VLAN handling (e.g. the VLAN tag to be inserted or removed, S-TAG to be inserted or removed).

Support for SMF/UPF to add/remove VLAN tags with different values per subscriber is thus not available. Only pre-configured VLAN tag values, per DNN/S-NSSAI, can be added per current specification. However, in deployments for e.g. FWA or wholesale Ethernet services there is a need to add subscriber specific VLAN tag values, or remove VLAN tags only for certain subscribers. There is thus a need to support per-subscriber specific VLAN tag handling.

An appropriate place for per-subscriber VLAN handling information is in SM subscription data in UDM/UDR. Another option is to provide the VLAN instructions from the DN-AAA, which today can provide a list of allowed VLAN tags (note that this is not the same as instructions for adding/removing S- or C-tags).

# 4	Objective

The following aspects will be specified:

WT 1:	Add support for VLAN handling instructions in SM subscription data in UDM/UDR and DN-AAA.

## TU estimates and dependencies

| Work Tas ID | TU Estimate (Study) | TU Estimate (Normative) | RAN Dependency (Yes/No/Maybe) | Inter Work Tasks Dependency |
| --- | --- | --- | --- | --- |
| WT 1 |  | 0.5 | No | None |

# 5	Expected Output and Time scale

{If this WID covers both stage 2 and stage 3, clearly indicate the different completion dates.}

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| TS 23.501 | Support of VLAN handling instructions in SM subscription data and DN-AAA | TSG#106 (Dec 2024) |  |
| TS 23.502 | Support of VLAN handling instructions in SM subscription data | TSG#106 (Dec 2024) |  |

# 6	Work item Rapporteur(s)

Stefan Rommer, Ericsson (stefan . rommer @ ericsson . com)

# 7	Work item leadership

SA WG2

# 8	Aspects that involve other WGs

# 9	Supporting Individual Members

{At least 4 supporting Individual Members are needed. There is an expectation that these companies will provide resources to progress the work. Note that having 4 supporting companies is a necessary but not sufficient condition: the usual TSG approval process by consensus is needed for the WID approval}

Supporting IM name

Ericsson

Verizon

Nokia

Nokia Shanghai Bell

Oracle

Samsung

CableLabs