# 3GPP TSG SA WG5 Meeting #153                 	           S5-240725

# Sevilla, SPAIN, 29 Jan - 3 Feb 2024

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.240 | CR | 0481 | rev | 1 | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR 32.240 Clarify the B2B charging architecture and principles |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | B2B_CH |  |  |  |  |  | Date: |  |  | 2024-02-021 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP o. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The concepts in B2B charging Annex G is not fully synchronized, i.e. the use of consumer CHF (C-CHF) and business CHF (B-CHF). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Synchronize the use of concept for B-CHF and C-CHF. Update the references. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The informative annex does not provide a clear guidance for B2B charging. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex G.2.1, G.2.2, G.2.3, G.2.4, G.3.2, G.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR … CR |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is revision of S5-240303. |  |  |  |  |  |  |  |  |

First change

G.2.1	General

In the B2B scenario, the charging architecture may involve three modes of interaction:

-	B-CHF only interaction: the NF (CTF) collects and reports the charging information per business, alternatively it collects and reports the charging information per consumer but for business charging purposes, to the business CHF (i.e. B-CHF).

-	B-CHF via C-CHF interaction: the NF (CTF) collects and reports the charging information per consumer to a consumer CHF (i.e. C-CHF), and the C-CHF reports the charging information per consumer to the business CHF (i.e. B-CHF).

-	C-CHF via B-CHF interaction: the NF (CTF) collects and reports the charging information per consumer to a business CHF (i.e. B-CHF), and the B-CHF reports the charging information per consumer to the consumer CHF (i.e. C-CHF).

Next change

G.2.2	B2B charging architecture and references with B-CHF only

In the B2B charging architecture with B-CHF only interaction, the charging information collected by the NF (CTF) is per business or per consumer but used for business charging purpose. When B-CHF or C-CHF may be applicable, is specified in the middle tier charging TSs.

The B-CHF performs charging functionalities for the business.

The related NF(s) and the corresponding references for B2B charging with B-CHF only, but not limited to:

-	Network exposure domain charging: NEF charging per API invocation/notification, as specified in TS 32.254 [14].

-	Edge charging: CEF based charging and EAS deployment charging, as specified in TS 32.257 [17].

-	Short message charging: SMSF charging for IoT, as specified in TS 32.274 [34], in this case the charging information is collected per consumer but used in a business context.

-	Prose charging: ProSe Function or DDNMF for prose discovery and prose communication charging, as specified in TS 32.277 [37], in this case the charging information is collected per consumer but used in a business context.

-	5G LAN charging: NEF charging/CEF based charging for 5G LAN management and configuration, as specified in TS 32.254 [14].

-	NPN charging: SMF and AMF charging for the inter-provider charging of NPN, as specified in TS 32.255 [15] and TS 32.256 [16], in this case the charging information is collected per consumer but used in a business context.

-	Network slice charging: NSPA, NSM, NSACF and NSSAAF charging per S-NSSAI for network slice, specified in TS 28.201 [70], TS 28.202 [71].

-	TSN charging: TSN AF charging for TSN Bridge management and configuration, NEF and TSCTSF charging for the enabler of TSC and TS, specified in TS 32.282 [42].

Next change

G.2.3	B2B charging architecture and references with B-CHF via C-CHF

In the B2B charging architecture with B-CHF via C-CHF interaction, the charging information collected by the NF (CTF) is per consumer.

The C-CHF performs charging functionalities for the consumer.

The B-CHF performs charging functionalities for the business.

The related NF(s) and the corresponding references for the B2B charging with business CHF via consumer CHF include, but not limited to:

-	Network slice charging: inter CHF interaction to support converged charging for tenant and UEs belonging to the tenant at same time, as specified in Figure 4.3-1 TS 32.255 [15].

-	TSN charging: inter CHF interaction to support converged charging for TSC traffic belonging to the same TSN bridge, as specified in Figure 4.3-1 TS 32.255 [15].

Next change

G.2.4	B2B charging architecture and references with B-CHF and C-CHF

In the B2B charging architecture where the NF (CTF) have direct B-CHF interaction and where the B-CHF interact with the C-CHF, the charging information collected by the NF (CTF) is per consumer and used for both B2C and B2B charging.

The related NF(s) and the corresponding references for the B2B charging with B-CHF, where the B-CHF interacts with the C-CHF include, but not limited to:

-	5G data connectivity charging in local breakout roaming scenario, as specified in Figure 4.2.6a TS 32.255 [15];

-	5G connection and mobility charging in local breakout roaming scenario, as specified in Figure 4.2.2.3 TS 32.256 [16];

Next change

G.3.2	B2B charging information utilisation

In the B-CHF only interaction, the NF (CTF) collect and report the charging information per business subscriber to B-CHF.

In the B-CHF via C-CHF interaction, the NF (CTF) collect and report the charging information to C-CHF. C-CHF report the charging information per business subscriber to B-CHF.

In the C-CHF via B-CHF interaction, the NF (CTF) collect and report the charging information to B-CHF. B-CHF report the charging information to C-CHF.

The identification for the business subscriber is specified in the respective middle tier TS, e.g.

-	Tenant Identifier in TS 28.201 [70], TS 32.282 [42].

-	EAS Provider Identifier in TS 32.257 [17].

Next change

G.3.3	B2B charging data generation and quota supervision

Both B-CHF and C-CHF can generate CDRs, with information for both individual subscriber identification and the related business subscriber identifications if any.

-	The B-CHF generate CDRs for the business subscriber.

-	The C-CHF generate CDRs for the individual subscriber.

In the B-CHF only interaction, the quota is requested and granted between CTF and B-CHF.

-	The CTF request and consume quota from B-CHF.

-	B-CHF grant quota to the CTF.

In the B-CHF via C-CHF interaction, the quota is requested and granted between CTF and B-CHF via C-CHF.

-	The CTF request and consume quota from B-CHF via C-CHF.

-	B-CHF grant quota to the CTF.

-	C-CHF forward and re-allocate quota.

The Nchf services for interaction between C-CHF and B-CHF are specified in TS 32.290 [57].

End of change