3GPP TSG-SA5 Meeting #153 	S5-240913

Sevilla, Spain, 29 January - 02 February 2024

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.557 | CR | 0011 | rev | 1 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR TS 28.557 Add solution for SLA monitoring and assurance in NPN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | OAM_NPN_Ph2 |  |  |  |  |  | Date: |  |  | 2024-02-02 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Solution for use case "SLA monitoring and assurance" described in TS 28.557 clause 5.1.0.4 is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Provide a solution for NPN SLA management. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The solution is missing in NPN specification TS 28.557 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, new clause 6.1.X, Annex B.Y |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 28.530: "Management and orchestration; Concepts, use cases and requirements".

[3]	3GPP TS 23.501: "System architecture for the 5G System (5GS)".

[4]	3GPP TS 22.261: "Service requirements for the 5G system".

[5]	5G-ACIA White paper: "5G Non-Public Networks for Industrial Scenarios", July 31, 2019.

[6]	3GPP TS 23.003: "Numbering, addressing and identification".

[7]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[8]	3GPP TS 28.531: "Management and orchestration; Provisioning".

[9]	3GPP TS 38.413: "NG-RAN; NG Application Protocol (NGAP)".

[10]	3GPP TS 38.473: "NG-RAN; F1 Application Protocol (F1AP)".

[11]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".

[12]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[13]	3GPP TS 28.554: "Management and orchestration; 5G end to end Key Performance Indicators (KPI)".

[14]	3GPP TS 28.532: "Management and orchestration; Generic management services".

[15]	3GPP TS 28.622: "Telecommunication management; Generic Network Resource Model (NRM); Integration Reference Point (IRP); Information Service (IS)".

[16]	3GPP TS 32.422: "Telecommunication management; Subscriber and equipment trace; Trace control and configuration management".

[17]	3GPP TS 28.537: "Management and orchestration; Management capabilities".

[18]	5G-ACIA: Exposure of 5G Capabilities for Connected Industries and Automation Applications, https://5g-acia.org/whitepapers/exposure-of-5g-capabilities-for-connected-industries-and-automation-applications-2/.

[19]	3GPP TS 28.104: "Management and orchestration; Management Data Analytics (MDA)".

[20]	3GPP TS 32.423: "Telecommunication management; Subscriber and equipment trace; Trace data definition and management".

[21]	3GPP TS 28.533: "Management and orchestration; Architecture framework"

[22]	3GPP TS 22.104: "Service requirements for cyber-physical control applications in vertical domains; Stage 1"

[m]	3GPP TS 28.536: "Management and orchestration; Management services for communication service assurance; Stage 2 and stage 3".

Next Change

## 6.1	Generic solutions for management of NPN

### 6.1.X	Procedure for NPN SLA management

Figure 6.1.X-1: Procedure for NPN SLA management

In figure 6.1.X-1, steps 1 to 3 describe the workflows of NPN provisioning which is specified as NPN provisioning procedures in clause 6.2.1 (for SNPN scenarios) and clause 6.3.1 (for PNI-NPN scenarios). The steps 4, 5 and 6 illustrate the NPN closed control loop assurance which includes SLA/SLS monitoring and assurance loop.

The 3GPP management system (e.g. NPN-SP/NPN-OP, see clause 4.1 roles related to NPN management for more details) collects performance data (e.g. packet delay in TS 28.552 [12] and reliability in TS 28.554 [13]) to monitor the NPN status through performance assurance MnS. The typical categories of KPI include Accessibility, Integrity, Utilization, Retainability, Mobility, Energy Efficiency and Reliability as defined in TS 28.554 [13].

Based on the network KPIs, the 3GPP management system evaluates the  SLA fulfilment. If the SLA is not fulfilled, the 3GPP management system adopts some optimization methods to promote the network performance, such as SLA parameter coordination in the corresponding NG-RAN and 5GC nodes and NSI(s)/NSSI(s). For SLS fulfilment information monitoring, assurance closed control loop introduced in clause 4.1.3.1 of TS 28.536 [m] may be applied to achieve the SLS assurance control. NPN-SC, as AssuranceControlLoop_consumer, derives the AssuranceControlLoopGoal from the ServiceProfile in the case of slice scenarios and provides the AssuranceControlLoopGoal to Entities_Participating_in_loop by utilizing the provision management services defined in clause 11.1.1.3 of TS 28.532 [14], see more details in clause 4.1.3.1 SLS Assurance Procedure in TS 28.536 [m].

Next Change

## B.Y	Procedure for NPN SLA management

The following PlantUML source code is used to describe the procedure for NPN SLA management, as depicted by Figure 6.1.X-1:

@startuml

"NPN-SC" --> [ 1. SLA Requirements\n mapping] "NPN-SP"

--> [ 2. SLS Requirements\n decomposition] "NPN-OP"

--> [ 3. Network \n provisioning]" NPN"

--> [ 4. Network performance \n monitoring and assurance]"NPN-OP"

--> [ 5. SLS fulfilment \n information monitoring]"NPN-SP"

--> [ 6. SLA fulfilment \n information monitoring]"NPN-SC"

@enduml

End of changes