3GPP TSG-SA5 Meeting #153 	S5-240352

Sevilla, Spain, 29 January - 02 February 2024

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.557 | CR | 0014 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR TS 28.557 Correction on procedure figure for SNPN provisioning with 3GPP segments only |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | OAM_NPN_Ph2 |  |  |  |  |  | Date: |  |  | 2024-01-19 |
| Category: | D |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | There are typos in the procedure figure 6.2.1-1 which is for SNPN provisioning with 3GPP segments only. It is proposed to fix this issue from Rapporteur point of view. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Fixing the typos in the procedure figure 6.2.1-1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Typos exist in the spec. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1, Annex B.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

### 6.2.1	Solution for SNPN provisioning with 3GPP segments only

An SNPN, which includes 3GPP segment only, may need to be created for use of an NPN-SC. It is illustrated as provisioning a SNPN in figure 6.2.1-1 which can be used for create SNPN in the MNO Managed Mode and Vertical Managed Mode (see clause 4.3.2).

Figure 6.2.1-1: Procedure of SNPN provisioning with 3GPP segments only

1)	NPN-SP receives SLA requirements of the requested SNPN from NPN-SC. The SLA requirements specifies NPN related SLA according to different vertical industry requirements (e.g. coverage requirement within a specific geographic area, downlink/uplink throughput requirements, latency requirement, etc.) together with other business related information (e.g. NPN lifetime, etc.). The work flow between NPN-SP and NPN-SC is out of scope of present specification.

2)	Based on the requirements from NPN-SC, NPN-SP maps SLS into 3GPP-related NPN requirements including RAN/CN/TN part-related requirements.

3)	 The NPN-SP sends the 3GPP-related NPN requirements in form of NRM fragments (e.g. ServiceProfile <dataType>) to NPN-OP.

4)	The NPN-OP determines the constituent network resources and topology needed for the SNPN creation. The related Managed Object instance (reference to related information models for NR, 5GC in TS 28.541 [7] and generic NRM in TS 28.622 [15], e.g., GNBCUCPFunction IOC, GNBDUFunction IOC, GNBCUUPFunction IOC, SubNetwork IOC, Top IOC and etc.) would be created for the requested SNPN using the operations (e.g. createMOI operations) of generic provisioning MnS in TS 28.532 [14].

The NPN-OP determines to reuse an existing 3GPP segment or create a new 3GPP segment for the requested NPN. If a 3GPP segment from an existing stand-alone NPN can be reused, the NPN-OP may reconfigure that SNPN:

a)	In case of creating a new 3GPP segment for the SNPN:

-	Based on RAN part-related requirements, the 3GPP network management system determines to utilize new RAN NE(s).

-	Based on CN part-related requirements, the 3GPP network management system determines to utilize new CN NF(s) or CN NF service(s).

-	Based on TN part-related requirements, the NPN operator configures the underlying transport network, considering the information on SNPN topology (e.g. external connection points of AN and CN) and performance (e.g. latency, bandwidth).

5)	The NPN-OP notifies the created 3GPP segment information (e.g. the DN of created MOI) to the NPN-SP which subscribes the provisioning notification by re-using the notifications (e.g. NotifyMOICreation notifications) of generic provisioning MnS in TS 28.532 [14].

Next Change

## B.2	Procedure for SNPN provisioning with 3GPP segments only

The following PlantUML source code is used to describe the procedure for SNPN provisioning with 3GPP segments only, as depicted by Figure 6.2.1-1:

@startuml

"NPN-SC" -> "NPN-SP":1. SLA requirements(e.g. coverage, \n DL/UL throughout, latency, NPN lifetime)

"NPN-SP" -> "NPN-SP": 2. Map SLA into 3GPP-related NPN requirements

"NPN-SP" -> "NPN-OP": 3. Send the 3GPP-related NPN requirements\n in form of the corresponding NRM fragments

"NPN-OP" -> "NPN-OP": 4. Decide on the constituent\n network resources and topology\n to trigger MOI creation

"NPN-OP"-> "NPN-SP": 5. NotifyMOICreation Notification

skinparam sequenceMessageAlign center

@enduml

End of changes