3GPP TSG-SA5 Meeting #153 	S5-240881

Sevilla, Spain, 29 January - 02 February 2024

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0523 | rev | 1 | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | New performance measurements for connected mode power saving mechanisms |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple, Deutsche Telekom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | PM_KPI_5G_Ph3 |  |  |  |  |  | Date: |  |  | 2024-01-29 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Efficient connected mode power saving is of paramount importance to User equipment’s (UEs) battery life. 3GPP has defined several functionalities that allow UEs to go to sleep while still in connected mode, without necessarily affecting end user perceived experience. One such main functionalities is the concept of Wake-Up Signal (WUS), for which the detailed procedure is described in TS 38.213-10.3. Currently there is no performance measure defined in TS 28.552 quantifying the utilization and usefulness of such connected mode power saving features, thus, in this CR, it is proposed to define new performance measures for successfully enabling devices to be power efficient in connected mode. Given that WUS is one of the main connected mode power saving features, the focus in this proposal is to define the number of connections that enable WUS as well as number of connections that see WUS being used by UEs (i.e DCI Format 2.6 scheduled, see TS 38.213-10.3). These performance measures are used to derive connected mode power saving functionality enablement and utilization per connection KPIs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding a new performance measure capturing the number of RRCReconfiguration for successfully configuring the connected mode power saving WUS functionality as well as number of connections that see such functionality being used by UEs (DCI Format 2.6 scheduled, see TS 38.213-10.3). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Without this performance measure, assessment of how useful the connected mode power saving WUS functionality is not possible. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1.x (new), 5.1.1.y (new), A.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# >>>>BEGINNING OF CHANGES<<<<

###### 5.1.1.x	Number of Successfully applied RRCReconfiguration for connected mode power saving Wake-Up Signal functionality.

a) Configuring the connected mode power saving WUS functionality is done through RRC signalling by including the dcp-Config-r16 IE (see TS 38.331, 6.3.2- PhysicalCellGroupConfig) in RRCReconfiguration.

This measurement provides the number of such RRCReconfiguration messages that are successfully applied on UE side, for an RRC connection established within the existing NRCellCU.

b)	CC

c) Two subcounters will be maintained. One for MCG and one for SCG. On reception of RRCReconfigurationComplete message from the UE, following a transmission of RRCReconfiguration message to that same UE, If the transmitted RRCReconfiguration message to the UE carries masterCellGroup or secondaryCellGroupConfiguration, and one or both of these containers contain setup of the dcp-Config-r16 IE, then the corresponding subcounters will be incremented accordingly.

d)	Each measurement is an integer value.

e)	The measurement name has the form RRC.WUS.DEPLOYMENT where DEPLOYMENT identifies whether WUS is configured over Master Cell Group, in which case DEPLOYMENT = MCG, or Secondary Cell Group, in which case DEPLOYMENT = SCG.

f)	NRCellCU

g)	Valid for packet switched traffic

h)	5GS

i) One usage of this performance measurement is to characterize the WUS enablement.

# >>>>NEXT CHANGE<<<<

###### 5.1.1.y	Number of scheduled connected mode power saving WUS functionality resources.

a)	After being configured, the connected mode power saving WUS functionality needs to be scheduled to the respective UEs. Such a scheduling happens with the help of a specific Downlink control information format (DCI), namely DCI Format 2.6. So in order to assess the actual utilization of this connected mode power saving functionality, it is proposed to define a new measurement that provides the number of DCI Format 2.6 messages scheduled by the network for an RRC connection established within the existing NRCellCU.

b)	CC

c) Upon transmission of a DCI Format 2.6 message by the network, the counter will be incremented.

d). Each measurement is an integer value.

e)	The measurement name has the form RRC.WusScheduled.

f)	NRCellDU

g)	Valid for packet switched traffic

h)	5GS

i) One usage of this performance measurement is to characterize the WUS utilization.

# >>>>NEXT CHANGE<<<<

A.X Use case for connected mode power saving Wake-Up Signal functionality related measurements.

The above newly proposed performance measurements in 5.1.1.x (RRC.WUS.DEPLOYMENT) and 5.1.1.y (RRC.WusScheduled) are needed for the assessment of the usefulness of the 3GPP defined connection mode power saving mechanisms (WUS specifically). If implemented, these will measure the impact of the applying these mechanisms on the UE power consumption from the standpoint of being able to spend the whole day on minimal number of charge cycles.

# >>>>END OF CHANGES<<<<