3GPP TSG-SA5 Meeting #153 	S5-240662

Sevilla, Spain, 29 January - 02 February 2024

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0525 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | New performance measurements for the UE assistance information (UAI) with release preference feature |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | PM_KPI_5G_Ph3 |  |  |  |  |  | Date: |  |  | 2024-29-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) |  |
| Reason for change: |  | Currently there is no performance measure defined in TS 28.552 quantifying the utilization of this feature in both network and UE side. In this CR, it is proposed to define the number of successfully applied RRCReconfiguration for configuring UE assistance information with release preference as well as number of received UE assistance information with release preference. This performance measure is used to derive UAI with release preference enablement and utilization per connection KPIs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding a new performance measure capturing the number of successfully applied RRCReconfiguration for configuring UE assistance information with release preference as well as number of received UE assistance information with release preference. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Without this performance measure, average number of times UAI with release preference is configured per connection and average number of UAI with release preference utilization per connection cannot be derived. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1.x (new), 5.1.1.y (new), A.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# >>>>BEGINNING OF CHANGES<<<<

###### 5.1.1.x	Number of Successfully applied RRCReconfiguration for configuring UE assistance information with release preference.

a)	This measurement provides the number of RRCReconfiguration carrying the UAI configuration with releasePreference-r16 for an RRC connection established within the existing NRCellCU.

b)	CC

c)	On reception of RRCReconfigurationComplete message from the UE, following a transmission of RRCReconfiguration message to that same UE, If the transmitted RRCReconfiguration message to the UE carries OtherConfig-v1610, and if this latter container contains setup of the releasePreferenceConfig-r16, the counter will be incremented.

d)	Each measurement is an integer value.

e)	The measurement name has the form RRC.OtherConfig.UAI.releasePreferenceConfig

f)	NRCellCU

g)	Valid for packet switched traffic

h)	5GS

i) One usage of this performance measurement is to characterize the UAI with release preference enablement.

# >>>>NEXT CHANGE<<<<

###### 5.1.1.y	Number of received UE assistance information with release preference

a)	This measurement provides the number of releasePreference-r16 messages received by the network for an RRC connection established within the existing NRCellCU.

b)	CC

c)	Upon reception of UEAssistanceInformation message by the network, if the message contains the releasePreference-r16 IE, the counter will be incremented.

d)	Each measurement is an integer value.

e)	The measurement name has the form RRC.UAI.releasePreference.

f)	NRCellCU

g)	Valid for packet switched traffic

h)	5GS

i) One usage of this performance measurement is to characterize the UAI with release preference utilization.

# >>>>NEXT CHANGE<<<<

A.X Use case for UE assistance with release preference information related measurements

The above newly proposed performance measurements in 5.1.1.x (RRC.OtherConfig.UAI.releasePreferenceConfig) and 5.1.1.y (RRC.UAI.releasePreference.) are needed for the assessment of the usefulness of the 3GPP defined connection mode power saving mechanisms (UE Assistance with release preference specifically). If implemented, these will measure the impact of the applying these mechanisms on the UE’s daily battery consumption.

# >>>>END OF CHANGES<<<<