3GPP TSG-SA5 Meeting #153	S5-240982

# Sevilla, Spain, 29 January - 02 February 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.422 | CR | 0426 | rev | 1 | Current version: | 15.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-15 CR TS 32.422 Add missing steps of Trace Start in N3IWF for untrusted non-3GPP access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15 |  |  |  |  |  | Date: |  |  | 2024-01-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Signaling based trace activation of untrusted non-3GPP access scenarios are defined in 32.422. But there is a gap that N3IWF is not enabled to trace the messages in the mentioned scneario. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding the missing step of sending the trace activation parameters from AMF to N3IWF and activation of trace in N3IWF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No means to trace N3IWF messages in case of untrusted non-3GPP access scenarios. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 4.1.2.15.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S5-240982 is a revision of S5-240257 |  |  |  |  |  |  |  |  |

Start of First change

# 2	References

The following documents contain provisions, which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

NOTE:	Overall management principles are defined in 3GPP TS 32.101 [1].

[1]	3GPP TS 32.101: "Telecommunication management; Principles and high level requirements".

[2]	3GPP TS 32.421: "Telecommunication management; Subscriber and equipment trace: Trace concepts and requirements".

[3]	3GPP TS 32.423: "Telecommunication management; Subscriber and equipment trace: Trace data definition and management".

[4]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[5]	3GPP TS 52.008: "Telecommunication management; GSM subscriber and equipment trace".

[6]	3GPP TS 23.060: "General Packet Radio Service (GPRS) Service description; Stage 2".

[7]	3GPP TS 23.205: "Bearer-independent circuit-switched core network; Stage 2".

[8]	3GPP TS 23.108: "Mobile radio interface layer 3 specification, core network protocols; Stage 2 (structured procedures)".

[9]	3GPP TS 23.246: "Multimedia Broadcast/Multicast Service (MBMS); Architecture and functional description".

[10]	3GPP TS 29.232: "Media Gateway Controller (MGC) - Media Gateway (MGW); interface; Stage 3".

[11]	3GPP TS 29.002: "Mobile Application Part (MAP) specification".

[12]	3GPP TS 29.060: "General Packet Radio Service (GPRS); GPRS Tunnelling Protocol (GTP) across the Gn and Gp interface".

[13]	3GPP TS 25.413: "UTRAN Iu interface Radio Access Network Application Part (RANAP) signalling".

[14]	3GPP TS 23.218: "IP Multimedia (IM) session handling; IM call model; Stage 2".

[15]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[16]	3GPP TS 29.228: "IP Multimedia (IM) Subsystem Cx and Dx Interfaces; Signalling flows and message contents".

[17]	3GPP TS 29.328: "IP Multimedia Subsystem (IMS) Sh interface; Signalling flows and message contents".

[18]	Enabler Release Definition for OMA Device Management Specifications, version 1.2, The Open Mobile Alliance™ (URL:http://www.openmobilealliance.org/).

[19]	3GPP TS 32.240: "Telecommunication management; Charging management; Charging architecture and principles".

[20]	3GPP TS 32.260: "Telecommunication management; Charging management; IP Multimedia Subsystem (IMS) charging".

[21]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[22]	3GPP TS 23.402: "Architecture enhancements for non-3GPP accesses".

[23]	3GPP TS 36.401: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Architecture description".

[24]	3GPP TS 32.442: "Telecommunication management; Trace management Integration Reference Point (IRP); Information Service (IS)".

[25]	3GPP TS 29.273: "Evolved Packet System (EPS); 3GPP EPS AAA interfaces".

[26]	3GPP TS 29.272: "Evolved Packet System (EPS); Mobility Management Entity (MME) and Serving GPRS Support Node (SGSN) related interfaces based on Diameter protocol".

[27]	3GPP TS 32.615: "Telecommunication management; Configuration Management (CM); Bulk CM Integration Reference Point (IRP): eXtensible Markup Language (XML) definitions".

[28]	3GPP TS 32.342: "Telecommunication management; File Transfer (FT) Integration Reference Point (IRP): Information Service (IS)".

[29]	3GPP TS 29.212: " Policy and Charging Control (PCC);Reference points".

[30]	3GPP TS 37.320: "Universal Terrestrial Radio Access (UTRA) and Evolved Universal Terrestrial Radio Access (E-UTRA); Radio measurement collection for Minimization of Drive Tests (MDT);Overall description; Stage 2".

[31]	3GPP TS 25.331: "Radio Resource Control (RRC); Protocol specification"

[32]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification".

[33]	3GPP TS 24.301: "Non-Access-Stratum (NAS) protocol for Evolved Packet System (EPS); Stage 3".

[34]	3GPP TS 29.274: "3GPP Evolved Packet System (EPS); Evolved General Packet Radio Service (GPRS) Tunnelling Protocol for Control plane (GTPv2-C); Stage 3".

[35]	3GPP TS 32.622: "Telecommunication management; Configuration Management (CM); Generic network resources Integration Reference Point (IRP): Network Resource Model (NRM)".

[36]	3GPP TS 36.413: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); S1 Application Protocol".

[37]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN): Overall description stage 2".

[38]	3GPP TS 36.214: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical layer - Measurements".

[39]	3GPP TS 32.130: "Network sharing; Concepts and requirements".

[40]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[41]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2"

[42]	3GPP TS 38.300: "NR and NG-RAN Overall Description; Stage 2".

[43]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".

[44]	3GPP TS 38.401: "NG-RAN; Architecture Description".

[45]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[46]	3GPP TS 28.541: "Network Resource Model (NRM); Stage 2 and stage 3".

[x]	3GPP TS 38.413: "NG-RAN; NG Application Protocol (NGAP)".

Start of First change

##### 4.1.2.15.3	Non-3GPP access scenarios

Figure 4.1.2.15.3.1 illustrates the signaling Trace Session activation procedure in 5GC as part of the Registration via Untrusted non-3GPP Access procedure:

Figure 4.1.2.15.3.1: Trace activation in 5GC following the Registration via Untrusted non-3GPP Access procedure

The steps 3-11, 14 and 17 below are parts of the Registration via Untrusted non-3GPP Access procedure - see 3GPP TS 23.502 [41] clauses 4.12.2 and 4.2.2 for specific details. Present document does not attempt to re-define how Registration via Untrusted non-3GPP Access procedure works, but rather illustrates the signaling Trace Activation aspects.

1.	Management system activates Trace Session to the UDM. The following trace control and configuration parameters shall be included in the Trace Activation message:

-	SUPI or IMEISV.

-	Trace Reference.

-	Triggering events for AMF, SMF, UPF and PCF.

-	Trace Depth.

-	List of NE types to trace.

-	List of Interfaces for AMF, SMF, UPF, PCF and N3IWF.

-	IP address of Trace Collection Entity .

2.	UDM stores the trace control and configuration parameters received from the management system.

3.	UE connects to the Untrusted non-3GPP Access Network and obtains an IP address

4.	UE performs IKE authentication with N3IWF

5.	N3IWF selects appropriate AMF

6.	N3IWF sends N2 Registration request to AMF

7.	AMF requests AAA Key from AUSF

8.	UE authenticates with AUSF via N3IWF and AMF

9.	UE establishes Signaling IPsec SA with N3IWF

10.	UE sends SMC Complete to AMF

11.	AMF receives the trace control and configuration parameters information from UDM via Nudm_SDM_Get operation (see step 14 in clause 4.2.2.2.2 and clause 5.2.3.3 of 3GPP TS 23.502 [41]).

12.	AMF stores the trace control and configuration parameters received from the UDM.

13.	AMF starts the Trace Session according to the received configuration.

14.	AMF sends INITIAL CONTEXT SETUP REQUEST (or START TRACE message as defined in TS 38.413 [x]) message to N3IWF containing the “Trace Activation” IE. (see clause 8.11.1 for information on TRACE START procedure, clause 8.3.1 for information on INITIAL CONTEXT SETUP REQUEST and clause 9.3.1.14 for details on Trace Activation IE of 3GPP TS 38.413 [x]).

15.	N3IWF stores the trace control and configuration parameters received from AMF.

16.	N3IWF starts the Trace Session according to the received configuration.

17.	AMF establishes Policy Association with PCF (see step 16 in clause 4.2.2.2.2 of 3GPP TS 23.502 [41]) and provides the trace control and configuration parameters information to the PCF.

18.	PCF stores the trace control and configuration parameters received from the AMF as part of Policy Association.

19.	PCF starts the Trace Session according to the received configuration.

20.	AMF provides the trace control and configuration parameters information to the SMF as part of the SM Context (see step 18 in clause 4.2.2.2.2 of 3GPP TS 23.502 [41]).

21.	SMF stores the trace control and configuration parameters received from the AMF.

22.	SMF starts the Trace Session according to the received configuration.

Figure 4.1.2.15.3.2 illustrates the signaling Trace Session activation procedure in 5GC as part of the PDU Session Establishment via Untrusted non-3GPP Access procedure for the UE that has already been registered:

Figure 4.1.2.15.3.2: Trace activation in 5GC following the PDU Session Establishment via Untrusted non-3GPP Access procedure

The steps 6 - 9, 13 and 16 - 19 below are parts of the UE Requested PDU Session Establishment via Untrusted non-3GPP Access procedure - see 3GPP TS 23.502 [41] clause 4.12.5 for specific details. Present document does not attempt to re-define how UE Requested PDU Session Establishment via Untrusted non-3GPP Access procedure works, but rather illustrates the signaling Trace Activation aspects.

1.	Management system activates Trace Session to the UDM. The following trace control and configuration parameters shall be included in the Trace Activation message:

-	SUPI or IMEISV

-	Trace Reference

-	Triggering events for AMF, SMF, UPF and PCF

-	Trace Depth

-	List of NE types to trace

-	List of Interfaces for AMF, SMF, UPF, PCF and NG-RAN

-	IP address of Trace Collection Entity

2.	UDM stores the trace control and configuration parameters received from the management system.

3.	UDM sends Nudm_SDM_Notification to AMF with the trace control and configuration parameters information (see clauses 4.5.1 and 5.2.3.3 of 3GPP TS 23.502 [41]).

4.	AMF stores the trace control and configuration parameters received from the UDM.

5.	AMF starts the Trace Session according to the received configuration.

6.	AMF sends START TRACE message or INITIAL CONTEXT SETUP REQUEST message to N3IWF containing the “Trace Activation” IE. (see clause 8.11.1 for information on TRACE START procedure, clause 8.3.1 for information on INITIAL CONTEXT SETUP REQUEST and clause 9.3.1.14 for details on Trace Activation IE of 3GPP TS 38.413 [x]).

7.	N3IWF stores the trace control and configuration parameters received from AMF.

8.	N3IWF starts the Trace Session according to the received configuration.

9.	IPSec tunnel is established for NAS signalling established between UE and N3IWF as specified in clause 4.12.2 of 3GPP TS 23.502 [41].

10.	UE sends PDU Session Establishment request to AMF

11.	AMF selects an appropriate SMF

12.	AMF sends the Nsmf_PDUSession_CreateSMContext request to the selected SMF

13.	SMF stores the trace control and configuration parameters received from the AMF.

14.	SMF starts the Trace Session according to the received configuration.

15.	SMF selects an approprite PCF

16.	SMF establishes Session Management Policy Association with PCF (see step 7 in clause 4.3.2.2.1 of 3GPP TS 23.502 [41]) and provides the trace control and configuration parameters information to the PCF.

17.	PCF stores the trace control and configuration parameters received from the SMF as part of Policy Association.

18.	PCF starts the Trace Session according to the received configuration.

19.	SMF performs N4 Session Establishment with UPF (see step 10 in clause 4.3.2.2.1 of 3GPP TS 23.502 [41]) and provides the trace control and configuration parameters information to the UPF.

20.	UPF stores the trace control and configuration parameters received from the SMF as part of N4 Session Establishment.

21.	UPF starts the Trace Session according to the received configuration.

22.	AMF sends N2 PDU Session Request to N3IWF

23. N3IWF determines the necessary number of IPsec child SAs and establishes them with UE

24.	N3IWF sends PDU Session Establishment accept to UE

25.	N3IWF sends N2 PDU Session Request Ack to AMF

Figure 4.1.2.15.3.3 illustrates the signaling Trace Session activation procedure in 5GC as part of the PDU Session Establishment via Untrusted non-3GPP Access procedure for the UE that has already been registered where SMF obtains trace control and configuration parameters from UDM via Nudm_UECM_Registration procedure:

Figure 4.1.2.15.3.3: Trace activation in 5GC following the PDU Session Establishment via Untrusted non-3GPP Access procedure

The steps 6 - 9, 13 and 16 - 19 below are parts of the UE Requested PDU Session Establishment via Untrusted non-3GPP Access procedure - see 3GPP TS 23.502 [41] clause 4.12.5 for specific details. Present document does not attempt to re-define how UE Requested PDU Session Establishment via Untrusted non-3GPP Access procedure works, but rather illustrates the signaling Trace Activation aspects.

1.	Management system activates Trace Session to the UDM. The following trace control and configuration parameters shall be included in the Trace Activation message:

-	SUPI or IMEISV

-	Trace Reference

-	Triggering events for AMF, SMF, UPF and PCF

-	Trace Depth

-	List of NE types to trace

-	List of Interfaces for AMF, SMF, UPF, PCF and N3IWF

-	IP address of Trace Collection Entity

2.	UDM stores the trace control and configuration parameters received from the management system.

3.	UDM sends Nudm_SDM_Notification to AMF with the trace control and configuration parameters information (see clauses 4.5.1 and 5.2.3.3 of 3GPP TS 23.502 [41]).

4.	AMF stores the trace control and configuration parameters received from the UDM.

5.	AMF starts the Trace Session according to the received configuration.

6.	AMF sends START TRACE message or INITIAL CONTEXT SETUP REQUEST message to N3IWF containing the “Trace Activation” IE. (see clause 8.11.1 for information on TRACE START procedure, clause 8.3.1 for information on INITIAL CONTEXT SETUP REQUEST and clause 9.3.1.14 for details on Trace Activation IE of 3GPP TS 38.413 [x]).

7.	N3IWF stores the trace control and configuration parameters received from AMF.

8.	N3IWF starts the Trace Session according to the received configuration.

9.	IPSec tunnel is established for NAS signalling established between UE and N3IWF as specified in clause 4.12.2 of 3GPP TS 23.502 [41].

10.	UE sends PDU Session Establishment request to AMF

11.	AMF selects an appropriate SMF

12.	AMF sends the Nsmf_PDUSession_CreateSMContext request to the selected SMF

13.	SMF performs NuDM_UECM_Registration procedure with UDM and receives the trace control and configuration parameters from UDM

14.	SMF stores the trace control and configuration parameters received from the UDM.

15.	SMF starts the Trace Session according to the received configuration.

16.	SMF selects an approprite PCF

17.	SMF establishes Session Management Policy Association with PCF (see step 7 in clause 4.3.2.2.1 of 3GPP TS 23.502 [41]) and provides the trace control and configuration parameters information to the PCF.

18.	PCF stores the trace control and configuration parameters received from the SMF as part of Policy Association.

19.	PCF starts the Trace Session according to the received configuration.

20.	SMF performs N4 Session Establishment with UPF (see step 10 in clause 4.3.2.2.1 of 3GPP TS 23.502 [41]) and provides the trace control and configuration parameters information to the UPF.

21.	UPF stores the trace control and configuration parameters received from the SMF as part of N4 Session Establishment.

22.	UPF starts the Trace Session according to the received configuration.

23.	AMF sends N2 PDU Session Request to N3IWF

24. N3IWF determines the necessary number of IPsec child SAs and establishes them with UE

25.	N3IWF sends PDU Session Establishment accept to UE

26.	N3IWF sends N2 PDU Session Request Ack to AMF

NOTE:	The specific scenarios where SMF receives trace control and configuration parameters either from UDM or from AMF are specified in 3GPP TS 23.502 [41].

End of Changes