3GPP TSG-SA Meeting #103	SP-240284

Maastricht, The Netherlands, 19 - 22 March 2024

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.143 | CR | - | rev | - | Current version: | 2.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for audio codecs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | - |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Apple Inc., Qualcomm Incorporated, Dolby Laboratories Inc., Fraunhofer IIS |  |  |  |  |  |  |  |  |  |
| Work item code: | PROMISE |  |  |  |  |  | Date: |  |  | 2024-03-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Agreed pCR S4-240420 to TS 26.143 from SA4#127 inadvertently flipped the mandate for xHE-AAC and eAAC+ in clause 6.2.3 (profile for content playback/consumption). How it stands currently in TS 26.143 v1.1.0 clause 6.2.3 is that xHE-AAC is mandated as a “shall” codec, while eAAC+ is mandated with a “should”. This is the reverse of the mandate for content generation (clause 6.3.3, there xHE-AAC should be, while eAAC+ shall be supported), making this a FASMO issue (conforming clients and servers cannot interoperate). The aim of specification is to stay aligned with existing SA4 specifications, where e.g. in TS 26.511 and agreements for TS 26.140 also specify the same mandate (i.e. xHE-AAC should be, while eAAC+ shall be supported). This pCR corrects the issue. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The mandate for xHE-AAC and eAAC+ in clause 6.2.3 (profile for content playback/consumption) is fixed such that xHE-AAC should be, while eAAC+ shall be supported. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A FASMO error will remain in the specification that would prevent interoperability. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This pCR was agreed in SA4 MBS SWG Telco on 7th March 2024. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 6.2.3	Media Types

The capability 26143_TEXT_PLAIN as defined in clause 5.3 shall be supported.

If still images are supported,

-	the 26143_IMG_JPEG capability as defined in clause 5.4.1 shall be supported,

-	the 26143_IMG_HEIC capability as defined in clause 5.4.1 should be supported.

If bitmap graphics are supported,

-	the 26143_IMG_GIF capability as defined in clause 5.4.1 should be supported.

-	the 26143_IMG_PNG capability as defined in clause 5.4.1 should be supported.

If the reception of audio or speech is supported, then the following applies:

-	the 26143_AUDIO_EVS capability as defined in clause 5.5.1 shall be supported.

-	the 26143_AUDIO_AMR-WB capability as defined in clause 5.5.1 shall be supported.

-	the 26143_AUDIO_XHE-AAC capability as defined in clause 5.5.1 should be supported.

-	the 26143_AUDIO_AMR capability as defined in clause 5.5.1 shall be supported.

-	the 26143_AUDIO_EAAC+ capability as defined in clause 5.5.1 shall be supported.

If the reception of video is supported, then the following applies:

-	the 26143_VIDEO_AVC-HD capability as defined in clause 5.6.1 shall be supported.

-	the 26143_VIDEO_HEVC-HD capability as defined in clause 5.6.1 should be supported.

If the reception of HD-HDR video is supported, then the following applies:

-	the 26143_VIDEO_AVC-FullHD capability as defined in clause 5.6.1 shall be supported.

-	the 26143_VIDEO_HEVC-FullHD capability as defined in clause 5.6.1 shall be supported.

-	the 26143_VIDEO_HEVC-UHD capability as defined in clause 5.6.1 should be supported.

If timed text is supported,

-	the 26143_TT_3GPP capability as defined in clause 5.7.1 shall be supported.

-	the 26143_TT_IMSC11 capability as defined in clause 5.7.1 should be supported.

If a processor for media type 'model' as defined in RFC2077 [33] is supported (i.e. a processor for 3D scenes and objects), then a processor for the media subtype 'model/gltf' should be supported. If a processor for the media subtype 'model/gltf' is supported,

-	the 26143_SCENE_GLTF20 capability and the 26143_SCENE_GLTF20_GLB capability as defined in clause 5.8 shall be supported assuming either a single body part or a multipart/related body part as defined in clause 5.2.

-	and if the device is a device type as defined in TS 26.119 [34], clause 10, the 26143_SCENE_GLTF20_AR and the 26143_SCENE_GLTF20_GLB_AR capability as defined in clause 5.8 shall be supported assuming either a single body part or a multipart/related body part as defined in clause 5.2.

If a processor for the media type text/html is supported, the 26143_PRESENTATION_HTML5 capability as defined in clause 5.9 should be supported assuming either a single body part or a multipart/related body part as defined in clause 5.2. The media formats shall be restricted to the capabilities defined in this clause.

* * * End of Changes * * * *