3GPP TSG-SA WG6 Meeting #59	S6-240301

Athens, Greece 26th February – 1st March 2024	(revision of S6-24xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0289 | rev | - | Current version: | 17.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correct IE presence condition |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eSEAL |  |  |  |  |  | Date: |  |  | 2024-01-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The presence condition for IE Monitoring profile ID in cl.14.3.2.18 is not described properly. In addition, some IE presence condition is not described properly like “only one of two IEs may be provided” for optional IEs, w/o mentioning condition. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In cl.14.3.2.18, add missing condition. In cl.14.3.2.20, change it to “if provided, only one of the two IEs shall be provided”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inappropriate description for IE presence condition. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 14.3.2.18, 14.3.2.20 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 14.3.2.18	Monitoring Events Subscription Response

Table 14.3.2.18-1 describes the information flow from the NRM server to the VAL server for Monitoring Events Subscription response.

Table 14.3.2.18-1: Monitoring Events Subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Subscription status | M | It indicates the subscription result |
| Monitoring profile ID | O | It indicates the monitoring profile identifier. It is present when Subscription status is success and Event Details are provided in the Monitoring Events Subscription request. |

* * * Next Change * * * *

#### 14.3.2.20	Unicast QoS monitoring subscription request

Table 14.3.2.20-1 describes the information flow from the VAL server to the network resource management server for unicast QoS monitoring subscription request.

Table 14.3.2.20-1: Unicast QoS monitoring subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the VAL server performing the request. |
| List of VAL UE IDs | O (see NOTE 1) | List of VAL UEs whose QoS monitoring data is requested. |
| VAL group ID | O (see NOTE 1) | The group ID used for the VAL group for which QoS monitoring data is requested. |
| List of VAL stream IDs | O (see NOTE 1) | List of VAL streams for which QoS monitoring data is requested. |
| Measurement requirements | O | It describes the requirements on the QoS measurements. |
| > type of data | M | At least one of the following measurement index shall be provided. |
| >> packet delay | O | UL/DL/RT packet delay. For VAL stream, the packet delay means measurement from a source UE to a target UE. |
| >> packet loss rate | O | Average packet loss rate. |
| >> data rate | O | Average data rate and/or maximum data rate. |
| >> traffic volume | O | Average traffic volume for UL and/or DL |
| > measurement time period | O | The measurement time period indicates a current time period. If absent, current time and 5 minutes duration are used as default setting. |
| > aggregation granularity window | O | Indicates how long the QoS monitoring is performed to compute the average value and gather the maximum value. If absent, 1 minute is used as default setting. |
| Reporting requirements | O | It describes the requirements for QoS monitoring reporting. |
| > Frequency of reporting | O | It indicates the requested frequency of reporting. The reporting frequency may be periodic or event triggered (i.e. threshold reached). If absent in the request, default event triggered reporting is used. |
| > Reporting threshold | O | If the Frequency of reporting is event triggered (threshold reached), the reporting threshold corresponding to the measurement index shall be provided. |
| > Threshold reaching direction | O | It indicates the reaching direction (i.e. ascending, descending or crossed) for the reporting threshold. It shall be provided if reporting threshold is present. |
| > Reporting periodicity | O | If the Frequency of reporting is periodic, the reporting periodicity shall be provided. |
| > Termination of reporting | O | It indicates when the reporting of QoS monitoring shall stop. It can be event triggered (i.e. either number of reports reached or threshold reached) or when reporting duration expires or when explicit termination is requested. If absent in the request, it defaults to explicit termination. Once the termination condition is met, the monitoring subscription ceases to exist. |
| > Max number of reports | O (see NOTE 3) | It indicates the maximum number of reports, it shall be provided when Termination of reporting is set to event triggered (number of reports reached). |
| > Termination threshold | O (see NOTE 2, NOTE 3) | It indicates the reporting termination threshold corresponding to the measurement index. Each measurement index has only one termination threshold. |
| > Termination condition for termination threshold | O (see NOTE 2) | It indicates threshold-based termination condition (e.g. all reached or any of them reached) if multiple measurement indexes are provided. |
| > Reporting duration | O (see NOTE 3) | It indicates the reporting duration, it shall be provided when Termination of reporting is set to duration expiration. |
| NOTE 1: Only one of these information elements shall be present. NOTE 2: It shall be provided when Termination of reporting is set to event triggered (threshold reached). NOTE 3: If provided, only one of these information elements shall be present. |  |  |

* * * End of Change * * * *