3GPP TSG-SA WG6 Meeting #59	S6-240513

Athens, Greece 26th February – 1st March 2024	(revision of S6-240143)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.435 | CR | 0009 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Fault diagnosis subscription request |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19, NSCALE |  |  |  |  |  | Date: |  |  | 2024-02-18 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | (1)In the Fault diagnosis subscription request information flow defined in clause 9.15.3.2, the Network slice related Identifier(s) is listed as the child item of Fault diagnosis information IE which is not correct. They should be the same level. Table 9.15.3.2-1: Fault diagnosis subscription Information element; Status; Description. VAL server ID; M; The identifier of the VAL server. VAL service ID; M; The identifier of the VAL service for which the request applies. VAL UE ID list; O; The list of VAL UE IDs for which the request applies. Fault diagnosis ID; M; Identifier of the fault diagnosis task. Fault diagnosis information; M; The information of performance and analytics monitoring. > Network slice related Identifier(s); O; Identifier(s) of the network slice to be monitored. (2)It is not clear when and how the VAL server obtains the Fault diagnosis ID. Whether the NSCALE server understands such Fault diagnosis ID. So it is proposed to remove the Fault diagnosis ID and its corresponding Fault diagnosis information. And the NSCALE will based on the VAL service ID and its knowledge to provide the fault description report to the VAL server. |
| Summary of change: |  | - Remove the ‘>’ from IE Network slice related Identifier(s) - Remove the Fault diagnosis ID IE and Fault diagnosis information IE; |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect information flow table leads misunderstanding to the downstream groups. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.15.3.2, 9.15.3.3, 9.15.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# /************************** 1st changes ****************************/

9.15.3.2	Fault diagnosis subscription request

Table 9.15.3.2-1 describes information elements for fault diagnosis subscription.

Table 9.15.3.2-1: Fault diagnosis subscription

| Information element | Status | Description |
| --- | --- | --- |
| VAL server ID | M | The identifier of the VAL server |
| VAL service ID | M | The identifier of the VAL service for which the request applies |
| VAL UE ID list | O | The list of VAL UE IDs for which the request applies |
| Fault diagnosis information | O | Indicate what kind of fault is need for the subscription, e.g., time, specific problem, etc.. |
| Network slice related Identifier(s) | O | Identifier(s) of the network slice to be monitored |

9.15.3.3	Response of fault diagnosis subscription

Table 9.15.3.3-1 describes information elements for response of fault diagnosis subscription.

Table 9.15.3.3-1: Response of fault diagnosis subscription

| Information element | Status | Description |
| --- | --- | --- |
| Subscription ID | M | Subscription identifier corresponding to the subscription |
| Result | M | The result of the request (positive or negative acknowledgement) |

9.15.3.4	Fault diagnosis notification

Table 9.15.3.4-1 describes information elements for fault diagnosis notification.

Table 9.15.3.3-4: Fault diagnosis notification

| Information element | Status | Description |
| --- | --- | --- |
| Subscription ID | M | Subscription identifier corresponding to the subscription |
| Fault Report ID | M | Identifier of the fault Report |
| FaultReport | M | The report of the fault diagnosis |
| >CorrelatedAlarmList | M | The list of the correlated alarms |
| >>CorrelatedAlarm | M | The correlated alarms |
| >rootCause | M | The root cause of the fault |