3GPP TSG-SA WG6 Meeting #59	S6-240694

Athens, Greece 26th February – 1st March 2024	(revision of S6-240299)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0586 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Service continuity in ENS via leading ECSP |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph3 |  |  |  |  |  | Date: |  |  | 2024-01-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The ENS service continuity via leading ECSP is not completed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add more procedure description for ENS service continuity via leading ECSP. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing procedures in supporting service continuity for ENS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Existing clauses: 6.7.2, 8.18.2.4.1 New clauses: 8.18.2.4.X1, 8.18.2.4.X2, 8.18.2.4.X3, 8.18.2.4.X4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

##### 8.18.2.4.1	General

The AC connected EAS may be changed when the UE moves within the same partner ECSP' service coverage or to a different partner ECSP' service coverage or back to leading ECSP' service coverage.

S-EAS and T-EAS can be both provided by partner ECSP.

Some EEC request (over EDGE-1) or EAS request (over EDGE-3) may be relayed between leading EES and partner EES (over EDGE-9), with necessary modification (e.g. along with the ECSP ID of the sender). The following identifies a list of relayed procedures:

-	ACR launching

-	Selected T-EAS declaration

-	ACR status update

-	EAS Information provisioning

##### 8.18.2.4.X1	ACR launching procedure

The S-EES of the leading ECSP uses the procedure as defined in clause 8.8.3.4 to forward the ACR request with ACR action indicating ACR initiation from the EEC towards the partner EES over EDGE-9 reference point, and then forwards the ACR response from the partner EES to the EEC.

##### 8.18.2.4.X2	Selected T-EAS declaration procedure

Upon receiving the Selected target EAS declaration request from the S-EAS, the partner EES uses the procedure as defined in clause 8.8.3.7 to send the request to the leading EES over EDGE-9 reference point. Upon receiving the response from the leading EES, the partner EES sends the response back to the S-EAS.

##### 8.18.2.4.X3	ACR status update procedure

Upon receiving ACR status update request from the EAS (either S-EAS or T-EAS), the partner EES uses the procedure as defined in clause 8.8.3.8 to send the request to the leading EES over EDGE-9 reference point. Upon receiving the response from the leading EES, the partner EES sends the response back to the EAS.

##### 8.18.2.4.X4	EAS Information provisioning procedure

Upon receving EAS Information provisioning request from the EEC, if the request contains selected EAS ID and selected EAS Endpoint and if the selected EAS is not registered the leading EES, then the leading EES uses the procedure as defined in clause 8.15.2.2 to send the EAS Information provisioning request to the partner EES over EDGE-9 reference point to indicate about selection of the EAS for the service consumption. The request includes EES ID and EES endpoint address along with the information sent by the EEC. Upon receiving the response from the partner EES, the leading EES sends the response back to the EEC.

* * * Next Change * * * *

### 6.7.2	APIs provided by the Edge Enabler Layer

Table 6.7.2-1 summarizes the APIs exposed by the ECS.

Table 6.7.2-1: APIs provided by the ECS

| API Name | Known Consumers | References |
| --- | --- | --- |
| Eecs_ServiceProvisioning | EEC | 8.3 |
| Eecs_EESRegistration | EES | 8.4.4 |
| Eecs_TargetEESDiscovery | EES | 8.8.3.3 |
| Eecs_ECSRegistration | ECS | 8.17.2.2 |
| Eecs_ECSDiscovery | ECS | 8.17.2.3 |
| Eecs_ECSServiceProvisioning | ECS | 8.17.2.4 |
| Eecs_EASInfoManagement | EES | 8.20.2 |

Table 6.7.2-2 summarizes the APIs exposed by the EES.

Table 6.7.2-2: APIs provided by the EES

| API Name | Known Consumers | References |
| --- | --- | --- |
| Eees_EECRegistration | EEC | 8.4.2 |
| Eees_EASRegistration | EAS | 8.4.3 |
| Eees_EASDiscovery | EEC | 8.5 |
| Eees_UELocation | EAS | 8.6.2 |
| Eees_ACRManagementEvent | EAS | 8.6.3 |
| Eees_AppClientInformation | EAS | 8.6.4 |
| Eees_UEIdentifier | EEC, EAS | 8.6.5 |
| Eees_SessionWithQoS | EAS | 8.6.6 |
| Eees_TrafficInfluenceEAS | EAS | 8.6.7 |
| Eees_TargetEASDiscovery | EAS, EES | 8.8.3.2 |
| Eees_AppContextRelocation | EEC, EAS, EES | 8.8.3.4 |
| Eees_ACREvents | EEC | 8.8.3.5 |
| Eees_EELManagedACR | EAS | 8.8.3.6 |
| Eees_SelectedTargetEAS | EAS, EES | 8.8.3.7 |
| Eees_ACRStatusUpdate | EAS, EES | 8.8.3.8 |
| Eees_ACRParameterInformation | EES | 8.8.3.9 |
| Eees_EECContextPull | EES | 8.9.4.2 |
| Eees_EECContextPush | EES | 8.9.4.3 |
| Eees_EASInformationProvisioning | EEC, EES | 8.15 |
| Eees_CommonEasAnnouncement | EES | 8.19 |

NOTE:	The event exposure related APIs (e.g. Eees_EASDiscovery and Eees_ACREvents) can be realized as single event subscription API.

Table 6.7.2-3 summarizes the APIs exposed by the CAS.

Table 6.7.2-3: APIs provided by the CAS

| API Name | Known Consumers | References |
| --- | --- | --- |
| Ecas_SelectedEES | EES | 8.8.3.10 |

Table 6.7.2-4 summarizes the APIs exposed the EEC.

Table 6.7.2-4: APIs provided by the EEC

| API Name | Known Consumers | References |
| --- | --- | --- |
| Eeec_ACRegistration | AC | 8.14.4.2 |
| Eeec_EASDiscovery | AC | 8.14.4.3 |
| Eeec_ACRTrigger | AC | 8.14.4.4 |
| Eeec_Services | AC | 8.14.4.5 |
| Eeec_UEId | AC | 8.14.4.6 |

* * * END of Change * * * *