3GPP TSG-SA WG6 Meeting #59	S6-240716

Athens, Greece 26th February – 1st March 2024	(revision of S6-240622)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0600 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add missing common EAS removal |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph3 |  |  |  |  |  | Date: |  |  | 2024-01-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The ECS-ER maintains common EAS information but there is no procedure defined to remove stored EAS information in ECS-ER. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add common EAS removal with ECS-ER. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TS remains incomplete as the ECS-ER cannot remove outdated information when no group member is using the common EAS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.20.2.x (new), 8.20.3.x (new), 8.20.3.y (new), 8.20.4.1, 8.20.4.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 8.20.2.x	Common EAS information removal

Pre-condition:

1.	Common EAS is deregistered from the common EES.

Editor's note:	It is FFS about other triggers for common EES to use this procedure.

Figure 8.20.2.3-1: Common EAS information removal

1.	The common EES sends Common EAS information remove request message to the ECS-ER. The request message includes EES ID, EAS ID, EAS endpoint and Application Group ID.

2.	Upon receiving the request, the ECS-ER removes the stored information associated with the common EAS identified in the request.

3.	The ECS-ER responds the common EES with Common EAS information remove response message.

* * * Next Change * * * *

#### 8.20.3.x	Common EAS information remove request

Table 8.20.3.x-1 describes the information elements for common EAS information remove request from the EES to the ECS-ER.

Table 8.20.3.x-1: Common EAS information remove request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the EES. |
| Security credentials | M | Security credentials resulting from a successful authorization for the edge computing service. |
| EAS ID | M | The identifier of the EAS. |
| Application Group ID | M | Application group identifier as defined in clause 7.2.11. |
| EAS endpoint | M | Endpoint information (e.g. URI, FQDN, IP address) used to communicate with the EAS. |

#### 8.20.3.y	Common EAS information remove response

Table 8.20.3.y-1 describes the information elements for common EAS information remove response from the ECS-ER to the EES.

Table 8.20.3.5-1: Common EAS information remove response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response (NOTE) | O | Indicates that the request was successful. |
| Failure response (NOTE) | O | Indicates that the request has failed. |
| > Cause | O | Indicates the failure cause. Only included when Failure response is included. |
| NOTE: One of these IEs shall be present in the message. |  |  |

* * * Next Change * * * *

#### 8.20.4.1	General

Table 8.20.4.1-1 illustrates the API for EAS information management.

Table 8.20.4.1-1: EAS information management APIs

| API Name | API Operations | Operation Semantics | Consumer(s) |
| --- | --- | --- | --- |
| Eecs_EASInfoManagement | Store | Request/Response | EES |
|  | Remove | Request/Response | EES |
|  | Get | Request/Response | EES |

* * * Next Change * * * *

#### 8.20.4.x	Eecs_EASInfoManagement_Remove operation

API operation name: Eecs_EASInfoManagement_Remove

Description: The consumer requests to remove EAS information from the ECS.

Inputs: See clause 8.20.3.x.

Outputs: See clause 8.20.3.y.

See clause 8.20.2.x for details of usage of this operation.

* * * END of Change * * * *