3GPP TSG-SA WG6 Meeting #59	S6-240487

Athens, Greece 26th February – 1st March 2024	(revision of S6-240267)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.433 | CR | 0029 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on content delivery in SEALDD-S interface |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | SEALDD_Ph2 |  |  |  |  |  | Date: |  |  | 2024-02-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For content/data delivery between VAL server and SEALDD server, there are separate clause in 9.1 and 9.5, this paper is proposed to add the related description to align the data storage and data query in clause 9.5 with push mode and pull mode defined in clause 9.1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This paper proposes to add the related description to clarify the content/data delivery in SEALDD-S interface. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The mechanism about the SEALDD data storage in clause 9.5 is not completed, and the relationship with push/pull mode define in clause 9.1 is not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.4.4, 9.1, 9.5.2.1, 9.5.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

7.4.4	SEALDD-S

Reference point between SEALDD server and VAL server to enable northbound server side API exposed by SEALDD server to VAL server for data delivery and SEALDD service provisioning, control, reporting etc.

NOTE 1: The data/content delivery between SEALDD server and VAL server can use the pull mode or push mode over SEALDD-S interface, as specified in clause 9.1.

* * * Next Change * * * *

9	Procedures and information flows

9.1	General

The VAL application data/content can be stored in the VAL server, or the SEALDD server, or other cloud platform. For the downlink traffic transmission, the SEALDD server can retrieve the data/content of VAL application, by using one of the following modes:

-	Downlink pull mode: the SEALDD server can pull the data/content of VAL application from the address provided by the VAL server (i.e. pull from the data/content address in VAL server over SEALDD-S interface, or in other cloud platform).

-	Downlink push mode: the VAL server can push the data/content of VAL application to the SEALDD server over SEALDD-S interface.

For the uplink traffic transmission, the SEALDD server can sends the data/content of VAL application to the address provided by the VAL server, by using one of the following modes:

-	Uplink pull mode: the VAL server can pull the data/content of VAL application from the SEALDD server over SEALDD-S interface.

-	Uplink push mode: the SEALDD server can push the data/content of VAL application to the address provided by the VAL server (i.e. push to the data/content address in VAL server over SEALDD-S interface, or in other cloud platform).

* * * Next Change * * * *

9.5.2.1	Data storage creation

Pre-conditions:

1.	The VAL server has discovered and selected the SEALDD server by CAPIF functions.

2.	The SEALDD client has discovered and selected the SEALDD server as specified in clause 9.4.3.

Figure 9.5.2.1-1: Storage service creation

1.	The VAL client/server determines to use SEALDD storage service, which could store/host data on behalf of the VAL client/server. The consumer (e.g. SEALDD client, VAL server) send a Sdd_DataStorage_Creation request to the SEALDD server, e.g. using the downlink push mode by VAL server over SEALDD-S interface, as defined in clause 9.1. The request includes the data to be stored and information associated with the data, such as access control policy, expiration time of the storage, etc. The consumer may also specify in the request the management or status information of the data storage that is required (e.g., information about how often the stored data is accessed or managed).

NOTE 1:	If the VAL client determines to use SEALDD storage service, the request is first sent to the SEALDD client hosted on the same UE, and then the SEALDD client send the Sdd_DataStorage_Creation request to the SEALDD server.

NOTE 2:	The detailed request between VAL client and SEALDD client is out of scope of this release of this specification.

2.	The SEALDD server checks for the authorization of the storage service creation request. If the request is successfully authorized, then the SEALDD server configures the storage service based on the request and stores the data at the storage server. If status information of the stored data is requested, the SEALDD server will start to monitor the status of the stored data, such as to track the accesses to the data for the data access status.

3.	The SEALDD server sends a response to the requesting consumer (e.g. SEALDD client, VAL server). The response indicates if the request is accepted and the identifier of the stored data (if applicable). Upon receiving the response, the requesting consumer may create a record for the stored data.

* * * Next Change * * * *

9.5.2.3	Data storage query

Pre-conditions:

1.	VAL client/server has stored data in SEALDD server.

2.	The consumer (e.g. SEALDD client/VAL server/other SEALDD server) requesting the stored data has got the identifier of the stored data (e.g. via application layer signalling or other mechanisms).

Figure 9.5.2.3-1: Access to stored data in SEALDD server

1.	The consumer (e.g. SEALDD client/VAL server/other SEALDD server) sends a Sdd_DataStorage_Query request to the SEALDD server with the identifier of the stored data.

NOTE:	The consumer requesting for the stored data can be the data storage creator or other network functions.

2.	Upon receiving the request, the SEALDD server performs an authorization check to verify if the requested data (according to the data access control policy) can be accessed by the consumer. The SEALDD server will also check whether the stored data is expired. If the requested data queried by SEALDD client is not be stored by the SEALDD server, the SEALDD server may use the downlink pull mode if possible, to retrieve the data/content from the VAL server over SEALDD-S interface, as defined in clause 9.1.

Editor's Note:	Regarding of unstored data/content retrieval from VAL server, whether it should be happened in new procedure (e.g. data storage fetch) is FFS.

3.	If the verification was successful, the SEALDD server responds with the requested data.